3GPP TSG-RAN WG3 Meeting #113-e	R3-214466

E-meeting, 16-27 Aug 2021

Title:	MDT M6 calculation for split bearers in MR-DC

Response to:

Release:	Rel-17

Work Item:	NR_ENDC_SON_MDT_enh

Source:	RAN3

To:	RAN2

Cc:

Contact person:

Name:	Hongzhuo Zhang

E-mail Address:	zhanghongzhuo@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN3 noted that there are some RAN2 agreements for the RAN part delay measurement calculation for split bearers in MR-DC for Qos monitoring.

7	For QoS monitoring related delay reporting to CN, the minimum value between two legs is defined as the total delay measurement M6 over MCG/SCG for split bearers WITH PDCP duplication.

Agreement:

For QoS monitoring related delay reporting to CN, ‘weighted average (consider the number of packets) over MN and SN’ is used to calculate the total delay measurement M6 over MCG/SCG for split bearers WITHOUT PDCP duplication.

RAN3 would like RAN2 to confirm whether above mentioned agreements are also applied to M6 for split bearers in MR-DC in MDT.

2	Actions

To RAN2

ACTION: RAN3 respectfully asks RAN2 to confirm whether above mentioned agreements are also applied to M6 for split bearers in MR-DC in MDT.

3	Dates of next RAN3 meetings

3GPP TSG RAN3#114-e		1-11 Nov 2021	E-Meeting