3GPP TSG-RAN WG3 Meeting #113-e	R3-21xxxx

16-26 Aug 2021, E-meeting

Title: 	TP to 37.817 on AI/ML based network energy saving

Source: 	Intel

Agenda item:	18.4.1

Document Type:	Other

# 1. Introduction

This TP tries to reflect the common understanding on the solution of AI-based network energy saving from CB # AIRAN3_ESSolution, including general descriptions of agreed solution.

# 5. Reference

- R3-214221, SoD_CB # AIRAN3_ESSolution

# Annex – TP for TR 37.817

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 5.1	Network Energy Saving

### 5.1.1	Use case description

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 5.1.2	Solutions and standard impacts

-----------------------------------Start of Changes-----------------------------------

#### 5.1.2.1	Model Training at OAM and Model Inference at NG-RAN

In this solution, NG-RAN predicts energy saving decisions by AI/ML model trained from OAM.

Editor’s Note: FFS “ML Model Deployment/update” between OAM and NG-RAN.

Figure 5.1.2.2-1. Model Training at OAM, Model Inference at NG-RAN

Step 1-3 (FFS): AI-based network energy saving is trained at OAM based on collected data, NG-RAN Node 1 uses the well-trained model for model inference.

Step 4: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 5: UE sends UE measurement report to NG-RAN node 1.

Step 6: Based on NG-RAN nodes local inputs and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output (e.g. energy saving strategy, handover strategy, etc).

Step 7: NG-RAN node 1 selects the most appropriate target cell for each UE before it goes to the predicted energy state and perform handover.

Step 8: UE reports UE measurement report to target NG-RAN node.

#### 5.1.2.2	Model Training at OAM and Model Inference at NG-RAN

In this solution, NG-RAN is responsible for model training and generates energy saving decisions (e.g., for online training).

Figure 5.1.2.2-1. Model Training and Model Inference at NG-RAN

Step 1: The NG-RAN collects data for training from UE, neighbor NG-RAN Node, and itself.

Step 2: The NG-RAN Node 1 trains the AI/ML model for AI-based energy saving based on collected data.

Step 3.1: UE sends UE measurement report to NG-RAN node 1.

Step 3.2: NG-RAN node 2 sends the required input data to NG-RAN node 1 for model inference of AI/ML-based network energy saving.

Step 4: Based on NG-RAN nodes local inputs and received inputs from NG-RAN node 2, NG-RAN node 1 generates model inference output (e.g. energy saving strategy, handover strategy, etc).

Step 5: NG-RAN node 1 selects the most appropriate target cell for each UE before it goes to the predicted energy state and perform handover.

Step 6: NG-RAN node 2 sends the performance feedback to NG-RAN node 1.

#### 5.1.2.3	Input of AI-based Network Energy Saving

To predict the optimized network energy saving decisions, NG-RAN may need following information as input data for AI-based network energy saving:

- Current/Predicted traffic load information of ES-Cell and its neighbor nodes

- Current/Predicted resource status of ES-Cell and its neighbor nodes

- Current/Predicted energy information of ES-Cell and its neighbor nodes

- UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc)

Editor’s Note: FFS other input information required for AI/ML-based network energy saving. FFS energy information is exact energy consumption value or energy efficiency gain

#### 5.1.2.4	Output of AI-based Network Energy Saving

AI-based network energy saving model can generate following information as output:

- Energy saving strategy

- Handover strategy, including recommended candidate cells for taking over the traffic

- Predicted energy information

- Accuracy of predicted energy saving decision

Editor’s Note: FFS other output information expected from AI/ML-based network energy saving. FFS detailed granularity and action of energy saving strategy

#### 5.1.2.5	Performance Feedback of AI-based Network Energy Saving

To optimize the performance of AI-based network energy saving model and get more accurate predicted decision, following performance feedback can be considered to collect from target nodes:

- Load measurement

- Energy information

Editor’s Note: FFS other output information expected from AI/ML-based network energy saving.

-----------------------------------End of Changes-----------------------------------