3GPP TSG-RAN WG3 Meeting #114-e	R3-21xxxx

Online, 01 – 11 November 2021

Agenda item:	19.2.3

Source:	Ericsson

Title:	(TP for NR_pos_enh BL CR for TS 38.473) Further details for on-demand PRS

Document for:	Discussion and Decision

# 1	Introduction

This F1AP TP captures the agreements from CB # 1903_Pos_OnDemandPRS.

# A	Appendix: Text Proposal for TS 38.473 BL CR

Start of Text Proposal for TS 38.473 BL CR

### 8.13.x	PRS Configuration Exchange

#### 8.13.x.1	General

The PRS Configuration Exchange procedure is initiated by the gNB-CU to request the gNB-DU to configure the PRS transmissions.

The procedure uses non-UE-associated signalling.

#### 8.13.x.2	Successful Operation

Figure 8.13.X.2-1: PRS Configuration Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a PRS CONFIGURATION REQUEST message to the gNB-DU node, indicating in the PRS TRP List IE the TRP(s) for which DL-PRS transmission is requested. The gNB-DU should use the information in the Requested DL PRS Transmission Characteristics IE to configure DL-PRS transmission by the indicated TRP(s).

If DL-PRS transmission is successfully configured for at least one of the TRPs, the gNB-DU shall responds with the PRS CONFIGURATION RESPONSE message.

[FFS]

#### 8.13.x.3	Unsuccessful Operation

Figure 8.13.X.2-1: PRS Configuration Exhcange procedure, unsuccessful operation

If the gNB-DU cannot configure DL-PRS transmission for any of the TRPs in the PRS TRP List IE of the PRS CONFIGURATION REQUEST message, it shall respond with a PRS CONFIGURATION FAILURE message with an appropriate cause value.

#### 8.13.X.4	Abnormal Conditions

Void.

Next Change

#### 9.2.12.x	PRS CONFIGURATION REQUEST

This message is sent by a gNB-CU to request a gNB-DU to configure PRS transmissions.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| PRS TRP List |  | 1 |  |  | YES | ignore |
| >PRS TRP Item |  | 1 .. <maxnoofTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>Requested PRS Transmission Characteristics [FFS] | [FFS] |  | 9.3.1.a |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPs | Maximum no. of TRPs in a gNB-DU Value is 65535 |

#### 9.2.12.y	PRS CONFIGURATION RESPONSE

This message is sent by a gNB-DU to ackknowledge the PRS transmissions.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| PRS Transmission TRP List |  | 1 |  |  | YES | ignore |
| >PRS Transmission TRP Item |  | 1 .. <maxnoofTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>On-demand PRS information [FFS] | [FFS] |  | 9.3.1.b |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPs | Maximum no. of TRPs in a gNB-DU Value is 65535. |

Next Change

#### 9.3.1.a	Requested PRS Transmission Characteristics [FFS]

This IE contains the requested PRS configuration for transmission by the gNB-CU.

Editor’s Note: All details of this IE are FFS

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PRS Resource Set List |  | 1..<maxnoofPRSresourceSet> |  |  |
| >PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >PRS bandwidth | M |  | INTEGER(1..63) | 24,28,…,272 PRBs |
| >Resource Set Periodicity | M |  | ENUMERATED(4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…) |  |
| >PRS Resource List | M | 1..<maxnoofPRSresources> |  | NR-DL-PRS-Resource-r16 as defined in TS 37.355 [14] |
| >>PRS Resource ID | M |  | INTEGER(0..63) |  |
| >> CHOICE QCL Info | O |  |  |  |
| >>>SSB |  |  |  |  |
| >>>>NR PCI | M |  | INTEGER(0..1007) |  |
| >>>> SSB Index | O |  | INTEGER(0..63) |  |
| >>>DL-PRS |  |  |  |  |
| >>>>QCL Source PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>>>QCL Source PRS Resource ID | O |  | INTEGER(0..63) | If it is absent, the QCL source PRS resource ID is the same as the PRS resource ID |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |

#### 9.3.1.b	On-demand PRS information [FFS]

This IE contains the updated PRS configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS |  |  |  |  |
| PRS Configuration [FFS] | FFS |  | 9.3.1.177 |  |

Next Change

ASN.1 to be provided later

End of Text Proposal for TS 38.473 BL CR