3GPP TSG-RAN WG3 #114-e																		   R3-214751

1-11 Nov 2021

Online

Agenda Item:	22.4

Source:	ZTE

Title:	(TP for 38.300 for Introduction of NR MBS) MBS broadcast service continuity

Document for:	Approval

- Introduction

The TP to be agreed is based on the progress from the following CB.

CB: # MBS6_BroadcastService

- Whether to exchange following info between gNB via Xn: SAI, MBS service group ID, list of ongoing TMGIs?

- Provide the MBS SAIs belonging to a TAI supported by the NG-RAN node to AMF5442?

- Whether to reuse LTE MBS frequency layer prioritization in NR based on RAN2 agreements?

- Details on SAI? F1AP and E1AP impact?

- Capture agreements and open issues

(CATT - moderator)

Summary of offline disc R3-215892 rev in R3-215990

Meanwhile, the following proposal is agreed according to CB: # MBS3_others.

Proposal 1:  For a location dependent broadcast session, Area session ID is included in the NGAP MBS session resource setup procedure to indicate MBS service area information.

- Text Proposal for TS38.300

<<<<<<<<<<<<<<<<<<<< Start Change >>>>>>>>>>>>>>>>>>>>

16.x.5.4	Broadcast Mobility

Editor’s Note: intends to cover information related to broadcast exchanged during mobility.

NR MBS supports MBS frequency layer prioritization for broadcast MBS sessions.

If SAI/ID is used to identify an MBS service area, it is configured by OAM at the NG-RAN.

The SAI/IDs of the neighbouring cell are provided by Xn signalling (i.e. Xn Setup and NG-RAN node Configuration Update procedures). (Note: The definition of the SAI/ID is FFS).

For a location dependent broadcast session, Area session ID is included in the NGAP MBS session resource setup procedure to indicate MBS service area information.

<<<<<<<<<<<<<<<<<<<< End Change >>>>>>>>>>>>>>>>>>>>