3GPP TSG-RAN WG3 Meeting #114-e	R3-216207

# E-meeting, 1-11 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.420 | CR | 0023 | rev | 0 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CPAC BL CR to TS 38.420 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2021-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add the support of CPAC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the CPC cancel procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CPAC cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------Start of Changes-----------------------------------

6	Xn interface procedures

6.1	General

The Xn interface supports procedures over the control plane (Xn-C) and user plane (Xn-U).

6.2	Control plane protocol procedures

6.2.1	Mobility management procedures

The mobility management procedures are used to manage the UE mobility in Connected or RRC_Inactive modes:

-	Handover Preparation

-	Handover Cancel

-	SN Status Transfer

-	Retrieve UE Context

-	RAN Paging

-	Xn-U Address Indication

-	UE Context Release

-	Handover Success Indication

-	Conditional Handover Cancel

6.2.2	Dual Connectivity procedures

The dual connectivity procedures are used to add, modify and releases resources for the operation of Dual Connectivity:

-	S-NG-RAN-node Addition Preparation

-	S-NG-RAN-node Reconfiguration Completion

-	M-NG-RAN-node initiated S-NG-RAN-node Modification Preparation

-	S-NG-RAN-node initiated S-NG-RAN-node Modification

-	M-NG-RAN-node initiated S-NG-RAN-node Release

-	S-NG-RAN-node initiated S-NG-RAN-node Release

-	S-NG-RAN-node Counter Check

-	RRC Transfer

-	Notification Control Indication

-	Activity Notification

-	Secondary RAT Data Usage Report

- 	Conditional PSCell Change Cancel

6.2.3	Global procedures

The global procedures are used to exchange configuration level data between two NG-RAN nodes, or to remove Xn connectivity between two NG-RAN nodes in a controlled manner:

-	Xn Setup

-	NG-RAN-node Configuration Update

-	Xn Removal

6.2.4	Interface Management procedures

The interface management procedures are used to align resources between two NG-RAN nodes in the event of failures, and to report detected protocol errors:

-	Reset

-	Error Indication

6.2.5	Energy saving procedures

-	Cell Activation procedure: enables an NG-RAN node to request the activation of a previously deactivated cell hosted in another NG-RAN node.

6.2.6	Resource coordination procedures

-	E-UTRA - NR Cell Resource Coordination procedure: enables an ng-eNB and a gNB to interact for resource coordination purposes.

6.2.7	UE Tracing procedures

The following procedures are used to trace the UE:

-	Trace Start procedure

-	Deactivate Trace procedure

6.2.8	Load management procedures

The load management procedures are used by NG-RAN nodes to indicate resource status, overload and traffic load to each other.

-	Resource Status Reporting Initiation

-	Resource Status Reporting

6.2.9	Data exchange for self-optimisation procedures

The data exchange for self-optimisation procedures are used to transfer failure and mobility related information among NG-RAN nodes to enable self-optimisation

-	Failure Indication

-	Handover report

-	Mobility Settings Change

-	Access and Mobility Indication

6.3	User plane protocol procedures

The user plane protocol procedures are used to exchange user plane information between Xn-U protocol peers:

-	Transfer of Downlink User Data procedure: enables the node hosting the NR PDCP entity to provide user plane information to the corresponding node.

-	Downlink Data Delivery Status procedure: enables the corresponding node to provide feedback to the node hosting the NR PDCP entity.

-	Transfer of Assistance Information: enables the corresponding node to provide assistance information to the node hosting the NR PDCP entity.

-	Transfer of PDU Session Information procedure: enables an NG-RAN node to provide user plane information associated with the forwarding of data towards a peer NG-RAN node, when using PDU session tunnels.

-----------------------------------End of Changes-----------------------------------