3GPP TSG-RAN WG3 Meeting #114-e	R3-215979

Online, 1 – 11 November 2021

Agenda item:	21.3

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR_IIOT_URLLC_enh BL CR for TS 38.423) Survival Time

Document for:	Discussion and Decision

# 1	Introduction

This XnAP TP captures the agreements from CB # NRIIOT2_NewQoS:

The maximum value of the Survival Time is 1.92s (i.e., option2).

# A	Appendix: Text Proposal for TS 38.423 BL CR

Start of Text Proposal for TS 38.423 BL CR

#### 9.2.3.xxx	Survival Time

This IE provides the Survival Time for a TSC QoS flow (see TS 23.501 [7]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..1920000, …) | Expressed in units of 1 us. |

Next Change

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

********************************

Skip to the next change

********************************

SurvivalTime ::= INTEGER (0..1920000, ...)

End of Text Proposal for TS 38.423 BL CR