3GPP TSG-RAN3 Meeting #114-bis-e	R3-220040

Electronic meeting, 17 - 26 January 2022

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0826 | rev | 3 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of QoE information transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE |  |  |  |  |  |  | Date: |  | 2022-1-3 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Support of QoE information transfer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For RAN visible QoE, the QoE information can be used for QoE aware scheduling. to support the QoE inforamtion transfer, the following changes are made: 1. New abbreviation of QoE is introduced. 2. New procedure QoE Information Transfer is added. 3. New IEs is added in QOE INFORMATION TRANSFER messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | gNB-DU cannot know the QoE informaiton. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 8. 1, 8.x, 9.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Rev0, - Add abbreviation of QoE. - Add procedure of QoE Information Transfer. - Add new IEs in QOE INFORMATION TRANSFER messages. Rev1, add FFS on DRB list in QOE INFORMATION TRANSFER message. Rev2, - add QoE Information List IE - change “DRB List [FFS]” to “DRB related info [FFS]” - add editior note for further refinement. Rev3, resubmission to RAN3#114-bis-e. |  |  |  |  |  |  |  |  |

<Start of change>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

ARP	Antenna Reference Point

ARPI	Additional RRM Policy Index

BH	Backhaul

CAG	Closed Access Group

CN	Core Network

CG	Cell Group

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

NID	Network Identifier

NPN	Non-Public Network

NSSAI	Network Slice Selection Assistance Information

posSIB	Positioning SIB

PNI-NPN	Public Network Integrated NPN

QoE	Quality of Experience

RANAC	RAN Area Code

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RRC	Radio Resource Control

RSRP	Reference Signal Received Power

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

TRP	Transmission-Reception Point

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

Z-AoA	Zenith Angles of Arrival

<Next change>

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Response message |  | Response message |  |
| Reset | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |  |
| --- | --- | --- |
| Error Indication | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging | PAGING |  |
| Notify | NOTIFY |  |
| PWS Restart Indication | PWS RESTART INDICATION |  |
| PWS Failure Indication | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |
| Deactivate Trace |  | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |
| Access Success |  | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |  |
| QoE Information Transfer | QOE INFORMATION TRANSFER |  |

Editor’s note: further refinement is possible. This note applies to the whole BL CR.

<Next change>

## 8.X	QoE Information Transfer procedures

### 8.X.1	QoE Information Transfer

#### 8.X.1.1	General

The purpose of the QoE Information Transfer procedure is to transfer RAN visible QoE information from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

#### 8.X.1.2	Successful operation

Figure 8.X.1.2-1: QoE Information Transfer procedures.

The gNB-CU initiates the procedure by sending the QOE INFORMATION TRANSFER message to the gNB-DU.

The gNB-DU considers QoE information for scheduling.

#### 8.X.1.3	Abnormal Conditions

Not applicable.

<Next change>

### 9.2.X	QoE Information Transfer messages

#### 9.2.X.1	QOE INFORMATION TRANSFER [FFS]

This message is sent by a gNB-CU to a gNB-DU, to indicate RAN visible QoE information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| QoE Information List |  |  |  |  |  |  |
| >QoE Information Item |  |  | 1..<maxnoofQoEInformation> |  |  |  |
| >>QoE Metric List | O |  | FFS |  | - | - |
| >>>Buffer level | O |  | FFS |  |  |  |
| >>>Playout delay | O |  | FFS |  |  |  |
| >DRB related info [FFS] |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoEInformation | Maximum no. of QoE information for one UE, the maximum value is FFS |

<End of change>