3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220167

# Online, January 17th – 26th 2022

Agenda Item:	15.2.1.1

Source:	Ericsson

Title:	(TP for QoE BL CR for TS 38.300): Stage-2 Aspects of NR QoE Management

Document for:	Agreement

# Introduction

In this paper we discuss certain remaining stage-2 aspects of NR QoE management. A TP for the QoE BL CR for TS 38.300 is presented in the Annex.

# Discussion

Some further changes are needed on the QoE BL CR for TS 38.300, as follows:

- There are several instances of redundant (i.e., repeated) text.

- The following important RAN3 agreements need to be incorporated into the text:

  - The network can replace a configuration with another one of m- or s-based configuration by deactivating an existing measurement and configuring another measurement of the same configuration type.

  - RAN3 assumes that the OAM will never provide the same QoE Reference to different QoE configurations irrespective of QoE type.

  - A UE should continue an ongoing measurement once it leaves the Area, unless the network indicates to the UE to release the QoE configuration.

  - In case of mobility to a target node not supporting QoE, the target node can release the QoE configuration.

- Some concepts require further clarification (e.g., RAN visible QoE).

- The terminology needs to be consistent (e.g., “gNB” should be replaced by “NG-RAN node”)

A TP for the QoE BL CR for TS 38.300 incorporating the above changes is presented in the Annex.

- Proposal: Agree the TP for the QoE BL CR for TS 38.300 presented in the Annex.

# Annex A: TP for QoE BL CR for TS 38.300

-------------------------------------------Change 1-------------------------------------------

# X 	QoE Measurement Collection

## X.1	 Overview

The QoE Measurement Collection function enables the collection of application layer measurements from the UE.

The application layer measurements are supported for the following service types:

-	Streaming services.

-	MTSI services.

-	VR services.

Both signalling based and management based QoE measurement collection are supported.

## X.2	 QoE Measurement Configuration

### X.2.1	 QoE Measurement Collection Activation and Reporting

The feature is activated in the NG-RAN either by direct configuration from the OAM system (management-based activation), or by signalling from the OAM via the Core Network (signalling-based activation). One or more QoE measurement collection jobs can be activated at a UE per service type, and each QoE measurement configuration is uniquely identified by a QoE Reference. The OAM will not provide the same QoE Reference to different QoE configurations, irrespective of QoE measurement type.

For signalling-based QoE measurements, the OAM initiates the QoE measurement activation for a specific UE via the Core Network, and the NG-RAN node receives one or more QoE measurement configurations by means of UE-associated signalling. The QoE measurement configuration for signalling-based activation includes application layer measurement configuration list and the corresponding information for QoE measurement collection. Each application layer measurement configuration is encapsulated in a transparent container. The NG-RAN node forwards the corresponding QoE measurement configuration(s) to the UE in a downlink RRC message, as specified in TS 38.331 [12].

For management-based QoE measurement activation, the OAM sends one or more QoE measurement configurations to the NG-RAN node. The QoE measurement configuration for management-based activation includes application layer measurement configuration list and the corresponding information for QoE measurement collection. Each application layer measurement configuration is encapsulated in a transparent container. The NG-RAN node selects the UE(s) that meet the required QoE measurement capability, Area Scope and Slice Scope.

Editor’s Note: The detailed content of QoE configuration needs to be revisited based on the outcome of each subtopic in RAN2 and RAN3.

The UE reports QoE measurement results to the NG-RAN node in an uplink RRC message, as specified in TS 38.331 [12]. The NG-RAN node transmits the QoE report and the corresponding QoE Reference ID to the MCE.

The application layer measurement configuration and measurement reporting are supported in RRC_CONNECTED state only. An RRC identifier conveyed in the RRC signalling is used to identify the QoE configuration and report between the NG-RAN node and the UE. The RRC identifier is mapped to the QoE Reference in the NG-RAN node. The QoE measurement report is forwarded to the OAM together with the QoE Reference. The NG-RAN node can release multiple application layer measurement configurations from the UE in one RRC message at any time.

### X.2.2	 QoE Measurement Collection Deactivation

QoE Measurement Collection deactivation permanently stops all or some of QoE measurement collection jobs towards a UE, resulting in the release of the corresponding QoE measurement configuration(s) in the UE. The deactivation of QoE measurement collection is supported by using UE-associated signalling. A list of QoE References is used to deactivate the corresponding QoE measurement collection job(s).

Upon reception of a QoE configuration release message, the UE discards any unsent QoE reports corresponding to the released application layer measurement configuration. The UE discards the reports received from application layer when it has no associated QoE configuration configured.

The network can replace a configuration with another one by deactivating an existing measurement and configuring another measurement of the same configuration type.

### X.2.3 RAN Overload QoE Handling

QoE Measurement Collection pause/resume procedure is used to pause/resume the reporting for all QoE reports or to pause/resume QoE reporting per QoE configuration in a UE in case of RAN overload.

Application layer measurements can be paused by the NG-RAN node using a downlink RRC message that is used to temporarily stop application layer measurement reports from being sent from the UE to the network.

### X.2.4 QoE Measurement Handling in RRC_IDLE and RRC_INACTIVE States

If the UE enters RRC_INACTIVE state, the UE AS configuration for the QoE is stored in the UE Inactive AS context. If the UE enters RRC_IDLE state, the UE releases all the QoE measurement configurations.

Editor’s Note: The structure of TS 38.300 BLCR needs further discussion.

### X.2.5	 Per-slice QoE Measurement

When a service is provided within a configured slice, the QoE Measurement for this service type could also be configured together with the corresponding slice scope, so that the user experience of this service could also be evaluated on a per-slice basis. The UE includes the slice ID inside the QoE report container when reporting QoE measurement result (pending SA4 decision).

## X.3	 QoE Measurement Continuity for Mobility

The QoE Measurement Collection continuity for intra-system intra-RAT mobility is supported, with the Area Scope parameters configured by the OAM, where the network is responsible for keeping track of whether the UE is inside or outside the Area. A UE should continue an ongoing measurement once it leaves the Area unless the network indicates to the UE to release the QoE configuration.

For RRC_CONNECTED state mobility, the source NG-RAN node may transmit the QoE measurement configuration(s) or the information related to the configuration(s) of a specific UE to the target NG-RAN node via XnAP or NGAP. For RRC_INACTIVE state mobility, QoE measurement configuration(s) of a specific UE can be restored from the node hosting the UE context when it resumes to RRC_CONNECTED state. Multiple sets of QoE measurement configurations should be supported during mobility.

For signalling-based QoE, at handover to a target NG-RAN node which supports QoE, the target NG-RAN node decides which QoE configurations to keep and which to release, e.g., based on QoE configuration information received from the source NG-RAN node in Xn/NG signalling.

When the UE resumes the connection in a NG-RAN node supporting QoE, the UE keeps the QoE measurement configuration(s) indicated by the target NG-RAN node and releases the QoE measurement configuration(s) which are not indicated by the target NG-RAN node for restoration. When the UE resumes the connection in a NG-RAN node not supporting QoE, the UE releases all QoE measurement configurations.

Editor’s Note: Further descriptions pending the work progress in RAN2.

## X.4	 RAN Visible QoE Measurements

RAN visible QoE measurements are a lightweight form of QoE measurements. RAN visible QoE measurements are configured by the NG-RAN node, where a subset of QoE metrics is reported from the UE as explicit IEs readable by the NG-RAN node.  RAN visible QoE metrics could be utilized by the NG-RAN node for optimization. RAN visible QoE measurements are supported for the DASH streaming and VR services.

Editor’s Note: This section is pending work in other WGs

## X.5	 Alignment of MDT and QoE Measurements

- Radio-related measurements may be collected via immediate MDT for all types of supported services for the purpose of QoE analysis. The immediate MDT can be configured before the QoE measurement is configured or the two could be configured simultaneously. The TCE/MCE is responsible for the correlation of the immediate MDT results and the QoE measurement results collected at the same UE. The NG-RAN node includes time stamp information and related ID information for the MCE/TCE to enable the correlation of corresponding measurement results of MDT and QoE.

-------------------------------------------End of changes-------------------------------------------