3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220169

# Online, January 17th – 26th 2022

Agenda Item:	15.2.2

Source:	Ericsson

Title:	Mobility Support for NR QoE Management

Document for:	Agreement

# Introduction

At the RAN3#114-e meeting, the following was agreed with respect to NR QoE mobility support:

For a service type, in Rel17, a UE can be simultaneously configured with multiple s- and/or m-based configurations, as long as the maximum number of simultaneous configurations at a UE is not exceeded.

The network can replace a configuration with another one of m- or s-based configuration by deactivating an existing measurement and configuring another measurement of the same configuration type.

RAN3 assumes that the OAM will never provide the same QoE Reference to different QoE configurations irrespective of QoE type.

A UE should continue an ongoing measurement once it leaves the Area, unless the network indicates to the UE to release the QoE configuration.

The following information about an m-based measurement configuration should be explicitly passed to the target during handover:

- The Measurement Configuration Application Layer ID corresponding to the QoE Reference.

- MDT Alignment info.

- MCE IP address.

- WA: Measurement status.

For m-based QoE, the QoE configuration container (XML file) is not included in NGAP and XnAP handover messages. FFS for s-based QoE.

The following information is explicitly passed to the target at handover:

- QoE reference.

- MCE IP address.

- The Measurement Configuration Application Layer ID corresponding to the QoE Reference.

- WA: Measurement status.

- MDT Alignment info.

- Area Scope.

- Slice list.

For both s- and m-based QoE, the QMC Information IE (which does not include the QoE configuration container) is explicitly included in the XnAP RETRIEVE UE CONTEXT RESPONSE.

In case of mobility to a target node not supporting QoE, the target node can release the QoE configuration.

In this paper we discuss the way forward with respect to the above agreements and the remaining issues related to NGAP and XnAP aspects of mobility support for QoE management. A corresponding TP for QoE BL CR for TS 38.423 is presented in R3-220170, and relevant changes are also mirrored in the TP for QoE BL CR for TS 38.413 in R3-220168.

# Discussion

In the following, we discuss the remaining open issues related to mobility support for NR QoE management.

## The presence of measurement type and measurement status indications in XnAP and NGAP handover messages

In the context of mobility, it is essential that the target RAN node handles appropriately the QMC configurations at the incoming UE, regardless of whether the corresponding measurements were initiated or not. For example, the target node should take the right decisions related to overriding, i.e., replacement of one QMC configuration with a new one. We note that a target RAN node is aware of the number of simultaneous QMC configurations at the UE (indicated by the UE Application Layer Measurement Information IE) and it is also aware of the maximum number of simultaneous QMC configurations that the UE can sustain.

So, the target RAN node may receive an incoming UE for which an m-based QoE measurement has been configured for one service type (e.g., streaming). Upon instruction from CN, the target RAN would like to configure the same UE for another QoE measurement for another service type (e.g., MTSI). As long as the maximum number of simultaneous configurations for the UE is not exceeded, the solution works and fulfils the following agreement taken at the RAN3#114-e meeting:

For a service type, in Rel17, a UE can be simultaneously configured with multiple s- and/or m-based configurations, as long as the maximum number of simultaneous configurations at a UE is not exceeded.

At RAN3#113-e we have also agreed that:

Signalling based QoE can override an existing management based QoE configuration.

When the maximum number of simultaneous configurations is reached, and the target RAN node has just received an s-based QoE configuration for the UE, the target node can release one of the configured QoE configurations and configure the newly arrived s-based QMC to the UE. To support the above, i.e., to be able to select which of the QMC configuration(s) it should release to make room for the newly arrived s-based configuration, the target RAN node needs to be informed about the measurement type for each measurement at the UE, i.e., whether the measurement is m-based or s-based. Therefore, we prefer that, as part of XnAP signalling, the QoE Measurement Type IE is indicated for each QoE measurement configured for a UE.

Proposal 1: The presence of QoE Measurement Type IE in the UE Application Layer Measurement Information IE in the QoE BL CR for TS 38.423 is mandatory.

Meanwhile, given that the UE Application Layer Measurement Information IE is present in both the NGAP messages for initial configuration and in the NG-based handover messages, the presence of QoE Measurement Type IE in the NGAP version of the UE Application Layer Measurement Information IE should be optional. This is because the indication is not needed at initial configuration, given that the RAN node can infer the measurement type based on whether the configuration is delivered via NGAP signalling or directly from the OAM.

Proposal 2: The presence of QoE Measurement Type IE in the UE Application Layer Measurement Information IE in the QoE BL CR for TS 38.413 is optional.

Another FFS to resolve is whether QoE Measurement Status IE is needed for mobility handling. In our view, this IE can be used by a target RAN to take more educated decisions in case the incoming UE is already configured with the maximum number of simultaneous configurations, in case the target RAN node would like to configure an additional configuration. For example, an m-based QoE configuration can be released without losing any data if the associated application session has not started, or if it has just stopped and QoE reports have been collected for it.

Note also that, for s-based QoE, the AMF is not aware of whether an application session is ongoing or not, and, consequently, the AMF is not aware of whether the corresponding QoE measurements are ongoing or not, so it cannot pass on this information to the target RAN during an NG-based handover. Therefore, this IE is needed for mobility handling of both s-based QoE and m-based QoE configurations.

We also note that providing the information about an “ongoing” QoE measurement can be useful for the target RAN node, since it is an indication of potential QoE reports that may be received from the UE.

Proposal 3: The presence of QoE Measurement Status IE in the UE Application Layer Measurement Information IE in the QoE BL CR for TS 38.423 is mandatory, to indicate that a QoE measurement is ongoing.

## The presence of the QoE configuration XML container in mobility signalling

A QoE configuration sent to a UE contains an application layer measurement configuration container (i.e., a file in XML format), which the RAN cannot interpret.

For m-based QoE mobility handling, the XML file is provided to the UE from OAM to each RAN node in the Area, hence it is OK to not exchange it over Xn or NG at mobility. This is captured in the following RAN3#114-e agreement:

For m-based QoE, the QoE configuration container (XML file) is not included in NGAP and XnAP handover messages. FFS for s-based QoE.

For s-based QoE mobility handling, we think that whether the XML file should be passed from the source RAN node to the target RAN node depends on some conditions, more specifically on:

- Whether the source RAN node has configured the UE for QoE measurements, i.e., whether the QoE XML file has been sent to the UE.

- Whether the source RAN node is aware that the UE was previously configured for QoE measurements, by a previously serving node.

For the first case, we recognize a problematic scenario, where a RAN node can receive as part of the QoE configuration parameters (outside the XML file) an Area Scope containing only a subset of its cells. If, in this scenario, the UE moves only in the RAN node’s cells that are not in the Area Scope (meaning that the RAN node has the configuration for the UE but does not configure the UE with this QoE configuration), and then the UE moves to another RAN node, the attempt to configure the QoE after mobility cannot succeed. The scenario is shown in Figure 1 below. We see two possible solutions:

- The source RAN node receiving an s-based QoE configuration sends it to the UE prior to the handover.

- The RAN node passes on the XML file containing the QoE configuration to the target RAN node.

The first option is probably better addressed by implementation, while the second is not needed all the time - it is not needed whenever the source RAN node is aware that UE has received, either from the RAN node itself, or from another RAN node, the XML file containing the configuration.

We propose the following solution:

- The source RAN node can send the XML file for a certain service type if it knows that such file was not previously sent to the UE. The condition can be indicated in a compact way via the QoE Measurement Status IE, using a codepoint “configured” or “ongoing” (as indicated in the current BL CR), or similar.

- So, if the QoE Measurement Status IE is set to “configured”, or “ongoing” then the source RAN node will not send the XML file to the target RAN, since, either:

  - It has itself configured the UE for QoE measurements

  - The UE was configured by some other RAN node, and the QoE measurements are ongoing.

- To account for the case when the source RAN node has not configured the UE for s-based QoE, or, after a series of mobility events, the UE is still not configured for s-based QoE (because, e.g., it connected to the cells outside of Area Scope), we propose to add an additional codepoint “not configured” to the QoE Measurement Status IE and let the source RAN node pass the XML file to the target RAN if the QoE Measurement Status IE has this value.

Figure 1: A scenario where a RAN node can receive as part of the QoE configuration parameters an Area Scope containing only a subset of its cells. If, in this scenario, the UE is not configured for QoE, and moves to another RAN node, the attempt to configure QoE after mobility cannot succeed.

Proposal 4: The Container for Application Layer Measurement Configuration IE is present, for s-based QoE, in the XnAP and NGAP handover messages if the value of the QoE Measurement Status IE sent to the target RAN node indicates that that UE has not yet been configured for QoE measurements.

Proposal 5: Introduce an additional codepoint “not-configured” in the QoE Measurement Status IE, to indicate that UE has not yet been configured for QoE measurements for a certain service type.

## The “Alignment with MDT required” indication

At the RAN3#114-e meeting, to enable the alignment of QoE and MDT measurements the MDT Alignment Information CHOICE structure was agreed and included in the QoE BL CR for TS 38.423 in R3-216226 and in the QoE BL CR for TS 38.413 in R3-216279 (as an FFS).

For a more general mechanism for the alignment of QoE and MDT, where MDT can be running independently of QoE measurements, we think it would be beneficial to introduce an option where the alignment is possible between QoE and any available MDT measurements. In other words, if there is currently no ongoing MDT measurement at the UE, the RAN can configure the UE with an m-based MDT measurement at the same time that it configures the UE with the s-based QoE measurement. If an MDT measurement is already ongoing at the UE, the RAN node aligns it with the s-based QoE measurement by adding the MDT-related identifiers into the s-based QoE reports and timestamping both the s-based QoE reports and MDT reports from the same UE, as per the RAN3#114-e agreement. To enable the alignment with any available MDT measurement, a generic indication can be signalled by the OAM to the NG-RAN node (via CN), indicating “alignment with any available MDT”, instead of indicating the exact Trace ID or Trace Reference.

Proposal 6: Extend the MDT Alignment Information CHOICE structure in the QoE BL CRs for TS 38.413 and TS 38.423 with an indication requesting the receiving NG-RAN node to align the s-based QoE measurement with any available MDT measurement.

## RAN visible QoE configuration indication at mobility

At the RAN3#114-e meeting, it was agreed to introduce the RAN Visible QoE Metric IE in the NGAP UE Application Layer Measurement Information IE, to indicate to the RAN node which RVQoE metrics it may request from the UE. We think that this information is needed in the XnAP/NGAP handover signalling as well. Moreover, we think that indicating the entire RVQoE configuration from the source to the target would be useful for the target node. Each RAN node serving the UE should have full control over the RVQoE configuration, but one benefit of passing the RVQoE configuration at mobility would be that, if target node decides to use the same RVQoE configuration as the source RAN node, the UE does not need to be reconfigured. In our view, the RVQoE configuration IE to be passed to the target, should contain the information listed in the below proposal.

Finally, for the purpose of handover performance evaluation, we think that the source node should be able to request the target node to forward to the source the first RVQoE report it receives from the UE.

Proposal 7: Introduce the RAN Visible QoE Configuration Information IE in both the NGAP and XnAP UE Application Layer Measurement Information IE, with the following content (and liaise RAN2 for RRC support):

- Reporting Interval.

- Configured RAN Visible QoE Metrics (i.e., Buffer Level and/or Playout Delay).

- An indication of whether the target should forward to the source the first RVQoE report it receives from the UE.

Please note that the RVQoE measurement ID is in fact the RRC ID, and it is present in the legacy QoE part of the IE, and so is the service type.

The proposals above are included in the TPs for the QoE BL CRs for TS 38.423 and TS 38.413 in R3-220170 and R3-220168, respectively.

Proposal 8: Agree the TP for the QoE BL CR for TS 38.423 in R3-220170.

# Conclusion

In this paper, following is proposed:

Proposal 1: The presence of QoE Measurement Type IE in the UE Application Layer Measurement Information IE in the QoE BL CR for TS 38.423 is mandatory.

Proposal 2: The presence of QoE Measurement Type IE in the UE Application Layer Measurement Information IE in the QoE BL CR for TS 38.413 is optional.

Proposal 3: The presence of QoE Measurement Status IE in the UE Application Layer Measurement Information IE in the QoE BL CR for TS 38.423 is mandatory, to indicate that a QoE measurement is ongoing.

Proposal 4: The Container for Application Layer Measurement Configuration IE is present, for s-based QoE, in the XnAP and NGAP handover messages if the value of the QoE Measurement Status IE sent to the target RAN node indicates that that UE has not yet been configured for QoE measurements.

Proposal 5: Introduce an additional codepoint “not-configured” in the QoE Measurement Status IE, to indicate that UE has not yet been configured for QoE measurements for a certain service type.

Proposal 6: Extend the MDT Alignment Information CHOICE structure in the QoE BL CRs for TS 38.413 and TS 38.423 with an indication requesting the receiving NG-RAN node to align the s-based QoE measurement with any available MDT measurement.

Proposal 7: Introduce the RAN Visible QoE Configuration Information IE in both the NGAP and XnAP UE Application Layer Measurement Information IE, with the following content (and liaise RAN2 for RRC support):

- Reporting Interval.

- Configured RAN Visible QoE Metrics (i.e., Buffer Level and/or Playout Delay).

- An indication of whether the target should forward to the source the first RVQoE report it receives from the UE.

Proposal 8: Agree the TP for the QoE BL CR for TS 38.423 in R3-220170.