3GPP TSG-RAN WG3 Meeting #114-bis-e	R3-220209

01-11 November 2021, E-meeting

Title: 	RAN Intelligence Framework – Further Discussion on remaining issues

Source: 	FUTUREWEI

Agenda item:	18.2

Document Type:	Discussion and approval

# Introduction

At RAN3 meeting 114e, the following RAN Intelligence Framework was agreed as specified in TR37.817[1]. The following open issues were identified as FFS:

- FFS on whether model performance evaluation / generating of model performance metrics is performed in Model Inference.

This contribution discusses the above AI/ML Model performance evaluation issue.

# Discussion

This section discusses the following topics:

- AI/ML Model performance evaluation process

### AI/ML Model Performance Evaluation after deployment

The Functional Framework for RAN Intelligence agreed from RAN3-113e meeting is shown in Figure 4.2-1 of [1] and captured below.

Figure 2.1-1: Functional Framework for RAN Intelligence as depicted in Figure 4.2-1 of [1]

After a trained/validated/tested AI/ML model is deployed to the Model Inference function, even though the details may depend on the type of learning that the AI/ML model is designed/realized for, in general, an on-going model performance evaluation monitoring procedure will take place. The purpose is to understand how well the trained model can generalize to field / real-world data and to determine whether/when to retrain/tune the model. This performance monitoring procedure may be performed on a periodic basis as the distribution of the field data may be different from the test dataset used in the Model Training phase, and/or the distribution may change over time.

Figure 2.1-2: Logical procedure of generating Model Performance feedback at the Model Inference function.

To generate model performance metrics/KPIs, typically, both the inference/prediction results and the ground truth/labels have to be available. As the Model Inference function is responsible to generate the Model inference/prediction results, it is logical for it to perform the model evaluation and generate the desired model performance metrics. The Model Inference function may need to acquire the ground truth data from the Data Collection function/entity. The ground truth data may belong to part of the feedback received from the Actor. The logical procedure is illustrated in Figure 2.1-2.

Observation 1: It is logical for the Model Inference function to perform model performance evaluation after the model deployment as it already has the model inference results.

Observation 2: Additional information/ground truth, which may be part of or derived from the feedback of the Actor may be needed at the Model Inference function for generating the model performance feedback/metrics.

Proposal 1: When applicable, the Model Inference function performs the model performance evaluation after the model deployment using the inference results and additional information acquired.

Proposal 2: When applicable, the Model Inference function generates the model performance feedback by requesting additional information from the Data Collection function, e.g., feedback from the Actor. The derived model performance feedback/metrics may then be delivered to the Model Training function or other functions that subscribe to receive such information.

Proposal 3: Keep the Model Performance feedback line from the Model Inference function to the Model Training function and remove the “FFS”.

# Conclusion

Observation 1: It is logical for the Model Inference function to perform model performance evaluation after the model deployment as it already has the model inference results.

Observation 2: Additional information/ground truth, which may be part of or derived from the feedback of the Actor may be needed at the Model Inference function for generating the model performance feedback/metrics.

Proposal 1: When applicable, the Model Inference function performs the model performance evaluation after the model deployment using the inference results and additional information acquired.

Proposal 2: When applicable, the Model Inference function generates the model performance feedback by requesting additional information from the Data Collection function, e.g., feedback from the Actor. The derived model performance feedback/metrics may then be delivered to the Model Training function or other functions that subscribe to receive such information.

Proposal 3: Keep the Model Performance feedback line from the Model Inference function to the Model Training function and remove the “FFS”.

# References

[1]   TR37.817 v0.4.0, 2021-11, Study on enhancement for Data Collection for NR and EN-DC.