3GPP TSG-RAN WG3 #114bis-e																	   R3-220223

17-26 Jan 2022

Online

Agenda Item:	22.2.5

Source:	ZTE, Qualcomm Incorporated, Samsung, Huawei, Lenovo, Nokia, Nokia Shanghai Bell

Title:	(TP for BL CR for 38.413) Multicast group paging

Document for:	Approval

- Introduction

In the last RAN3 #114 e-meeting, we have deeply discussed the Multicast Group Paging (refer to R3-216212), then we have achieved the following progress.

RAN3 shall support Option 2 (i.e., Paging for multicast session activation notification is used in the relevant legacy POs for the UEs with non-activated multicast session(s)). It is proposed to include “UE Identity Index value” IE (i.e., 5G-S-TMSI mod 1024 defined in 9.3.3.23 in TS38.413) in the Multicast group paging message. It is proposed to include “Paging DRX” IE in the Multicast group paging message. FFS: Paging DRX is UE specific or Session specific. FFS: paging area shall include per list of UEs to be paged. It is proposed to include “MBS Service Area” IE in the Multicast group paging message, the detail is FFS. The IE“UE RAN Paging Identity” is not included in RAN Multicast Group Paging message. The set of TPs are technically correct

During the offline and online discussion, the set of TPs are technically correct. However, the corresponding CRs as below are noted.

R3-214753 rev [in R3-215972] – agreed 38.413 ZTE R3-215271 rev [in R3-215973] – agreed 38.423 Lenovo R3-215514 rev [in R3-215974] – agreed 38.473 Samsung R3-215296 rev [in R3-215975] – agreed 38.470 LG R3-214755 rev in R3-215976 rev [in R3-216213] – agreed LS to RAN2, CC:SA2

In the subsequent SA2 #148 e-meeting, the MBS counting issue was not discussed in SA2. So that, in this RAN3 e-meeting, the corresponding TP to TS38.413 without any change is resubmitted for agreement.

- TP to TS38.413

----------Start of the First Change--------------

### 8.5.X	Multicast Group Paging

#### 8.5.X.1	General

The purpose of the Multicast Group Paging procedure is to enable the AMF to notify CM-IDLE UEs which have joined an MBS Session about its activation.

The procedure uses non-UE associated signalling.

#### 8.5.X.2	Successful Operation

Figure 8.5.X.2-1: Multicast Group Paging

The AMF initiates the Multicast Group Paging procedure by sending the MULTICAST GROUP PAGING message to the NG-RAN node.

At the reception of the MULTICAST GROUP PAGING message, the NG-RAN node shall perform multicast group paging of the MBS Session identified by the MBS Session ID IE utilising information provided by the AMF.

If the Paging DRX IE is included in the PAGING message, the NG-RAN node shall use it according to TS 38.304 [12].

Editor’s Note:	Detailled parameterisation of the message is FFS.

#### 8.5.1.3	Abnormal Conditions

Void.

*****************Next changes*******************

#### 9.2.4.X	MULTICAST GROUP PAGING

This message is sent by the AMF to trigger multicast group paging for a multicast MBS session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.x |  | YES | ignore |
| MBS Service Area | M |  | (FFS) | Editor’s Note: The type definition of this IE is FFS. | YES | ignore |
| Multicast Paging Area | O |  | 9.3.1.y | Editor’s Note: The presence and the usage of this IE is type definition of this IE is FFS |  |  |
| Paging Priority (FFS) | O |  | 9.3.1.78 | Editor’s Note: The optional inclusion of this IE. | YES | ignore |
| UE Paging List |  | 0..1 |  |  | YES | ignore |
| >UE Paging Item |  | 1..<maxnoofUEsforPaging> |  |  | - |  |
| >> UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| >>Paging DRX | O |  | 9.3.1.90 | Editor’s Note: It is FFS that this IE is per UE or per MBS session. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEsforPaging | Maximum no. of UEs for paging. Value is FFS. |

9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | FFS |  |

#### 9.3.1.y	Multicast Paging Area

This IE is a list of TAIs corresponding to the multicast paging area.

Editor’s Note: The details of the IE is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TAI List for Paging |  | 1..<maxnoofTAIforPaging> |  |  |
| >TAI | M |  | 9.3.3.11 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for paging. Value is FFS. |

----------End of the Change--------------