3GPP TSG-RAN WG3 #114bis-e						R3-220248

E-meeting, 17– 26 January 2022

Agenda Item:  24.2

Source:       NEC

Title:		RACH based SDT discussion

Document for:	dicision

- Introduction

For the realization of the RA based SDT without anchor relocation, in the Xn interface, the already agreed baseline is using the Retrieve UE Context procedure, the new gNB sending Retrieve UE Context Request message with SDT Indicator, and then by returning Retrieve UE Context Failure message when there is no need to have anchor relocation.

The next detail is how to transfer the needed SDT UE Context e.g. SDT related RLC bearer configuration to the new gNB. Last RAN3#114 meeting quoted “SDT related RLC bearer configuration for SDT DRB/SRB should be transferred from anchor gNB to the receiving gNB. How to include those information is FFS.“

- 2. Discussion

For the signalling message aspect over Xn, there are some possible solutions. One solution we are giving here in this contribution is for the consideration when comparing all possible solutions.

See below figure for the case of RA-SDT that has uplink SDT only.

Figure x: SDT without anchor relocation procedure

- Step 2,  New gNB to Anchor gNB: RETRIEVE UE CONTEXT REQUEST message indicate the new SDT Indication IE with value e.g. “RA-SDT-UL-only”.

- Step 2a, Anchor gNB to New gNB: a class 2 signalling message for sending the SDT UE Context e.g. SDT related RLC bearer configuration for SDT DRB/SRB, and UL TNL address information for receiving UL SDT.

- New gNB forward the SDT UL data received from the UE to the Anchor gNB.

Regarding the step 2a, a class 2 signalling message, if to use the existing XN-U ADDRESS INDICATION message, then there will be no need to add new signallng only for this purpose, which can then have less impact on the specification.

Proposal 1: the signaling aspect in Xn for SDT without anchor relocation, reuse of Retrieve UE Context procedure and also a class 2 message from Anchor gNB to New gNB to give e.g. SDT related RLC bearer configuration and UL TNL address.

Proposal 2: the Class 2 message can be either existing XN-U ADDRESS INDICATION message or new class 2 signalling message.

- 3. proposal

Proposal 1: the signaling aspect in Xn for SDT without anchor relocation, reuse of Retrieve UE Context procedure and also a class 2 message from Anchor gNB to New gNB to give e.g. SDT related RLC bearer configuration and UL TNL address.

Proposal 2: the Class 2 message can be either existing XN-U ADDRESS INDICATION message or new class 2 signalling message.

References:

R3-216229 Summary of Offline Discussion on CB: # SDT1_RACHbased

R3-216276 RA-SDT BLCR to TS 38.423

Example of the given changes to TS38.423.

- 9.1.1.11	XN-U ADDRESS INDICATION

This message is either sent by the new NG-RAN node to transfer data forwarding information to the old NG-RAN node, or by the M-NG-RAN node to provide either data forwarding or Xn-U bearer address related information for SN terminated bearers to the S-NG-RAN node.

Direction: new NG-RAN node  old NG-RAN node, M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node | YES | ignore |
| Xn-U Address Information per PDU Session Resources List |  | 1 |  |  | YES | reject |
| >Xn-U Address Information per PDU Session Resources Item |  | 1..<maxnoofPDUSessions> |  |  | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>Data Forwarding Info from target NG-RAN node | O |  | Data Forwarding Info from target NG-RAN node 9.2.1.16 |  | – |  |
| >>Secondary Data Forwarding Info from target NG-RAN node List | O |  | 9.2.1.31 | This IE would be present only when the target M-NG-RAN node decide to split a PDU session between MN and SN | YES | ignore |
| >>PDU Session Resource Setup Complete Info – SN terminated | O |  | 9.2.1.30 |  | – |  |
| >>DRB IDs taken into use | O |  | DRB List 9.2.1.29 | Indicating the DRB IDs taken into use by the target NG-RAN node, as specified in TS 37.340 [8]. | YES | reject |
| >>Data Forwarding Info from target E-UTRAN node | O |  | 9.2.1.35 |  | YES | ignore |
| CHO MR-DC Indicator | O |  | ENUMERATED (true, ...) | Indicating that the XN-U ADDRESS INDICATION message is for Conditional Handover, as specified in TS 37.340 [8]. | YES | reject |
| CHO MR-DC Early Data Forwarding Indicator | O |  | ENUMERATED (stop, ...) |  | YES | ignore |
| SDT information Transfer- Old->Anchor | O |  |  |  |  |  |
| >RLC Bearer Configuration | M |  | OCTET STRING | RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 | YES | ignore |
| >SDT DRB Item | 1 |  |  |  |  |  |
| >>SDT DRB ID list |  |  |  |  |  |  |
| >>>DRB ID |  |  |  |  |  |  |
| >>>DL TNL address |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSsessions | Maximum no. of PDU sessions. Value is 256 |