3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220351

Online, 17 – 26 January 2022

Agenda item:	19.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR_pos_enh BL CR for TS 38.455) Associating ARP location with UL measurements

Document for:	Discussion and Decision

# 1	Introduction

RAN1 has discussed the association of ARP (Antenna Reference Point) location information with gNB/TRP UL measurements, and sent an LS to RAN3 in R3-220089 [1] to indicate the following RAN1 agreement:

Agreement - ARP location is associated with UL measurements for NR Positioning (UL AOA, UL-RTOA, UL SRS-RSRP, UL SRS-RSRPP and gNB Rx-Tx time difference measurements) - Use of ARP ID for potential overhead reduction in NRPPa signaling is up to RAN3 - Send LS to RAN3 to enable relevant signaling in RAN3 specification

The LS requests RAN3 to consider the agreement in ongoing work on NR positioning enhancements and implement relevant NRPPa signaling. In this paper, we evaluate the RAN3 impacts of the RAN1 agreement and provide a text proposal for NRPPa.

# 2	Discussion

The RAN1 agreement indicates that the ARP location information is applicable to all the UL measurements (i.e. UL AOA, UL-RTOA, UL SRS-RSRP, UL SRS-RSRPP and gNB Rx-Tx time difference), so it is natural to add the information in the TRP Measurement Result IE.

When the gNB reports an UL measurement, it also needs to report the ARP location information associated with the UL measurement. However, reporting ARP location information for every reporting instance creates unnecessary overhead since the location information associated with an ARP is static. It is far more efficient to assign an identifier for the ARP (“ARP ID”) so that the gNB need only report the ARP ID with the UL measurement result. Then, the LMF can determine the location information associated with the ARP ID either by configuration (OAM) or by retrieving from gNBs using the TRP Information Exchange procedure. In the latter case, the existing ‘geo-coordinates’ codepoint can be used and the Geographical Coordinates IE extended to optionally include ARP location information. Regarding the maximum number of ARPs associated with a TRP, it could be as high as the number of antenna elements, so at least 32 should be allowed.

Proposal 1:	Introduce a new ARP ID IE in the TRP Measurement Result IE. The ARP ID is used to uniquely identify an ARP associated with a TRP.

Proposal 2:	The maximum number of ARPs associated with a TRP should be 32.

Proposal 3:	Introduce a new ARP Location Information IE in the Geographical Coordinates IE.

According to the latest NR higher-layers parameter list (see [2] rows 71-72), ARP location information is the relative position of the ARP to TRP defined as either relative geodetic location (encoded as in 9.2.48 of TS 38.455) or relative cartesian location (encoded as in 9.2.50 of TS 38.455).

Proposal 4:	ARP location is the relative position of the ARP to its associated TRP, encoded as a CHOICE between the existing Relative Geodetic Location IE or the existing Relative Cartesian Location IE.

We further observe that the RAN1 agreement includes a new “UL SRS-RSRPP” measurement in the list of UL measurements applicable to ARP location. This was agreed by RAN1 as follows:

Agreement (from RAN1#107-e, see also [2] row 142) - Support the LMF to request UL SRS-RSRPP together with timing measurement as part of UL-TDOA and multi-RTT reporting enhancements

Therefore, UL SRS-RSRPP should be added in NRPPa, but the encoding of the measurement result can be left FFS for now since the definition seems still under discussion in RAN1 (see RAN1 LS to RAN4 in R1-2112744 [3]).

Proposal 5:	Introduce UL SRS-RSRPP as a new codepoint in the TRP Measurement Type IE of the MEASUREMENT REQUEST message.

Proposal 6:	Introduce UL SRS-RSRPP as a new measured results value in the TRP Measurement Result IE. The encoding of UL SRS-RSRPP is FFS (pending RAN1/RAN4).

# 3	Conclusions

In this paper, we evaluated the RAN3 impacts of associating ARP location information with UL measurements. The following is proposed:

Proposal 1:	Introduce a new ARP ID IE in the TRP Measurement Result IE. The ARP ID is used to uniquely identify an ARP associated with a TRP.

Proposal 2:	The maximum number of ARPs associated with a TRP should be 32.

Proposal 3:	Introduce a new ARP Location Information IE in the Geographical Coordinates IE.

Proposal 4:	ARP location is the relative position of the ARP to its associated TRP, encoded as a CHOICE between the existing Relative Geodetic Location IE or the existing Relative Cartesian Location IE.

Proposal 5:	Introduce UL SRS-RSRPP as a new codepoint in the TRP Measurement Type IE of the MEASUREMENT REQUEST message.

Proposal 6:	Introduce UL SRS-RSRPP as a new measured results value in the TRP Measurement Result IE. The encoding of UL SRS-RSRPP is FFS (pending RAN1/RAN4).

A text proposal for the NRPPa BL CR [4] is provided in Annex A, where the changes reflecting the above proposals are highlighted in yellow.

# References

- R3-220089 LS on ARP association with UL measurements for NR positioning, RAN1

- R3-220094 LS on updated Rel-17 LTE and NR higher-layers parameter list, RAN1

- R1-2112744 LS on UL SRS-RSRPP definition, RAN1

- R3-216270 Introduction of NR Positioning enhancements to NRPPa, Ericsson

# Annex A:	TP for TS 38.455 BL CR

Start of Text Proposal for TS 38.455 BL CR

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >> AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.x4 |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms) | The codepoint 60min is not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…, UL-SRS-RSRPP) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Response Time | O |  | 9.2.x6 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

Next Change

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |  |  |
| >CHOICE Measured Results Value | M |  |  |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |  | - |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA | M |  | 9.2.39 |  | - |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |  | - |  |
| >>Zenith Angle of Arrival | M |  | 9.2.x5 |  | YES | reject |
| >>UL SRS-RSRPP | M |  | 9.2.a3 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >ARP ID | O |  | 9.2.a2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |

Next Change

### 9.2.46	Geographical Coordinates

This information element contains the geographical coordinates for the TRP and any associated ARP(s).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE TRP Position Definition Type | M |  |  |  |
| >Direct |  |  |  |  |
| >>CHOICE Accuracy | M |  |  |  |
| >>>normal accuracy |  |  |  |  |
| >>>>TRP Position | M |  | NG-RAN Access Point Position 9.2.10 | The configured estimated geographical position of the antenna of the cell/TRP. |
| >>>high accuracy |  |  |  |  |
| >>>>TRP High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.2.49 | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. |
| >Referenced |  |  |  |  |
| >>Reference Point | M |  | 9.2.51 | The reference point is used to derive the TRP position |
| >>CHOICE Type | M |  |  |  |
| >>>Geodetic |  |  |  |  |
| >>>>TRP Position Relative Geodetic | M |  | Relative Geodetic Location 9.2.48 | The configured estimated relative geodetic coordinate of the antenna of the cell/TRP |
| >>>Cartesian |  |  |  |  |
| >>>>TRP Position Relative Cartesian | M |  | Relative Cartesian Location 9.2.50 | The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP |
| DL-PRS Resource Coordinates | O |  | 9.2.47 | DL-PRS Resource Coordinates relative to the TRP coordinate |
| ARP Location Information | O |  | 9.2.a1 |  |

Next Change

#### 9.2.a1	ARP Location Information

This IE contains the relative position of ARP(s) to the TRP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ARP Location Information |  | 1 |  |  |
| >ARP Location Information Item |  | 1..<maxnoARPs> |  |  |
| >>ARP ID | M |  | 9.2.a2 |  |
| >>CHOICE ARP Location Type | M |  |  |  |
| >>>geodetic |  |  |  |  |
| >>>>ARP Position Relative Geodetic | M |  | Relative Geodetic Location 9.2.48 |  |
| >>>cartesian |  |  |  |  |
| >>>>ARP Position Relative Cartesian | M |  | Relative Cartesian Location 9.2.50 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoARPs | Maximum no. of ARPs associated with a TRP. Value is 32. |

Next Change

### 9.2.a2	ARP ID

This IE is used to uniquely identify an ARP associated with a TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| ARP Identifier | M |  | INTEGER (1..32, …) |  |

Next Change

### 9.2.a3	UL SRS-RSRPP Measurement

This IE contains the uplink SRS-RSRPP measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |

Editor’s Note: The details of this IE are FFS.

Next Change

Next Change

### 9.3.5	Information Element definitions

** SKIPPING UNCHANGED TEXT **

IMPORTS

id-MeasurementQuantities-Item,

id-CGI-NR,

id-SFNInitialisationTime-NR,

id-GeographicalCoordinates,

id-ResultSS-RSRP,

id-ResultSS-RSRQ,

id-ResultCSI-RSRP,

id-ResultCSI-RSRQ,

id-AngleOfArrivalNR,

id-ResultNR,

id-ResultEUTRA,

maxCellinRANnode,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-OtherRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxWLANchannels,

maxnoFreqHoppingBandsMinusOne,

id-TDD-Config-EUTRA-Item,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs,

maxnoPosMeas,

maxnoTRPs,

maxnoTRPInfoTypes,

maxNoOfMeasTRPs,

maxNoPath,

maxnoofAngleInfo,

maxnolcs-gcs-translation,

maxnoBcastCell,

maxnoSRSTriggerStates,

maxnoSpatialRelations,

maxNRMeas,

maxEUTRAMeas,

maxIndexesReport,

maxCellReportNR,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxPRS-ResourceSets,

maxPRS-ResourcesPerSet,

maxNoSSBs,

maxnoofPRSresourceSet,

maxnoofPRSresource,

maxnoARPs,

id-Cell-ID,

id-TRPInformationTypeItem,

id-SrsFrequency,

id-OnDemandTRPPRS,

id-AoA-SearchWindow,

id-ZoA,

id-ARPLocationInfo,

id-ARP-ID,

id-UL-SRS-RSRPP

** SKIPPING UNCHANGED TEXT **

ARP-ID ::= INTEGER (1..32, ...)

ARPLocationInformation ::= SEQUENCE (SIZE (1..maxnoARPs)) OF ARPLocationInformation-Item

ARPLocationInformation-Item ::= SEQUENCE {

aRP-ID				ARP-ID,

aRPLocationType		ARPLocationType,

iE-Extensions		ProtocolExtensionContainer { { ARPLocationInformation-ExtIEs} } OPTIONAL,

...

}

ARPLocationInformation-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ARPLocationType ::= CHOICE {

aRPPositionRelativeGeodetic			RelativeGeodeticLocation,

aRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension					ProtocolIE-Single-Container { { ARPLocationType-ExtIEs } }

}

ARPLocationType-ExtIEs NRPPA-PROTOCOL-IES ::= {

...

}

Assistance-Information ::= SEQUENCE {

systemInformation				SystemInformation,

iE-Extensions					ProtocolExtensionContainer { { Assistance-Information-ExtIEs} } OPTIONAL,

...

}

Assistance-Information-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

** SKIPPING UNCHANGED TEXT **

GeographicalCoordinates ::= SEQUENCE {

tRPPositionDefinitionType	TRPPositionDefinitionType,

dLPRSResourceCoordinates	DLPRSResourceCoordinates	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { GeographicalCoordinates-ExtIEs } } OPTIONAL,

...

}

GeographicalCoordinates-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ARPLocationInfo		CRITICALITY reject TYPE ARPLocationInfo 	PRESENCE optional},

...

}

** SKIPPING UNCHANGED TEXT **

TRPMeasurementQuantities-Item ::= ENUMERATED {

gNB-RxTxTimeDiff,

uL-SRS-RSRP,

uL-AoA,

uL-RTOA,

...,

uL-SRS-RSRPP

}

** SKIPPING UNCHANGED TEXT **

TrpMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoPosMeas)) OF TrpMeasurementResultItem

TrpMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				TrpMeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TrpMeasurementQuality						OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{TrpMeasurementResultItem-ExtIEs}}		OPTIONAL,

...

}

TrpMeasurementResultItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ARP-ID		CRITICALITY ignore TYPE ARP-ID 	PRESENCE optional},

...

}

TrpMeasuredResultsValue ::= CHOICE {

uL-AngleOfArrival	UL-AoA,

uL-SRS-RSRP			UL-SRS-RSRP,

uL-RTOA				UL-RTOAMeasurement,

gNB-RxTxTimeDiff	GNB-RxTxTimeDiff,

choice-extension							ProtocolIE-Single-Container { { TrpMeasuredResultsValue-ExtIEs } }

}

TrpMeasuredResultsValue-ExtIEs NRPPA-PROTOCOL-IES ::= {

{ ID id-ZoA					CRITICALITY reject TYPE ZoA 			PRESENCE mandatory},

{ ID id-UL-SRS-RSRPP		CRITICALITY reject TYPE UL-SRS-RSRPP 	PRESENCE mandatory},

...

}

** SKIPPING UNCHANGED TEXT **

UL-SRS-RSRPP ::= OCTET STRING -- details of this IE are FFS

Next Change

### 9.3.7	Constant definitions

** SKIPPING UNCHANGED TEXT **

maxnoofPRSresource							INTEGER ::= 64

maxnoARPs									INTEGER ::=	32

** SKIPPING UNCHANGED TEXT **

id-ResponseTime													ProtocolIE-ID ::= x7

id-ARPLocationInfo												ProtocolIE-ID ::= xx

id-ARP-ID														ProtocolIE-ID ::= xx

id-UL-SRS-RSRPP													ProtocolIE-ID ::= xx

End of Text Proposal for TS 38.455 BL CR