3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220445

Online, 17th -26th  January 2022

Title:	Discussion on F1 impacts for supporting CG-based SDT

Source:	Ericsson

Agenda Item:	24.3

Document for:	Discussion, Approval

# 1	Introduction

In last e-meeting, RAN3 touched the topic of F1 impacts for support of CG-based SDT [1]. Some points remain open. In this paper, we give our analysis based on the design of F1 interface and current RAN2 and RAN3 progress.

# 2	Discussion

## 2.1	Configuration of CG-SDT at gNB-CU

As agreed in last e-meeting F1AP BL CR [2], when UE transits from RRC_CONNECTED to RRC_INACTIVE state, a CG-SDT Query Indication IE is provided to the gNB-DU in the UE CONTEXT MODIFICATION REQUEST message to request the gNB-DU to provide the CG-SDT configuration.

Figure 1. Example procedure for SDT when UE stays in RRC_INACTIVE

When the gNB-DU receives the query indication, it should transfer the CG-SDT related resources within the DU to CU RRC Information IE. According to current RAN2’s discussion, CG-SDT-Config info can be added to SuspendConfig IE within RRCRelease message in TS 38.331. From F1 perspective, the SDT-MACPHY-Config can be added to existing DU to CU RRC Information IE, to be transferred from DU to CU.

Proposal 1: Introduce SDT-MACPHY-Config IE to DU to CU RRC Information IE for the gNB-CU to generate the RRC Release message with CG-SDT config.

Once the UE Context Release procedure is triggered by the gNB-CU, the gNB-DU transparently forwards the message directly to the UE containing the RRCRelease message. Then, the next question is whether the gNB-DU should maintain the logical connection with the gNB-CU and keep the SDT RLC Context? If the UE will resume back to the same DU from RRC_INACTIVE, then having the DU keeping the resources/context even after it receives the release command message makes sense. However, if the UE reselects in a cell in another NG-RAN node, then it would have been a waste of DU resources.

Observation 1: If the UE will resume back to the same DU, then having the DU keeping the resources/context even after it receives the release command message makes sense. However, if the UE reselects in a cell in another NG-RAN node, then it would have been a waste of DU resources.

Therefore, as the gNB-DU cannot put restriction on UE’s mobility when releasing it, the gNB-DU should behave as normal during the existing UE Context Release procedure. The only new thing that could be added and that can support an implementation, where e.g., the gNB-DU keeps the resources/context, is a text clarification that the procedure can be triggered also for the SDT case.

Proposal 2: the gNB-DU should behave as normal during the existing UE Context Release procedure. A text clarification can be added that the procedure can be triggered also for the SDT case.

In case the TAT-SDT is expired, the CG-SDT resources can no longer be used, and the UE may fallback to using RA-SDT mechanism. While the CG-SDT configuration is no longer valid in this scenario, the SDT related F1 tunnels can still be re-used in the scenario RA-SDT fallback. The gNB-CU can notify the gNB-DU to use the stored F1-C and F1-U tunnel for the next SDT SRB and DRB transmissions. This can be more appropriate than setting up new tunnels. Hence, it is proposed that the gNB-CU provides the old UE F1AP IDs couple in the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU.

Proposal 3: the gNB-CU provides the previous UE F1 AP IDs to the gNB-DU to re-use the tunnel info during e.g. fallback from CG- to RA-SDT.

A TP to F1 BL CR is proposed in the annex below.

A TP to F1AP Stage 2 BL CR is provided in [3].

# Conclusion

To fulfil the new requirement of CG-SDT, RAN3 proposes the followings.

Proposal 1: Introduce SDT-MACPHY-Config IE to DU to CU RRC Information IE for the gNB-CU to generate the RRC Release message with CG-SDT config.

Observation 1 : If the UE will resume back to the same DU, then having the DU keeping the resources/context

even after it receives the release command message makes sense. However, if the UE reselects in a cell in another NG-RAN node, then it would have been a waste of DU resources.

Proposal 2: the gNB-DU should behave as normal during the existing UE Context Release procedure. A text clarification can be added that the procedure can be triggered also for the SDT case.

Proposal 3: the gNB-CU provides the previous UE F1 AP IDs to the gNB-DU to re-use the tunnel info during e.g. fallback from CG- to RA-SDT.

# References

R3-216023, SDT2_CGbased (LG - moderator)

R3-220033, CG-SDT BLCR to TS38.473, Samsung

R3-220602, TP to TS 38.470 BL CR: Support of reuse the F1-C/F1-U tunnel, Ericsson

# Annex: TP to F1 BL CR

START OF CHANGES

### 8.3.3	UE Context Release (gNB-CU initiated)

#### 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection or candidate cells in conditional handover or conditional PSCell change. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release (gNB-CU initiated) procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message.

If the old gNB-DU UE F1AP ID IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall additionally release the UE context associated with the old gNB-DU UE F1AP ID.

If the UE CONTEXT RELEASE COMMAND message contains the RRC-Container IE, the gNB-DU shall send the RRC container to the UE via the SRB indicated by the SRB ID IE.

If the UE CONTEXT RELEASE COMMAND message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall consider that the gNB-CU is cancelling only the conditional handover or conditional PSCell change associated to the cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the UE CONTEXT RELEASE COMMAND message includes the CG-SDT Config IE, the gNB-DU shall include it in the RRC Release message to UE.

Interactions with UE Context Setup procedure:

The UE Context Release procedure may be performed before the UE Context Setup procedure to release an existing UE-associated logical F1-connection and related resources in the gNB-DU, e.g. when gNB-CU rejects UE access it shall trigger UE Context Release procedure with the cause value of UE rejection.

#### 8.3.3.4	Abnormal Conditions

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT RELEASE COMMAND message were not prepared using the same UE-associated signalling connection, the gNB-DU shall ignore those non-associated candidate cells.

NEXT CHANGE

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

-----------<unchanged part is omitted>---------

If the SCG Indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and it is set to “released”, the gNB-DU shall, if supported, deduce that an SCG is removed.

If the Estimated Arrival Probability IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If the CG-SDT Query Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message and it is set to ‘true’, the gNB-DU, if supported, shall provide the CG-SDT resource configuration to the gNB-CU in UE CONTEXT MODIFICATION RESPONSE message.

Editor’s note: FFS on the details of CG-SDT resource configuration.

Editor’s note: How to provide CG-SDT resource configuration in UE CONTEXT MODIFICATION RESPONSE message is pending to RAN2 discussion.

Editor’s note: Whether CG-SDT Query Indication IE is per DRB basis or not is FFS.

-------------------------------------------Next Change-------------------------------------------

## 9.2	Message Functional Definition and Content

-----------------------------------------------------<unchanged part is omitted>-----------------------------------------------------

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <unchanged part is omitted> |  |  |  |  |  |  |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| CG-SDT Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| gNB-CU UE F1AP ID 2 | O |  | gNB-CU UE F1AP ID 9.3.1.4 | gNB-DU UE F1AP ID of the previous CG-SDT session | YES | reject |
| gNB-DU UE F1AP ID 2 | O |  | gNB-DU UE F1AP ID 9.3.1.5 | gNB-CU UE F1AP ID of the previous CG-SDT session | YES | reject |

NEXT CHANGE

#### 9.2.2.5	UE CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the UE-associated logical F1 connection or candidate cells in conditional handover or conditional PSCell change.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8]. | YES | ignore |
| SRB ID | C- ifRRCContainer |  | 9.3.1.7 | The gNB-DU sends the RRC message on the indicated SRB. | YES | ignore |
| old gNB-DU UE F1AP ID | O |  | 9.3.1.5 | Include it if RRCReestablishmentRequest is not accepted | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| CG-SDT Config |  |  | OCTET STRING | As defined in TS 38.331 [8]. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifRRCContainer | This IE shall be present if the RRC container IE is present. |

NEXT CHANGE

#### 9.3.1.26	DU to CU RRC Information

This IE contains the RRC Information that are sent from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CellGroupConfig | M |  | OCTET STRING | CellGroupConfig, as defined in TS 38.331 [8]. |  |  |
| MeasGapConfig | O |  | OCTET STRING | MeasGapConfig as defined in TS 38.331 [8]. For EN-DC/NGEN-DC operation, includes the gap for FR2, as requested by the gNB-CU via MeasConfig IE. For NG-RAN,NE-DC and MN for NR-NR DC, includes the gap(s) for FR1 and/or FR2, as requested by the gNB-CU via MeasConfig IE and according to the requested gap type (per-UE or per-FR). |  |  |
| Requested P-MaxFR1 | O |  | OCTET STRING | requestedP-MaxFR1, as defined in TS 38.331 [8]. For EN-DC, NGEN-DC and NR-DC operation, this IE should be included. |  |  |
| DRX Long Cycle Start Offset | O |  | INTEGER (0..10239) | Identical to the value of the drx-LongCycleStartOffset IE within the DRX-Config as defined in TS 38.331 [8]. This field is not used in NR-DC. |  |  |
| Selected BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination; if this IE is included, the gNB-CU uses this information to deduce the selected band. | YES | ignore |
| Selected FeatureSetEntryIndex | O |  | OCTET STRING | FeatureSetEntryIndex, as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected FeatureSet. | YES | ignore |
| Ph-InfoSCG | O |  | OCTET STRING | PH-TypeListSCG, as defined in TS 38.331 [8].For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in SN. | Yes | ignore |
| Requested BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Band Combination. | YES | ignore |
| Requested FeatureSetEntryIndex | O |  | OCTET STRING | FeatureSetEntryIndex, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Feature Set. | YES | ignore |
| DRX Config | O |  | OCTET STRING | DRX-Config, as defined in TS 38.331 [8]. This field is only used in NR-DC. | YES | ignore |
| PDCCH BlindDetectionSCG | O |  | OCTET STRING | pdcch-BlindDetectionSCG, as defined in TS 38.331 [8]. This IE is used between the MgNB-DU and the MgNB-CU. | YES | ignore |
| Requested PDCCH BlindDetectionSCG | O |  | OCTET STRING | requestedPDCCH-BlindDetectionSCG, as defined in TS 38.331 [8]. This IE is used between the SgNB-DU and the SgNB-CU. | YES | ignore |
| Ph-InfoMCG | O |  | OCTET STRING | PH-TypeListMCG, as defined in TS 38.331 [8]. For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in MCG. | YES | ignore |
| MeasGapSharingConfig | O |  | OCTET STRING | MeasGapSharingConfig as defined in TS 38.331 [8]. | YES | ignore |
| SL-PHY-MAC-RLC-Config | O |  | OCTET STRING | SL-PHY-MAC-RLC-Config as defined in TS 38.331 [8]. | YES | ignore |
| SL-ConfigDedicatedEUTRA-Info | O |  | OCTET STRING | SL-ConfigDedicatedEUTRA-Info as defined in TS 38.331 [8]. | YES | ignore |
| Requested P-MaxFR2 | O |  | OCTET STRING | RequestedP-MaxFR2, as defined in TS 38.331 [8]. For NR-DC operation, this IE should be included. | YES | ignore |
| SDT-MACPHY-Config | O |  | OCTET STRING | SDT-MACPHY-Config, as defined in TS 38.331 [8]. | YES | ignore |

NEXT CHANGE