3GPP TSG-RAN WG3 Meeting #114-bis-e	R3-220467

E-meeting, 17-26 Jan 2022

Title: 	Discussion of paging cause for MUSIM

Source: 	Huawei

Agenda item:	30.2

Document Type:	Discussion

- Introduction

In the updated WID for Multi-SIM devices [1], the objectives related to RAN3 are provided as follows.

- Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose) [RAN2, RAN3]: - RAT Concurrency: Network A is NR. Network B can either be LTE or NR. - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx - Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR[ RAN2, RAN3]. - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR. - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

This objective 3 is aligned with Key Issue 1 described in 3GPP TR 23.761 [2]. In this contribution, we discuss and provide our views related to the above two objectives, in align with the progress of RAN2 and SA2.

2. Discussion

2.1 Paging cause indication for voice service

For the paging cause indication, the TS 23.501 and TS 23.401 have provided the general procedures as follows.

<Excerpt from TS 23.501> 5.38.3 Paging Cause Indication for Voice Service A Multi-USIM UE and the network may support Paging Cause Indication for Voice Service feature. The network that supports Paging Cause Indication for Voice Service feature shall provide a Voice Service Indication for IMS voice service in the Paging message, only if the UE indicates the Paging Cause Indication for Voice Service feature is supported to the network. The network determines the IMS voice service based on the Paging Policy Indicator as specified in clause 5.4.3.2. Upon reception of the Voice Service Indication in NGAP Paging Message from AMF, the NG-RAN supporting Paging Cause Indication for Voice Service should include the Voice Service Indication in the Uu Paging message to the UE. When the UE context in the AMF indicates Paging Cause Indication for Voice Service feature is supported, in order to require NG RAN to deliver the Voice Service Indication in RAN paging for the UE in RRC-Inactive state, the AMF provides an indication indicating the Paging Cause Indication for Voice Service feature is supported to the NG-RAN. Upon reception of the indication, the NG-RAN that supports the feature stores a Paging Cause Indication for Voice Service indication in its the UE context. For a UE in RRC-Inactive, the NG-RAN should provide the Voice Service Indication in the RAN Paging message only when there is Paging Cause Indication for Voice Service indication in the UE context and detects the downlink data which triggers the RAN Paging message is related to voice service based on the Paging Policy Indicator, in the header of the received downlink data, as specified in clause 5.4.3.2. UE that supports the Paging Cause Indication for Voice Service feature is capable of differentiation between Paging from a network that does not support the Paging Cause Indication for Voice Service feature and Paging without the Voice Service Indication. How the UE distinguishes the Paging from a network that does not support the Paging Cause Indication for Voice Service feature and Paging without the Voice Service Indication is defined in TS 38.331 [28].

In addition, RAN2 has sent a LS to RAN3 [3], where it clearly mentions the introduction of Paging cause in the Paging message:

- Paging cause for service indication will be introduced in NR and LTE - RAN2 agreed to provide the paging cause value “voice” to UEs by extending the Paging message. The presence of the extension, which can be empty list if none of the UEs have paging cause “voice”, indicates that network supports transmission of paging cause for service indication. - For paging reception in RRC_IDLE (i.e. CN paging), UE forwards the paging cause to NAS. It’s up to NAS whether to accept or reject the paging. - For paging reception in RRC_INACTIVE (i.e. RAN paging), the AS-NAS interaction is left up to UE implementation.

It can be observed that a new ‘Paging Cause’ IE is needed in the Paging message in NG interface to indicate the Paging message is originated due to IMS voice. Then, upon reception the Paging Cause in NGAP Paging Message from AMF, the NG-RAN supporting Paging Cause Indication for Voice Service should include the Voice Service Indication in the Uu Paging message to the UE as stated in the above LS from RAN2. Similarly, as MUSIM feature is also applicable to LTE, the same ‘Paging Cause’ IE should also be introduced in the Paging message in S1 interface.

For a UE in RRC Inactive state, for realizing the Paging Cause Indication for Voice Service feature, it is also needed to introduce the same Paging Cause in Xn paging, so all the relevant gNBs in the RNA can further include the voice service indication in the Uu paging messages.

Then in case of CU-DU split structure, it is required to provide the paging information from CU to DU to page a UE, and correspondingly necessary to include the Paging Cause in F1 paging to indicate the paging message is originated due to voice service.

Proposal 1: Introduce a ‘Paging Cause’ IE in NG, S1, Xn and F1 interface to realize the Paging Cause Indication for Voice Service feature.

In addition, SA2 has sent a LS [4] to RAN3, notifying that it is required to include an indication indicating the Paging Cause Indication for Voice Service feature is supported to the NG-RAN in the RRC Inactive Assistance Information. More specifically, if the Multi-USIM UE has indicated support for the Paging Cause Indication for Voice Service feature, and the network supports the feature as well, an indication has to be included in the Core Network Assistance Information for RRC INACTIVE IE over the NG interface. Thus, after receiving the indication, NG-RAN knows the feature is supported by UE, and consequently delivers the paging cause in RAN paging for the UE in RRC-Inactive state, when it detects the downlink data which triggers the RAN Paging message is related to voice service.

Proposal 2: Introduce an indication indicating the Paging Cause Indication for Voice Service feature is supported in Core Network Assistance Information for RRC INACTIVE IE over NG.

2.2 UE notification on network switching

In addition, RAN2 has discussed the configuration of MUSIM scheduled gap, such that during the gap, the UE who camped on NW A, can tune away to USIM B in order to perform the USIM B procedures. In RAN2 #115-e, the following agreements has been reached:

- The network is allowed to configure at most 3 gap patterns (for any MUSIM purpose). - Switching Gaps (of any type) are configured or released by RRC signalling (e.g. RRCReconfiguration message) in Rel-17. FFS if gap can be released autonomously by UE after N repetitions.

Correspondingly, in the running CR [R2-2111602], a MUSIM-GapConfig-r17 IE which specifies the MUSIM gap configuration has been introduced in the RRCReconfiguration message.

RRCReconfiguration-v17xy-IEs ::=         SEQUENCE {

otherConfig-v17xy                       OtherConfig-v17xy                                     OPTIONAL, -- Need M

musim-GapConfig-r17                   MUSIM-GapConfig-r17                                     OPTIONAL, -- Need M

nonCriticalExtension                     SEQUENCE {}                                          OPTIONAL

}

In case of CU/DU split architecture, RAN3 can discuss whether the gap config is determined by the CU or the DU. It is more approximate to locate the gap configuration at the DU, since this is related to the UE dynamic scheduling. The procedure is given as follows.

- Before switching from Network A, a MUSIM device sends the UE assistance information (UAI) to the CU indicating a preference to be kept in RRC_CONNECTED state in Network A while temporarily switching to network B;

- The CU sends the UAI to the DU in the CU to DU RRC Information;

- The DU signals the determined gap configurations (at most 3 gap patterns for MUSIM purpose: two periodic gaps and a single aperiodic gap) to the CU, which can be sent to the UE.

Hence a new MUSIM GapConfig IE should be included in the DU to CU RRC Information IE of F1 interface.

Proposal 3: Introduce a MUSIM GapConfig IE in the DU to CU RRC Information in F1AP to specify the MUSIM gap configuration.

For capturing the above changes, we have provided corresponding CRs in [xx]

- Conclusion

In this contribution, we discussed RAN3 impact for capturing Paging Cause Indication for Voice Service feature, according to the results and LS from RAN2 and SA2, and propose the following:

Proposal 1: Introduce a ‘Paging Cause’ IE in NG, S1, Xn and F1 interface to realize the Paging Cause Indication for Voice Service feature.

Proposal 2: Introduce an indication indicating the Paging Cause Indication for Voice Service feature is supported in Core Network Assistance Information for RRC INACTIVE IE over NG.

Proposal 3: Introduce a MUSIM GapConfig IE in the DU to CU RRC Information in F1AP to specify the MUSIM gap configuration.

- References

- RP-201309 3GPP WID: Support for Multi-SIM devices for LTE/NR

- 3GPP TR 23.761 v1.3.0 (2021-03) Study on system enablers for devices having multiple Universal Subscriber Identify Modules (USIM), draft version

- R2-2111330 LS on RAN2 agreements for paging with service indication

- S2-2109303 LS on Paging Cause Indication for Voice Service Supported in RRC Inactive assistance