3GPP TSG-RAN WG3 #114bis-e	R3-220542

E-meeting, 17 – 26th Jan 2022

Source:	CATT

Title:	Discussion on unmapped QoS flow handling

Agenda Item:	8.1

Document for:	Discussion and decision

- Introduction

In RAN3#113-e meeting, there was controversy on whether NG-RAN node could transmit data of an unmapped flow to other configured/default DRB and a LS was sent to RAN2.In this meeting, the reply LS is received. We make some analysis based on the reply LS and provide proposal accordingly.

- Discussion

In the reply LS from RAN2, it is stated that delivery of DL packets to UE via any configured DRB of the PDU session ‎ before the new QFI to DRB mapping is configured for the QoS flow is allowed. And it is also clarified that ‎from RAN2 perspective, the concept of ‘default DRB’ is limited to uplink.

So, based on the agreement in RAN2, gNB could deliver the DL packets of unmapped QoS flow via any configured DRB of the PDU session before the new QFI to DRB mapping is configured for the QoS flow.

However, in current 38.300, for the unmapped flow scenario, it seems the user plane data could be delivered in the Uu interface only after there is associated DRB configured for this DRB which is not aligned with the response for RAN2. For example, in A.3 as copied below, the user plane data could only be transmitted between gNB and UE after step 5.

To reflect the current conclusion and avoid delaying the data transmission, we propose to clarify that the user plane data transmission could be started as long as gNB receives DL packet for the flow even there is no mapped DRB for this flow.

Proposal: It is proposed to clarify that the user plane data transmission could be started as long as gNB receives DL packet for the flow even there is no mapped DRB for this flow.

- Conclusion

Proposal: It is proposed to clarify that the user plane data transmission could be started as long as gNB receives DL packet for the flow even there is no mapped DRB for this flow.

The draft CR is in [1].

- Reference

[1] R3-220543 Support of unmapped Qos flow CATT