3GPP TSG-RAN3 Meeting #114bis-e	R3-220543

# E-meeting, 17th – 26th Jan 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev | 0 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on unmapped QoS flow handling in RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the LS from RAN2, in case NG-RAN node receives a QoS flow which does not have mapped DRB,it may allocate a new DRB for the QoS flow and could deliver the data via any confiured DRB before the establishment of new DRB. However,in current 38.300,basd on the procedure description,the DL data could only be delievered to UE after there is mapped DRB for the QoS flow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that NG-RAN node could deliver the data via any configued DRB for an unmapped QoS flow before allocate a new DRB for it. Impact assessment towards the previous version of the specification (same release): This CR is a clarification and has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on QoS flow handling. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The delivery of DL data for unmmaped QoS flow maybe delayed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2,A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

A.2	New QoS Flow with RQoS

The following figure shows an example message flow when RQoS is used for a new QoS flow. In this example, the gNB receives from UPF a first downlink packet associated with a QFI for which the QoS parameters are known from the PDU session establishment, but for which there is no association to any DRB yet in AS.

Figure A.2-1: DL data with new QFI sent over existing DRB

0.	PDU session and DRB(s) have been already established.

1.	gNB receives a downlink packet with a new QFI from UPF.

2.	gNB decides to send the new QoS flow over an existing DRB.

NOTE:	If gNB decides to send it over a new DRB, it needs to establish the DRB first.GNB may deliver user plane data via any configured DRB before the establishment of DRB.

3.	gNB sends the DL packet over the selected DRB with the new QFI and RDI set in the SDAP header.

4.	UE identifies the QFI and RDI in the received DL packet and the DRB on which the packet was received. The AS mapping rules are then updated accordingly.

5.	User Plane Data for the new QoS flow can then be exchanged between UE and gNB over the DRB according to the updated mapping rules and between UPF and gNB over the tunnel for the PDU session.

A.3	New QoS Flow with Explicit RRC Signalling

The following figure shows an example message flow when explicit RRC signalling is used for a new QoS flow. In this example, the gNB receives from UPF a first downlink packet associated with a QFI, for which the QoS parameters are already known from the PDU session establishment, but for which there is no association to any DRB yet in AS.

Figure A.3-1: DL data with new QFI sent over existing DRB

0.	PDU session and DRB(s) have been already established.

1.	gNB receives a downlink packet with a new QFI from UPF.

2.	gNB decides to send the new QoS flow over an existing DRB using explicit RRC signalling for updating the AS mapping rules.

3.	gNB sends an RRCReconfiguration message to UE with the new QFI to DRB mapping rule. gNB may also decide to update the DRB configuration if required to meet the QoS requirements for the new QoS Flow.

4.	UE updates the QFI to DRB mapping rules and configuration (if received).

5.	UE sends an RRCReconfigurationComplete message to gNB.

6.	User Plane Data for the new QoS flow can then be exchanged between UE and gNB over the DRB according to the updated mapping rules and between UPF and gNB over the tunnel for the PDU session.To avoid delay of data transmission,the NG-RAN node may deliver the user plane data via any configured DRB before step 3.