3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220585

# Online, 17th-26th Jan 2022

Title:	(draft) Reply LS to RAN2 on beam measurement reports

Response to:	R2-2111476

Release:	Rel-17

# Work Item:	     NR_ENDC_SON_MDT_enh

Source:	Ericsson

To:	3GPP RAN2

Cc:	3GPP SA5

Contact Person:	Angelo Centonza

E-mail Address:	Angelo.Centonza (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	R3-220584 - TPs for the introduction of the on Beam level measurement report configuration in TS38.413 and TS38.423

1. Overall Description:

RAN3 thanks RAN2 for the Reply LS on Beam Measurement reports for the MDT measurements.

RAN2 has discussed this topic and the following agreements were made:

-	RAN2 confirms that includeBeamMeasurements is mandatorily configured by gNB as part of ReportConfigNR with the value TRUE or FALSE.

-	RAN2 confirms that the OAM could indicate operator’s preference on the beam level measurement inclusion (includeBeamMeasurements) in the measurement reports associated to the M1 measurement so that the beam level coverage map can be generated by the OAM.

-	RAN2 confirms that an indicator (includeBeamMeasurements) can be added in the M1 Configuration signaled over the RAN interfaces (e.g., NG, Xn, F1) to indicate to RAN whether the beam measurements are preferred be included for MDT purposes by the UE for M1 measurements or not.

-	RAN2 confirms that for NR M1: Immediate MDT purposes involve reporting triggers and criteria utilized for RRM.

-	RAN2 also wants to inform the RAN3 that the reference signal type (i.e. SSB and CSI-RS) used for reporting the beam level measurement information is the same as that of cell level measurement information.

RAN3 would like to confirm that the above information from RAN2 has been taken into account and that the attached text proposal has been endorsed. The TP adds an indicator (Include Beam Measurements Indication IE) to the M1 Configuration signalled over the NG-RAN interfaces (e.g., NG, Xn) to indicate to the NG-RAN whether the beam measurements should be included for MDT purposes by the UE for M1 measurements or not.

2. Actions:

To RAN2 group

ACTION: 	RAN3 kindly asks RAN2 to take the above information into account.

3. Date of Next RAN3 Meetings:

RAN3#115-e	21st February – 3rd March 2022		Online

RAN3#116-e	16th May – 27th May 2022		Online