3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220625

# Online, 17 – 26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0730 | rev | - | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE XnAP ID in the ERROR INDICATION message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the ERROR INDICATION message, current semantics description for both the Old NG-RAN node UE X2AP ID and New NG-RAN node UE X2AP ID IEs could be misleading. For example, for the Old NG-RAN node UE X2AP ID it says “Allocated for handover at the source NG-RAN node and for dual connectivity at the S-NG-RAN node or at the NG-RAN node from which a DRB is offloaded.”. The original intention of having “at the NG-RAN node from which a DRB is offloaded” should be the same as for LTE, i.e., clarify for SN STATUS TRANSFER message, however such add-on causes confusion for the other messages. For instance, if Error Indication is triggered for the SN Addition procedure, then the Old NG-RAN node UE X2AP ID can be either filled up with SN for DC or MN where the DRB is actually offloaded from, which is incorrect. To solve this issue, there are two ways. Option 1 is to add clarification that the last part is only applicable for the SN Status Transfer procedure. Option 2 is to remove the last part which causes the confusion. Compared the two, Option 1 is a safer way without changing the legacy implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description that “at the NG-RAN node from which a DRB is offloaded” for both Old NG-RAN node UE X2AP ID and New NG-RAN node UE X2AP ID within the ERROR INDICATION message is only applicable for SN Status Transfer procedure. Impact analysis: This CR has no ASN.1 impact. This CR only has an impact on the handling of Error Indication procedure. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability issue exists when Error Indication procedure is triggered in case of DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.423 CR 1661 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------Start of the change--------------------------------------------------

#### 9.1.3.12	ERROR INDICATION

This message is used to indicate that some error has been detected in the NG-RAN node.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Old NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated for handover at the source NG-RAN node and for dual connectivity at the S-NG-RAN node or at the NG-RAN node from which a DRB is offloaded for a SN Status Transfer procedure. | YES | ignore |
| New NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated for handover at the target NG-RAN node and for dual connectivity at the M-NG-RAN node or the NG-RAN node to which a DRB is offloaded for a SN Status Transfer procedure. | YES | ignore |
| Cause | O |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

------------------------------------------------End of the change--------------------------------------------------