3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220695

E-meeting, 17-26 Jan 2022

Title: 	(TP to TS 38.413 BL CR) MBS information in PDU Session Management procedures

Source: 	Huawei, CBN, China Unicom, China Telecom

Agenda item:	22.2.2

Document Type:	Other

# 1. Introduction

- In the RAN3#114e meeting, there were more agreements about how to provide MBS information and Qos flow information from CN to RAN, as following:

Multicast Session Management - About providing mapped QoS flow and associated QoS flow information from CN to RAN, update UE associated NGAP: PDU Session Resource Modify Request Transfer IE and PDU Session Resource Setup Request Transfer IE. - Introduce a non-UE associated NGAP Class1 Multicast Session Update procedure triggered by MB-SMF to support multicast session update in case the change of some of QoS parameters and/or service area. - Introduce non-UE associated NGAP Class1 Multicast Session Activation procedure and Multicast Session Deactivation procedure, FFS on whether a single procedure or separate procedures should be used for activation/update/deactivation. - WA: introduce new MB-SMF containers in TS 38.413, subject to SA2/CT4. Applicable for both MC and BC. - Introduce one or two non-UE associated Class1 NGAP procedure(s), triggered by the gNB to implement Multicast Distribution Setup function, and Multicast Distribution Release function. - For location dependent Multicast MBS service, the MBS Distribution Setup/Release procedure is used to setup/release the NG-U tunnel for an area Session.

- In this paper, we provide the text proposal to introduce those information to TS 38.413 BL CR in Section 2.

# 2. Text Proposal to BL CR of TS 38.413

## *****************First change*******************

#### 9.3.1.51	QoS Flow Identifier

This IE identifies a QoS flow within a PDU Session or a MBS QoS flow within a MBS Session. The definition and use of the QoS Flow Identifier is specified in TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Identifier | M |  | INTEGER (0..63, …) |  |

## *****************Next changes*******************

#### 9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.003 [23].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TMGI | M |  | OCTET STRING (SIZE(6)) | Coded as Temporary Mobile Group Identity (TMGI) defined in TS 23.003 [23]. |
| NID | O |  | 9.3.1.155 |  |

#### 9.3.1.y	MBS Supported

This IE indicates MBS is supported

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Supported | M |  | ENUMERATED (true, …) |  |

#### 9.3.1.z	MBS Service Area information

This IE contains the MBS service area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS service Area Cell Item |  | 0..<maxnoofCellsforMBS> |  |  |
| >>NR CGI | M |  | 9.3.1.7 |  |
| MBS service Area TAI Item |  | 0..<maxnoofTAIforMBS> |  |  |
| >>TAI | M |  | 9.3.3.11 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is FFS. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is FFS. |

#### 9.3.1.a	MBS Area Session ID

This IE indicates the Area Session ID for MBS Session with location dependent context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Area Session ID | M |  | INTEGER (0 .. 255, …) |  |

## *****************Next changes*******************

#### 9.3.4.1	PDU Session Resource Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 | This IE shall be present when at least one Non-GBR QoS flow is being setup and is ignored otherwise. | YES | reject |
| //skip the unchanged part |  |  |  |  |  |  |
| Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.120 |  | YES | ignore |
| Redundant PDU Session Information | O |  | 9.3.1.136 |  | YES | ignore |
| MBS Session Context Setup Request List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Context Setup Request Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS Area Session ID | O |  | 9.3.1.a |  | - | - |
| >>MBS QoS Flow Add Request List |  | 0..1 |  |  | - | - |
| >>>MBS QoS Flow add Request Item | M | 1..<maxnoofMBSQoSflows> |  |  | - | - |
| >>>>MBS QoS Flow Identifier | Ｍ |  | 9.3.1.51 |  | - | - |
| >>>>Associated unicast QoS Flow Identifier | Ｏ |  | 9.3.1.51 |  | - | - |
| >>MBS Service Area Info | O |  | 9.3.1.z |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMBSSessions | Maximum no. of MBS Sessions allowed within one PDU session. Value is FFS. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is FFS. |

## *****************Next changes*******************

#### 9.3.4.2	PDU Session Resource Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL QoS Flow per TNL Information | M |  | QoS Flow per TNL Information 9.3.2.8 | NG-RAN node endpoint of the NG-U transport bearer for delivery of DL PDUs, together with associated QoS flows. | - |  |
| //skip the unchanged part |  |  |  |  |  |  |
| Used RSN Information | O |  | Redundant PDU Session Information 9.3.1.136 |  | YES | ignore |
| Global RAN Node ID of Secondary NG-RAN Node | O |  | Global RAN Node ID 9.3.1.5 |  | YES | ignore |
| MBS Session Context Setup Response List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Context Setup Response Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS Area Session ID | O |  | 9.3.1.a |  | - | - |
| >>MBS QoS Flow Failed to Add List | O |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS Flow Identifiers of the MBS QoS Flows Failed to Add. | - | - |
| MBS Session Context Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Context Failed to Setup Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS Area Session ID | O |  | 9.3.1.a |  | - | - |
| >>Cause | M |  | 9.3.1.2 |  | - | - |
| MBS Supported | O |  | 9.3.1.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS Sessions allowed within one PDU session. Value is FFS. |

## *****************Next changes*******************

#### 9.3.4.3	PDU Session Resource Modify Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 |  | YES | reject |
| //skip the unchanged part |  |  |  |  |  |  |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission of the Redundant QoS Flow(s). | YES | ignore |
| Security Indication | O |  | 9.3.1.27 |  | YES | ignore |
| MBS Session Context Setup or Modify Request List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Context Setup or Modify Request Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS Area Session ID | O |  | 9.3.1.a |  | - | - |
| >>MBS QoS Flow Add Request List |  | 0..1 |  |  | - | - |
| >>>MBS QoS Flow add Request Item | M | 1..<maxnoofMBSQoSflows> |  |  | - | - |
| >>>>MBS QoS Flow Identifier | Ｍ |  | 9.3.1.51 |  | - | - |
| >>>>Associated unicast QoS Flow Identifier | Ｏ |  | 9.3.1.51 |  | - | - |
| >>MBS QoS Flow to Release List | O |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS Flow Identifiers of the MBS QoS Flows to be released. | - | - |
| >>MBS Service Area Info | O |  | 9.3.1.z |  | - | - |
| MBS Session Release List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Release Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS Area Session ID | O |  | 9.3.1.a |  | - | - |
| >>Cause | M |  | 9.3.1.2 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMultiConnectivity | Maximum no. of connectivity allowed for a UE. Value is 4. The current version of the specification supports up to 2 connectivity. |
| maxnoofMBSSessions | Maximum no. of MBS Sessions allowed within one PDU session. Value is FFS. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is FFS. |

## *****************Next changes*******************

#### 9.3.4.4	PDU Session Resource Modify Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| //skip the unchanged part |  |  |  |  |  |  |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of redundant DL PDUs for split PDU session, together with associated QoS flows. | YES | ignore |
| Additional Redundant NG-U UP TNL Information | O |  | UP Transport Layer Information Pair List 9.3.2.11 | NG-RAN node endpoint of the NG-U transport bearer for delivery of redundant DL PDUs corresponding to the modified UPF endpoint(s) received in the UL NG-U UP TNL Modify List IE of the PDU Session Resource Modify Request Transfer IE in case of PDU session split. | YES | ignore |
| MBS Session Context Setup or Modify Response List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Context Setup or Modify Response Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS Area Session ID | O |  | 9.3.1.a |  | - | - |
| >>MBS QoS Flow Failed to Add List | O |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS Flow Identifiers of the MBS QoS Flows Failed to Add. | - | - |
| MBS Context Failed to Setup or Modify List |  | 0..1 |  |  | YES | ignore |
| >MBS Session Item |  | 1..<maxnoofMBSSessions> |  |  | - | - |
| >>MBS Session ID | M |  | 9.3.1.x |  | - | - |
| >>MBS Area Session ID | O |  | 9.3.1.a |  | - | - |
| MBS Supported | O |  | 9.3.1.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |
| maxnoofMBSSessions | Maximum no. of MBS Sessions allowed within one PDU session. Value is FFS. |

## *****************Next changes*******************

//asn.1 part to be added

## *****************End of the changes*******************