3GPP TSG-RAN WG3 Meeting #114-bis-e	R3-220712

# Online, 17th - 26th January 2022

Agenda Item:	31.2.6

Source:	Peraton Labs, CISA ECD, AT&T

Title:	Clarification of DU Early Identification capability for MPS

Document for:	Discussions & Approval

# 1	Introduction

Within the NG-RAN, Multimedia Priority Service (MPS) relies on a number of prioritized handlings at the RRC, NAS and gNB to ensure prioritized service for its users. When an MPS-enabled UE attempts initial access to the RAN it uses Access Identity 1 at the NAS and a dedicated mps-PriorityAccess establishment cause at the RRC. The mps-PriorityAccess establishment cause in the RRCSetupRequest allows the gNB to treat the MPS request with priority during times of congestion.

In the case of the disaggregated gNB comprising a DU and CU unit, the DU unit encounters the RRCSetupRequest message first and forwards it to the CU. At times of congestion, where gNB overload can occur at both the DU and CU, the CU can treat the MPS request with priority but it is currently unclear whether the DU can identify the RRC properties of the MPS call and properly prioritize its processing. MPS objective is to ensure that MPS access attempts and session establishment can maintain a high probability of successful completion.

This discussion paper identifies the need, and proposes two possible ways to address the MPS needs at the disaggregated gNB.

# 2	Discussion

## 2.1	MPS Prioritized Handling

In the NG-RAN, MPS prioritized handling consists of:

- Assignment of Access Identity 1 to the MPS UE originating or terminating the call. It enables access control exemptions during the barring check,

- Prioritized RACH for fast and higher probability of success RACH completion, and

- Establishment of RRC with an MPS dedicated establishment cause value: mps-PriorityAccess.

All three aspects above, among many more defined throughout the RAN and core, help the MPS access attempts to be received and processed at a higher probability of success even in cases of congestion.

The establishment cause value is dedicated to MPS and is processed by the gNB so that the MPS RRC connection can be established even at moments of congestion when normal users may be responded with RRC Reject. At the disaggregated gNB, the processing of the MPS RRCSetupRequest must be received and forwarded by the DU and forwarded to the CU for processing. The procedure, including the F1 tunnelling, is described in Section 8.1 of [1].

Observation 1:	Successful establishment of MPS user’s RRC connection requires the RRCSetupRequest to be received and processed at the DU or forwarded by the DU and processed by the CU.

## 2.2	DU handling of MPS access requests

At the disaggregated gNB, the DU must first receive the RRCSetupRequest (Msg3) from the UE before forwarding it to the DU. Regarding the need to prioritize the MPS access attempt, there are two possible ways by which the DU can act in the existing specification:

- The DU has the capability to analyze the RRCSetupRequest and inspect the establishment cause value. If it detects an MPS corresponding value, the DU will process with priority and then forward the message to CU.

- The DU does not have the capability to analyze the RRCSetupRequest and it just forwards the message to the CU. In that case, when the DU experiences an overload, the MPS is susceptible to not being processed, along with other regular users’ attempts.

Observation 2:	A DU that cannot analyze the RRCSetupRequest message contents cannot provide priority to MPS users’ access attempts.

Observation 3:	In the non-disaggregated gNB architecture, priority treatment can be provided to MPS access attempts at the gNB. However, in the disaggregated gNB, the DU cannot provide that treatment; this is a gap that needs to be addressed.  Therefore, the DU must be able to detect the presence of an MPS access attempt at the RRC level or below.

## 2.2	Proposed DU handling of MPS access requests

The DU must be able to detect the MPS access attempts so as to avoid not-processing them at time of congestion. Two options are possible:

- Option A: the DU is made capable to process RRCSetupRequest messages in order to inspect the establishment cause value and determine if it is set to mps-PriorityAccess. This allows the DU to selectively forward RRCSetupRequest messages to the CU at time of congestion. Currently no RAN3 specification states such a requirement for the DU.

Proposal 1: The DU is made capable to process RRC messages and selectively forward them to the CU at time of congestion.

- Option B: The RRCSetupRequest message is sent with layer 2 early identification indicator (e.g., through Logical Channel ID - LCID) that marks it as MPS specific. This option simplifies the processing tasks of the DU compared to Option A, but introduces a new requirement on the UE.

Proposal 2: The DU is enhanced with early identification capability, e.g., LCID, that can take specific values for MPS access attempts.

We propose that RAN3 formalizes the requirement for prioritized processing at the DU and adopts one of the two proposals.

# 3	Conclusion and Proposals

We made the following observations:

Observation 1:	Successful establishment of MPS user’s RRC connection requires the RRCSetupRequest to be received and processed at the DU or forwarded by the DU and processed by the CU.

Observation 2:	A DU that cannot analyze the RRCSetupRequest message contents cannot provide priority to MPS users’ access attempts.

Observation 3:	In the non-disaggregated gNB architecture, priority treatment can be provided to MPS access attempts at the gNB. However, in the disaggregated gNB, the DU cannot provide that treatment; this is a gap that needs to be addressed.  Therefore, the DU must be able to detect the presence of an MPS access attempt at the RRC level or below.

We propose that RAN3 adopts one of the proposals below:

Proposal 1: The DU is made capable to process RRC messages and selectively forward them to the CU at time of congestion.

Proposal 2: The DU is enhanced with early identification capability, e.g., LCID, that can take specific values for MPS access attempts.

# 4	References

[1]	TS 38.401, “NG-RAN Architecture Description,” v16.8.0