3GPP TSG-RAN WG3 Meeting #114b-e	R3-220744

E-meeting, 17-26 Jan 2022

Title:	[DRAFT] Reply LS on MDT M6 calculation for split bearers in MR-DC

Response to:	R2-2111290

Release:	Rel-17

Work Item:	NR_ENDC_SON_MDT_enh-Core

Source:	Huawei [will be RAN3]

To:	RAN2, SA5

Cc:

Contact person:	Zhang Hongzhuo

Zhanghongzhuo(at)Huawei(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN2 thanks RAN2 for the reply LS in R2-2111290. Based on the answers in the reply LS, RAN3 studies the following use cases for M6 calculation for split bearers in MR-DC:

- Case 1: PDCP duplication is activated within the report interval of M6

- Case 2: PDCP duplication is not activated within the report interval of M6

- Case 3: PDCP transmission mode switches between duplication and non-duplication within the report interval of M6

In order to calculate the M6 in TCE correctly, RAN3 made the following agreements:

- For case 1 and case 2, a PDCP duplication status indication and a ratio of packets transmission between MN and SN are sent to the TCE together with the RAN part delay samples.

For case 3, the M6 calculation for split bearers needs each RAN nodes to produce a delay sample once the PDCP duplication mode switches within a M6 reporting interval which may impact on F1 and E1 protocols. RAN3 assumes that it will also impact on UE on UL D1 reporting.

Therefore, RAN3 would like to postpone the support of case 3 to Rel-18 and ask RAN2 if it is agreeable.

2	Actions

To RAN2

ACTION: RAN3 kindly asks RAN2 to take above RAN3 agreements into account and confirm if postponing case 3 is agreeable or not.

To SA5

ACTION: RAN3 kindly asks SA5 to update the specifications accordingly.

3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#115 	2022-02-21 - 2022-03-03