3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220842

Electronic Meeting, January 17th – 26th, 2022

Agenda item:	24.3

Source: 	Intel Corporation

Title: 	(TP for CG-SDT BL CR for TS 38.473, 38.463, 38.401) Discussion on CG based SDT

Document for:	Discussion and Decision

# Introduction

In the last RAN3-114e meeting, the moderator on CG-SDT did a fairly good job and RAN3 made progresses as follows [1]:

CG-SDT query indication IE is provided to the gNB-DU in UE CONTEXT MODIFICATION REQUEST message to request the gNB-DU to provide the CG-SDT configuration.

CG-SDT resource configuration is provided to the gNB-CU in UE CONTEXT MODIFICATION RESPONSE message.

When receiving RRCResumeRequest message for CG-SDT, the gNB-DU triggers the UL RRC Message Transfer procedure.

WA: Lower layer configuration for SDT DRBs, F1AP association, and F1 tunnel information are kept in gNB-DU when gNB-CU sends the UE to RRC_INACTIVE.

FFS on whether to keep other information (e.g., C-RNTI) or maintain full UE context in gNB-DU.

WA: Once the UE initiates RRC Resume procedure from another cell, the gNB-CU shall indicate to the gNB-DU to release the assigned CG-SDT resource.

WA: New IE is included into the E1AP BEARER CONTEXT MODIFICATION REQUEST message to indicate resume or suspend operation for SDT bearers.

FFS on details of this new IE. FFS on whether to reuse the existing IE.

FFS on whether to use UE Context Setup procedure to exchange CG-SDT query indication and CG-SDT resource configuration.

FFS on whether to use the Initial UL RRC Message Transfer to send the CG-SDT resource configuration.

FFS on whether to exchange the updated tunnel information in UE context release procedure.

FFS on whether there is F1 impact to support the release of CG-SDT resource configuration due to TAT-SDT expiry.

FFS on whether the gNB-DU reconfigures or releases the old CG-SDT resource configuration via the new UE associated logical F1 connection in case of CG-SDT fallback to RA-SDT or non-SDT, and if yes, how to resolve it.

For CG-SDT, whether to include SDT indication and SDT assistant information in F1AP message and whether to include that access happened using CG SDT.

FFS on whether gNB-DU should be aware of which bearers are SDT bearers.

FFS on how to indicate that CG-SDT configuration should be kept in gNB-DU.

In this contribution, we focus on resolving some remaining WAs/FFSes and proposes the corresponding TPs.

# Discussion

## UE context handling in the last serving DU

The first thing to address is whether DU context (and how much) should remain in the last serving DU for CG SDT configured UE. For that, we have a working assumption with one FFS.

WA: Lower layer configuration for SDT DRBs, F1AP association, and F1 tunnel information are kept in gNB-DU when gNB-CU sends the UE to RRC_INACTIVE.

FFS on whether to keep other information (e.g., C-RNTI) or maintain full UE context in gNB-DU.

CG SDT is different to the legacy resume case. In legacy, DU context for a UE is released when CU sends the UE to INACTIVE. This was because NW doesn't know where the UE is going to request resume again. However, the whole point of CG SDT is for a UE to use the configured CG resources to transfer small data to the same cell. CU won't decide CG SDT for a UE from the beginning if the UE is not likely to transfer small data in the same cell. At least some UE context should remain in the last serving DU even after the UE is moved to INACTIVE, so that CG based SDT can work flawlessly.

And we don't think we save much by choosing which information to keep or take out. If DU suffers by keeping CG SDT UE contexts, DU can always initiate UE context release to the CU with the appropriate cause. It is up to CU's implementation on how to deal with this UE context release request. CU can page the UE to legacy resume and release the UE back to INACTIVE without configuring CG SDT. Or, CU can let UE fail CG SDT session (as no UE context in the DU) and let it fallback to RACH SDT or non-SDT. It should be the CU's responsibility to choose a right UE for CG SDT who is likely to transfer small data in the same cell. Whether to keep UE context in the DU or not, or keep how much should not be a show stopper for CG SDT.

Proposal 1: The whole point of CG SDT is for a UE to use the configured CG resources to transfer small data to the same cell (i.e. to the same DU). The UE context should remain in the last serving DU in order for CG SDT to work flawlessly. And CU won't decide CG SDT for a UE from the beginning if the UE is not likely to transfer small data in the same cell.

Proposal 2: If DU suffers by keeping CG SDT UE contexts, DU can always initiate UE context release to the CU with the appropriate cause. This doesn't break down the feature, and it can be left up to CU's implementation on how to deal with such UE context release request from the DU.

Proposal 3: For CG SDT, turn WA into agreement and maintain full UE context at the last serving DU.

From this sense, for the following FFS, among 5 options discussed in [1], we don't have to worry about how to indicate the keep CG SDT configuration in the DU and up to how much. Given that the full UE context is maintained in the last serving DU, we can simply rely on the existing DL RRC MESSAGE TRANSFER to deliver RRCRelease message containing CG SDT configuration.

FFS on how to indicate that CG-SDT configuration should be kept in gNB-DU.

-	Option 1: UE CONTEXT RELEASE COMMAND containing a new indication to keep CG-SDT resource configuration in DU

-	Option 2: UE CONTEXT MODIFICATION REQUEST containing a new indication to keep CG-SDT resource configuration in DU

-	Option 3: DL RRC MESSAGE TRANSFER containing a new indication to keep the CG-SDT resource configuration in DU

-	Option 4: UE CONTEXT RELEASE COMMAND containing a list of unreleased SDT bearers to keep some of SDT bearers

-	Option 5: DL RRC MESSAGE TRANSFER without a new indication to keep the CG-SDT resource configuration in DU

Proposal 4: When releasing the UE back to INACTIVE with CG SDT configuration, use the DL RRC MESSAGE TRANSFER to deliver RRCRelease message (i.e. Option 5).

Moreover, the following WA should also be converted to green.

WA: Once the UE initiates RRC Resume procedure from another cell, the gNB-CU shall indicate to the gNB-DU to release the assigned CG-SDT resource.

Even if we assume there is a timer associated with CG SDT resources, it would be a bad practice if we just let it expire in the last serving DU when the UE pops up in another cell. Removing the CG SDT UE context in the last serving DU doesn't require any enhancement. CU can simply trigger the UE Context Release procedure to delete the old UE context in the last serving DU.

Proposal 5: Turn WA into agreement that, once the UE initiates RRC Resume procedure from another cell, CU releases the old UE context and the assigned CG SDT resources in the last serving DU.

## Resolving other FFS

FFS on whether gNB-DU should be aware of which bearers are SDT bearers.

Based on offline discussions [1], it is generally understood that knowing which DRBs are subject to SDT is important in allocating the corresponding CG resources.

The easiest way would be to make "SDT" as one type of DRB configuration from the beginning (i.e. when the UE context is created in the DU). We don't think it is necessary to restrict to indicate which one is SDT DRB only when CU queries CG SDT from the DU. We first think that SDT capability of a certain DRB is rather static and not dynamically changed. And having "SDT" as one type of DRB configuration, CU can further change SDT capability of a DRB when it queries CG SDT via the UE Context Modification procedure. For the sake of progress and design simplicity, we propose to make "SDT" as one type of DRB configuration and add the optional "SDT" indicator in the DRB To Be Setup/Modified List in the UE Context Setup/Modification Request messages.

Proposal 6: Configure "SDT" as one type of DRB configuration in the DU context. Add an optional "SDT" indicator in the DRB To Be Setup/Modified List in the F1AP UE Context Setup/Modification Request messages to indicate whether a DRB is SDT capable or not.

The same logic can be applied toward CU-UP and over E1 interface as well. For the following WA and FFS,

WA: New IE is included into the E1AP BEARER CONTEXT MODIFICATION REQUEST message to indicate resume or suspend operation for SDT bearers.

FFS on details of this new IE. FFS on whether to reuse the existing IE.

we can make "SDT" as one type of DRB configuration from the beginning and simply add an optional codepoint for the already existing Bearer Context Status Change IE in the Bearer Context Setup/Modification Request messages to indicate CU-UP that the resume request is due to SDT. CU-UP will resume DRBs configured with SDT accordingly and DL data arrival on non-SDT bearers would be notified to CU-CP as expected.

Proposal 7: Configure "SDT" as one type of DRB configuration in the Bearer context. Add an optional "SDT" indicator in the DRB To Be Setup/Modified List in the PDU Session Resource To Setup/Modify List of the E1AP Bearer Context Setup/Modification Request messages to indicate whether a DRB is SDT capable or not.

Proposal 8: Add an optional codepoint for the already existing Bearer Context Status Change IE in the E1AP Bearer Context Setup/Modification Request messages to indicate CU-UP that the resume request is due to SDT.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

FFS on whether to use the Initial UL RRC Message Transfer to send the CG-SDT resource configuration.

FFS on whether the gNB-DU reconfigures or releases the old CG-SDT resource configuration via the new UE associated logical F1 connection in case of CG-SDT fallback to RA-SDT or non-SDT, and if yes, how to resolve it.

These FFSes are related to fallback operation in the same DU.

The first FFS is from [2], proposing that DU should be able to send the updated CG SDT configuration to CU (via Initial UL RRC TRSF) when the UE configured with CG SDT reconnects using RACH-based SDT.

We understand the scenario that the UE may trigger RACH SDT to the same DU if CG SDT configuration is no longer valid (e.g. beam has changed). But in order to work, the assumption is that DU should be able to know that the UE who initiates RACH SDT (instead of CG SDT) had been configured with CG SDT before. However, we think this is impossible at the moment when DU receives msg3 (RRCResumeRequest), given that a new C-RNTI is allocated for RACH SDT.

The same logic applies to the second FFS as well. From DU point of view, DU doesn't have to know that the old F1 and new F1 belong to the same UE. It is the CU who can know the fallback UE is the same UE who had been configured with CG-SDT. CU can do appropriate UE context managements. Assuming that the old logical F1 connection has been maintained between CU and DU for CG-SDT operation for this UE, CU who knows that the fallback UE is the same UE can release the old logical F1 connection and set up new UE context and new logical F1 connection in the same DU. Before releasing the old logical F1 connection, CU can retrieve old CG-SDT configuration for delta CG-SDT configuration toward the new UE context in the DU.

There is no need to make things complicated. In case of CG SDT fallback to RACH SDT or non-SDT toward the same DU, DU doesn't have to know that the old F1 and new F1 belongs to the same UE. CU can know the fallback UE is the same UE who had been configured with CG SDT and do appropriate UE context managements to maintain only one logical F1 connection.

Proposal 9: In case of CG SDT fallback to RACH SDT or non-SDT toward the same DU, DU doesn't have to know that the old F1 and new F1 belongs to the same UE. CU can know the fallback UE is the same UE who had been configured with CG SDT and do appropriate UE context managements to maintain only one logical F1 connection.

Proposal 10: In case of CG SDT fallback to RACH SDT or non-SDT toward the same DU, there is no need to enhance INITIAL UL RRC MESSAGE TRANSFER to send the CG SDT resource configuration.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

FFS on whether to use UE Context Setup procedure to exchange CG-SDT query indication and CG-SDT resource configuration.

Though not essential, we now think that allowing CU to use the UE Context Setup procedure to retrieve new CG SDT resource configuration could be useful when the UE configured with CG SDT fallbacks and uses RACH SDT toward the same DU. While new UE context is being created, CU who knows that the UE had been configured with CG SDT before can decide to retrieve new CG SDT configuration during context setup and shortly send back to INACTIVE, without triggering UE Context Modification procedure to do so. CU can further retrieve the old CG SDT configuration before releasing the old logical F1 connection for delta signalling toward the UE that had been configured with CG SDT.

Proposal 11: In case of CG SDT fallback to RACH SDT or non-SDT toward the same DU, allow CU to use the UE Context Setup procedure to retrieve new CG SDT resource configuration from the DU.

Proposal 12: Allow CU to provide old CG SDT configuration to the DU via the UE Context Setup procedure for delta signalling toward the UE that had been configured with CG SDT.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: The whole point of CG SDT is for a UE to use the configured CG resources to transfer small data to the same cell (i.e. to the same DU). The UE context should remain in the last serving DU in order for CG SDT to work flawlessly. And CU won't decide CG SDT for a UE from the beginning if the UE is not likely to transfer small data in the same cell.

Proposal 2: If DU suffers by keeping CG SDT UE contexts, DU can always initiate UE context release to the CU with the appropriate cause. This doesn't break down the feature, and it can be left up to CU's implementation on how to deal with such UE context release request from the DU.

Proposal 3: For CG SDT, turn WA into agreement and maintain full UE context at the last serving DU.

Proposal 4: When releasing the UE back to INACTIVE with CG SDT configuration, use the DL RRC MESSAGE TRANSFER to deliver RRCRelease message (i.e. Option 5).

Proposal 5: Turn WA into agreement that, once the UE initiates RRC Resume procedure from another cell, CU releases the old UE context and the assigned CG SDT resources in the last serving DU.

Proposal 6: Configure "SDT" as one type of DRB configuration in the DU context. Add an optional "SDT" indicator in the DRB To Be Setup/Modified List in the F1AP UE Context Setup/Modification Request messages to indicate whether a DRB is SDT capable or not.

Proposal 7: Configure "SDT" as one type of DRB configuration in the Bearer context. Add an optional "SDT" indicator in the DRB To Be Setup/Modified List in the PDU Session Resource To Setup/Modify List of the E1AP Bearer Context Setup/Modification Request messages to indicate whether a DRB is SDT capable or not.

Proposal 8: Add an optional codepoint for the already existing Bearer Context Status Change IE in the E1AP Bearer Context Setup/Modification Request messages to indicate CU-UP that the resume request is due to SDT.

Proposal 9: In case of CG SDT fallback to RACH SDT or non-SDT toward the same DU, DU doesn't have to know that the old F1 and new F1 belongs to the same UE. CU can know the fallback UE is the same UE who had been configured with CG SDT and do appropriate UE context managements to maintain only one logical F1 connection.

Proposal 10: In case of CG SDT fallback to RACH SDT or non-SDT toward the same DU, there is no need to enhance INITIAL UL RRC MESSAGE TRANSFER to send the CG SDT resource configuration.

Proposal 11: In case of CG SDT fallback to RACH SDT or non-SDT toward the same DU, allow CU to use the UE Context Setup procedure to retrieve new CG SDT resource configuration from the DU.

Proposal 12: Allow CU to provide old CG SDT configuration to the DU via the UE Context Setup procedure for delta signalling toward the UE that had been configured with CG SDT.

The corresponding TPs for F1AP, E1AP, and Stage-2 (TS 38.401) can be found in Sections 5, 6, and 7, respectively.

# Reference

[1] R3-216023, " SoD of CB: # SDT2_CGbased", LGE

[2] R3-215369, "Support of CG-based SDT", Nokia, Nokia Shanghai Bell

[3] R3-215320, "Support of CG-SDT", Lenovo, Motorola Mobility

[4] R3-220033, "CG-SDT BLCR to TS 38.473", Samsung

[5] R3-216059, "CG-SDT BLCR to TS 38.401", Huawei

# Text Proposal for CG-SDT BLCR to TS 38.473 [4]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB,DRB, BH RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the Estimated Arrival Probability IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If the CG-SDT Query Indication IE is included in the UE CONTEXT SETUP REQUEST message and it is set to ‘true’, the gNB-DU, if supported, shall allocate necessary resources and generates the corresponding lower layer configuration for the UE based on DRBs configured with SDT. Then, the gNB-DU shall provide the CG-SDT resource configuration to the gNB-CU via the SDT-MACPHY-Config IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message. If the SDT-MACPHY-Config IE is also provided in the CU to DU RRC Information IE of the UE CONTEXT SETUP REQUEST message, the gNB-DU shall provide the delta CG-SDT resource configuration to the gNB-CU based on the provided SDT-MACPHY-Config IE.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the Estimated Arrival Probability IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If the CG-SDT Query Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message and it is set to ‘true’, the gNB-DU, if supported, shall allocate necessary resources and generates the corresponding lower layer configuration for the UE based on DRBs configured with SDT. Then, the gNB-DU shall provide the CG-SDT resource configuration to the gNB-CU via the SDT-MACPHY-Config IE of the DU to CU RRC Information IE that is included inUE CONTEXT MODIFICATION RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT Indicator | O |  | ENUMERATED (true, …) | Indicates SDT DRB. | Yes | ignore |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| CG-SDT Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU ® gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT Indicator | O |  | ENUMERATED (true, …) | Indicates SDT DRB. | Yes | ignore |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>Transmission Stop Indicator | O |  | 9.3.1.209 |  | YES | ignore |
| >>SDT Indicator | O |  | ENUMTERATED (true, false, …) | Indicates SDT DRB or not. | Yes | ignore |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| CG-SDT Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.3.1.25	CU to DU RRC Information

This IE contains the RRC Information that are sent from gNB-CU to gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CG-ConfigInfo | O |  | OCTET STRING | CG-ConfigInfo, as defined in TS 38.331 [8]. | - |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| UEAssistanceInformationEUTRA | O |  | OCTET STRING | UEAssistanceInformation, as defined in TS 36.331 [41]. | YES | ignore |
| SDT-MACPHY-Config | O |  | OCTET STRING | SDT-MACPHY-Config as defined in TS 38.331 [8]. | YES | ignore |

#### 9.3.1.26	DU to CU RRC Information

This IE contains the RRC Information that are sent from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CellGroupConfig | M |  | OCTET STRING | CellGroupConfig, as defined in TS 38.331 [8]. |  |  |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Requested P-MaxFR2 | O |  | OCTET STRING | RequestedP-MaxFR2, as defined in TS 38.331 [8]. For NR-DC operation, this IE should be included. | YES | ignore |
| SDT-MACPHY-Config | O |  | OCTET STRING | SDT-MACPHY-Config as defined in TS 38.331 [8]. | YES | ignore |

# Text Proposal for TS 38.463 v16.8.0

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2]. If the Bearer Context Status Change IE is set to "SDT Resume", the gNB-CU-UP shall consider that the UE activity is resumed only for DRBs configured with SDT.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

#### 8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2]. If the Bearer Context Status Change IE is set to "SDT Resume", the gNB-CU-UP shall consider that the UE activity is resumed only for DRBs configured with SDT.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Security Information | M |  | 9.3.1.10 |  | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …, SDT Resume) | Indicates the status of the Bearer Context | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …, SDT Resume) | Indicates the status of the Bearer Context | YES | reject |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.3.3.2	PDU Session Resource To Setup List

This IE contains PDU session resource related information used at Bearer Context Setup Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>>QoS Flows Remapping | O |  | ENUMERATED (update, source configuration, …) | Indicates that the target gNB-CU-CP requests QoS flow remapping during an intra-system lossless handover as specified in TS 38.300 [4]. | YES | reject |
| >>>SDT Indicator | O |  | ENUMERATED (true, …) | Indicates SDT DRB | YES | ignore |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |
| >Redundant PDU Session Information | O |  | 9.3.1.80 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.3.3.10	PDU Session Resource To Setup Modification List

This IE contains PDU session resource to setup related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Setup Modification Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >Security Indication | M |  | 9.3.1.23 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >DRB To Setup List |  | 1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>>DAPS Request Information | O |  | 9.3.1.91 | This IE is not used in this version of the specification. | YES | ignore |
| >>>SDT Indicator | O |  | ENUMERATED (true, …) | Indicates SDT DRB | YES | ignore |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.3.3.11	PDU Session Resource To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >DRB To Setup List |  | 0..1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>>QoS Flows Remapping | O |  | ENUMERATED (update, source configuration, …) | This IE is not used in this version of the specification. | YES | reject |
| >>>SDT Indicator | O |  | ENUMERATED (true, …) | Indicates SDT DRB | YES | ignore |
| >DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |
| >>>Early Data Forwarding Indicator | O |  | ENUMERATED (stop, …) |  | YES | ignore |
| >>>SDT Indicator | O |  | ENUMERATED (true, false, …) | Indicates SDT DRB or not. | YES | ignore |
| //////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////// |  |  |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |
| maxnoofDataForwardingTunneltoE-UTRAN | Maximum no. of Data Forwarding Tunnels to E-UTRAN for a UE. Value is 256. |
| maxnoofQoSflows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

# Text Proposal for CG-SDT BLCR to TS 38.401 [5]

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.6.1	RRC connected to RRC inactive

This section gives the RRC connected to RRC inactive state transition given that gNB consists of gNB-CU and gNB-DU(s), as shown in Figure 8.6.1-1.

Figure 8.6.1-1: RRC connected to RRC inactive state transition procedure

0.	At first, the gNB-CU determines the UE to enter into RRC inactive mode from connected mode.

NOTE: In case that the gNB-CU decided CG based SDT, if required, the gNB-CU retrieves the corresponding lower layer configuration from the gNB-DU via the UE Context Setup or Modification procedure.

1.	The gNB-CU generates RRCRelease message which includes suspend configuration towards UE. The RRC message is encapsulated in UE CONTEXT RELEASE COMMAND message to the gNB-DU.

2.	The gNB-DU forwards RRCRelease message to UE.

3.	The gNB-DU responds with UE CONTEXT RELEASE COMPLETE message.

NOTE: In case of CG based SDT, instead of steps 1 and 3, the DL RRC MESSAGE TRANSFER procedure is used to forward RRCRelease message to the gNB-DU, and the UE context remains in the gNB-DU.