3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220860

Online: Jan 17th – Jan 26th, 2022

Title:	Discussion on Country-Specific Routing

Agenda item:	20.2.5

Source:	CMCC

Document for:	Discussion

Introduction

In RAN3#114e meeting, country specific routing was discussed and the related agreements are as follows:

It’s FFS whether it is needed to introduce a new cause value for UE Context Release Request.

To be continued...

In this contribution, we will solve the above issue for country specific routing.

Discussion

During this meeting, we continue to deal with the use case “UE moves across the country (within the same cell) and not support the new PLMN of the country”.

When UE is in a new country where local PLMN does not support UE, NG-RAN is aware that UE moving across the country based on the UE location information. Additionally, all companies agree that AMF obtaining the user location information provided by RAN can determine whether the UE is allowed to operate at its present location. Then, gNB should make decision to control UE behaviour, i.e., perform an NG handover changing UE to an appropriate AMF or initiate a UE Context Release Request procedure towards serving AMF with a suitable cause value. If UE initiates the UE Context Release Request, we think it is necessary to introduce a new cause value expressed as “UE is not supported in PLMN serving area”. We do not think the cause value “Handover target not allowed” provided by company in last meeting is appropriate. Actually, UE does not get access to target gNB and the reason is that serving PLMN cannot provide serving for UE.

Proposal 1: Introduce a new cause value express as “UE is not supported in PLMN serving area” in UE Context Release Request.

Conclusions

- In this contribution, country specific routing is discussed and we propose the following observations and proposals:

Proposal 1: Introduce a new cause value express as “UE is not supported in PLMN serving area” in UE Context Release Request.

References

- R3-216092 Summary of CB: # 2005_NTN_Country_Routing, CATT.

- R3-213344, Country-specific routing issues, Huawei.