3GPP TSG-RAN WG3 #114bis-e	  					    		R3-221259

17 - 26 Jan 2022

Online

Title:	LS on QMC related progress in RAN3

Release:	Rel-17

Work Item:	NR_QoE-Core

Source:	RAN3

To:	SA5

Cc:

Contact Person:

Name:	Man Zhang

E-mail Address:	zhang.man4@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

There are some more progress in RAN3 that we think SA5 should be informed about:

- QMC framework is decoupled with Trace framework.

- Multiple QMC configurations can be carried in the same message as a list, where each item in the list includes:

- Configuration container

- Area Scope

- service type

- QoE Reference

- MCE IP address

- Slice Scope

- MDT alignment information

- RAN visible QoE metrics indication

- There is no need for prioritization mechanism configured by OAM over NG to guide RAN behavior to release or pause in case of RAN overload situation.

- RAN3 assumes that the OAM will never provide the same QoE Reference to different QoE configurations irrespective of QoE type.

- For alignment of MDT and QoE measurement reporting, OAM may activate/deactivate appropriately.

A UE should continue an ongoing measurement once it leaves the Area, unless the network indicates to the UE to release the QoE configuration.

The following information about an m-based measurement configuration should be explicitly passed to the target during handover:

- The Measurement Configuration Application Layer ID corresponding to the QoE Reference.

- MDT Alignment information.

- MCE IP address.

- WA: Measurement status.

Clarification:

- Multiple QMC configurations can be configured for the same service type with different slice lists, where each QMC configuration is identified with its own QoE Reference.

- The Slice Scope is a list of S-NSSAI.

- The MDT alignment information is an indication from the OAM to the RAN, of the MDT measurement with which the present QoE measurement should be aligned. In case of signalling-based MDT, the indication is the NG-RAN Trace ID, while in case of management-based MDT, the indication is the Trace Reference. The indication is used to inform NG-RAN about whether it should forward the QoE report to MCE along with the MDT related trace details.

- The RAN visible QoE metrics indication is used to inform NG-RAN about the RAN visible QoE metrics which can be configured to UE.

Aside from the above information, there is no consensus in RAN3 about whether pause/resume indication should be sent to MCE or management system.

2. Actions:

To SA5

ACTION: 	RAN3 respectfully asks SA5 to consider the above information and decide whether corresponding specification support is needed.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN3 Meeting #115	    21st Feb - 3rd Feb 2022