3GPP TSG-RAN WG3 Meeting #114b-e	R3-220954

E-meeting, 17-26 Jan 2022

Title: 	(TP for POS BL CR for TS 38.455 & TS 38.473) on Positioning enhancement - light

Source: 	Huawei

Agenda item:	19.2.1

Document Type:	Discussion and Approval

# Introduction

The following document provide the only the F1 TEG proposal aligned with NRPPa proposal.

Note FFS has been provided as requested by some companies

# TP for 38.473

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

ARP	Antenna Reference Point

ARPI	Additional RRM Policy Index

BH	Backhaul

CAG	Closed Access Group

CN	Core Network

CG	Cell Group

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

NID	Network Identifier

NPN	Non-Public Network

NSSAI	Network Slice Selection Assistance Information

posSIB	Positioning SIB

PNI-NPN	Public Network Integrated NPN

RANAC	RAN Area Code

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RRC	Radio Resource Control

RSRP	Reference Signal Received Power

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

TEG	Timing Error Group

TRP	Transmission-Reception Point

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

Z-AoA	Zenith Angles of Arrival

<<<<<<<<<<< NEXT change >>>>>>>>>>>>>

- 8.13.16	Positioning Information Update

- 8.13.16.1	General

The Positioning Information Update procedure is initiated by the gNB-DU to indicate to the gNB-CU that a change has occurred in the SRS configuration, or to report UE Tx TEG information. The procedure uses UE-associated signalling.

- 8.13.16.2	Successful Operation

- Figure 8.13.16.2-1: Positioning Information Update procedure, successful operation

The gNB-DU initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the gNB-CU.

If the SRS Configuration IE is included in the POSITIONING INFORMATION UPDATE message, the gNB-CU shall consider this information as the updated SRS Configuration for the UE. If the SFN Initialisation Time IE is included in the POSITIONING INFORMATION UPDATE message, the gNB-CU shall consider this information as the SFN Initialisation Time associated to the SRS Configuration.

If the UE Tx TEG Association IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall, if supported, consider it as the UE TEG association for the SRS resources that have changed their TEG association since the last update.

Editor’s Note: the proposal above is subject to revision as needed

<<<<<<<<<<< NEXT change >>>>>>>>>>>>>

#### 9.2.12.26	POSITIONING INFORMATION UPDATE[FFS]

This message is sent by the gNB-DU to indicate that a change in the SRS configuration has occurred.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRS configuration | O |  | 9.3.1.192 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 |  | YES | ignore |
| UE Tx TEG Association | O |  | 9.2.X |  | YES | ignore |

<<<<<<<<<<< NEXT change >>>>>>>>>>>>>

#### 9.2.12.10	TRP INFORMATION REQUEST[FFS]

This message is sent by a gNB-CU to request information for TRPs hosted by a gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| TRP list |  | 0..1 |  |  | YES | ignore |
| >TRP list Item |  | 1..<maxnoofTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| TRP Information Type List |  | 1 |  |  | YES | reject |
| >TRP Information Type Item |  | 1 .. <maxnoofTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, trp Tx teg) |  | - |  |

<<<<<<<<<<< NEXT change >>>>>>>>>>>>>

#### 9.2.12.3	POSITIONING MEASUREMENT REQUEST[FFS]

This message is sent by the gNB-CU to request the gNB-DU to configure a positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  |  |  |
| >>TRP ID | M |  | 9.3.1.197 |  |  |  |
| >>Search Window Information | O |  | 9.3.1.204 |  |  |  |
| >>NR CGI | O |  | 9.3.1.12 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8) |  | YES | ignore |
| Positioning Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| Positioning Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, …, 20480ms, 40960ms) |  | YES | reject |
| Positioning Measurement Quantities |  | 1 |  |  | YES | reject |
| > Positioning Measurement Quantities Item |  | 1..<maxnoofPosMeas> |  |  | EACH |  |
| >> Positioning Measurement Type | M |  | ENUMERATED (gNB RX-TX, UL-SRS-RSRP, UL AoA, UL RTOA, …) |  |  | - |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | TS 38.133 [38] |  |  |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPosMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one measurement message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Positioning Report Characteristics IE is set to the value "Periodic". |

<<<<<<<<<<< NEXT change >>>>>>>>>>>>>

#### 9.3.1.176	TRP Information[FFS]

The TRP Information IE contains information for one TRP within a gNB-DU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.3.1.197 |  | - |  |
| TRP Information Type Response List |  | 1 |  |  | - |  |
| >TRP Information Type Response Item |  | 1 .. <maxnoofTRPInfoTypes> |  |  | - |  |
| >>CHOICE TRP Information Type Response Item | M |  |  |  | - |  |
| >>>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | - |  |
| >>>NR CGI |  |  | 9.3.1.12 |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>PRS Configuration | M |  | 9.3.1.177 |  | - |  |
| >>>SSB Information | M |  | 9.3.1.202 |  | - |  |
| >>>SFN Initialisation Time | M |  | Relative Time 1900 9.3.1.183 |  | - |  |
| >>>Spatial Direction Information | M |  | 9.3.1.179 |  | - |  |
| >>>Geographical Coordinates | M |  | 9.3.1.184 |  | - |  |
| >>>TRP Type | M |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp…) | TS 38.305 [42] | YES | reject |
| >>>TRP Tx TEG Association | M |  | 9.2.Y |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

<<<<<<<<<<< NEXT change >>>>>>>>>>>>>

#### 9.3.1.168	UL RTOA Measurement [FFS]

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  |  |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [38] |  |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [38] |  |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [38] |  |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [38] |  |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [38] |  |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [38] |  |  |
| Additional Path List | O |  | 9.3.1.169 |  |  |  |
| SRS Resource set ID | O |  | 9.2.33 |  | YES | ignore |
| SRS Resource ID | O |  | INTEGER (0.. 63) |  | YES | ignore |
| Port Index | O |  | ENUMERATED (1000, 1001, 1002, 1003,…) | TS 38.211 [x] | YES | ignore |
| TRP Rx TEG ID | M |  | INTEGER (1..FFS) |  |  |  |

<<<<<<<<<<< NEXT change >>>>>>>>>>>>>

#### 9.3.1.170	gNB Rx-Tx Time Difference[FFS]

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  | - |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [38] | - |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [38] | - |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [38] | - |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [38] | - |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [38] | - |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [38] | - |  |
| Additional Path List | O |  | 9.3.1.169 |  | - |  |
| CHOICE TEG ID | O |  |  |  |  |  |
| >RxTx TEG |  |  |  |  |  |  |
| >>TRP RxTx TEG ID | M |  | INTEGER (1..FFS) |  |  |  |
| >> TRP Tx TEG ID | O |  | INTEGER (1..FFS) |  |  |  |
| >Rx TEG |  |  |  |  |  |  |
| >>TRP Rx TEG ID | M |  | INTEGER (1..FFS) |  |  |  |
| >>TRP Tx TEG ID | M |  | INTEGER (1..FFS) |  |  |  |
| TRP Tx TEG Association | O |  | 9.2.Y | This IE is included when a TRP Tx TEG ID is present |  |  |

<<<<<<<<<<< NEXT change >>>>>>>>>>>>>

### 9.2.X UE Tx TEG Association [FFS]

This information element contains the UE Tx TEG association.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Tx TEG Association item | 1 .. <maxnoUETEGs> |  |  |  |
| >UE Tx TEG ID | M |  | INTEGER (1..FFS) |  |
| >SRS Resource Set ID | M |  | INTEGER (0..15) |  |
| >SRS Resource ID List |  | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >>SRS Resource ID | M |  | INTEGER(0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoUETEGs | Maximum no of reported UE Tx TEG association. Value is FFS. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

### 9.2.Y TRP Tx TEG Association [FFS]

This information element contains the TRP Tx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP TEG item | 1 .. <maxnoTRPTEGs> |  |  |  |
| >TRP Tx TEG ID | M |  | INTEGER (1..8, …FFS) |  |
| >DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >DL-PRS Resource ID List |  | 1..<maxPRS-ResourcesPerSet> |  |  |
| >>DL-PRS Resource ID | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPTEGs | Maximum no of reported TRP Tx TEG association. Value is FFS. |
| maxPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |

<<<<<<<<<<< NEXT change >>>>>>>>>>>>>