3GPP TSG-RAN WG3 Meeting #114bis-e	R3-221294

Online, 17th – 26th January 2022

Agenda Item:	10.2.6

Source:	Lenovo, Motorola Mobility (moderator)

Title:	Summary of Offline Discussion on SON Enhancements for CHO

Document for:	Approval

Introduction

CB: # SONMDT8_CHOEnh

- Continue the discussion on the open issues from last meeting

- Whether CHO Cell CGI is needed in HANDOVER REPORT message? Whether explicit Handover Report Type is needed in HANDOVER REPORT message for CHO? Whether new initiating condition is needed in FAILURE INDICATION message for CHO? Whether to reuse the existing one UE RLF Report Container in the FAILURE INDICATION message or HANDOVER REPORT message to transfer failure related information for CHO?

- LS to RAN2?

- Capture agreements and provide TPs if agreeable

(Lenovo - moderator)

Summary of offline disc R3-221023

For the Chairman’s Notes

The following proposals can be agreed:

Proposal 1: Network-based solution is needed, e.g. for the case that a RLF occurred in CHO target cell after completing handover.

Proposal 2: Reuse the existing Handover Report Type e.g. “HO too early” or “HO to wrong cell” in HANDOVER REPORT message for CHO.

Proposal 3: Do not introduce a new Handover Report Type e.g. “Inappropriate Configuration of a CHO candidate cell” in HANDOVER REPORT message for CHO.

Proposal 4: Reuse the existing one UE RLF Report Container in the FAILURE INDICATION message or HANDOVER REPORT message to transfer failure related information for CHO.

The following TPs can be agreed:

Proposal 5: The TP for SON BL CR for TS 38.300 in R3-221292 revised from R3-221089 is agreed.

Proposal 6: The TP for SON BL CR for TS 38.300 in R3-221308 revised from R3-220514 is agreed.

The following open issues are to be continued in the next meeting:

- which network-based solution is adopted, e.g. Option a-1/a-2/b/c, or combination of at least one of them;

- whether to introduce a new initiating condition for CHO recovery procedure in FAILURE INDICATION message;

- whether the FAILURE INDICATION message may be initiated without RLF report for CHO, if yes, whether to include an explicit CHO recovery cell ID in FAILURE INDICATION message and whether to include an explicit CHO recovery Cell CGI in HANDOVER REPORT message, in case of without RLF Report.

Phase 2 discussion

Network-based solution

In the phase 1 discussion, it is still controversial which network-based solution is adopted. Based on the votes, maybe we can firstly exclude solution a-3. Option a-1, Option a-2 and Option b have the same votes, and Option c has one more votes than them. It seems difficult to be converged but we have only two meetings left, to make some progress and guarantee functionality frozen in Rel17, moderator would like to ask companies whether to accept Option c considering it is up to implementation and has less spec impact on Xn signaling, if no, which option do your company have strong preference and please give your arguments.

| Options |  | Companies’ votes to each option |
| --- | --- | --- |
| Option a | Option a-1 | 3 |
|  | Option a-2 | 3 |
|  | Option a-3 | 1 |
| Option b |  | 3 |
| Option c |  | 4 |

Q-A: Companies are invited to provide their views on whether to accept Option c considering it is NW implementation and has less spec impact. If no, which option(s) do your company have strong preference and please give your arguments.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo and Motorola Mobility | Yes | CHO related configuration (e.g. candidate cell list and CHO execution condition(s)) is a part of UE context, storing CHO related configuration can be supported by implementation, it is acceptable to us if the solution can’t be down-selected. |
| Ericsson | Yes | This is the most straightforward solution. If needed, some UE or network enhancements could be discussed to shorten the time the source node needs to keep the context. This would still be much “simpler” than options a and b. |
| Samsung | No | When RAN3 sent reply LS to RAN2 in R3-212944, it was said: RAN3 has discussed the UE context handling and retention at the source node after HO, and concluded that it is not mandated that the source node stores the UE context. That’s why RAN2 start to define some RLF Reporting i.e. include Candidate cell list and CHO execution conditions for HOF failure. For the same issue, we should have the same assumption. It is not reasonable to base an assumption (no UE context) if one want something (UE reporting) and base on another assumption (store the UE context) if they want to object something. If c, Candidate cell list and CHO execution conditions for HOF failure are not needed as well. The source node anyway can save the UE context. Our preference is to have b+c. If the source node wants to keep UE context even after successful handover, it doesn’t need to send candidate cell list and CHO execution conditions to the target. If the source node has no plan to store UE context after successful handover, it sends it to the target. For both a-1 and b, new IE in SN Status Transfer message is needed. For b, it is easy to include them explicitly in Xn messages. Otherwise if a-1, it bring complexity for implementation to map Candidate Cell list and CHO Execution Conditions to an index. Because of this complexity, Mobility Information is not used widely in deployment. If only c would be agreed, we propose to send LS to RAN2 to update them, because RAN3 has sent wrong information to RAN2 before. |
| CATT | No if UE based solution could not always be guaranteed | For the solutions on table, we have a preference on solution b which could provide more precise information to the source node for further optimization. |
| Qualcomm | No | Totally agree with Samsung. When we sent the LS to RAN2, some companies said source node can’t store the CHO context and therefore need UE reporting. Now the same companies are saying source node can store via implementation. Now that we have realized UE based solution will not work in the case RLF occurs immediately after a CHO, we propose to support a network-based solution (b+c proposed by Samsung also seems fine) and send a correction LS to RAN2 to not support any UE based solution for CHO candidate cell list and CHO execution conditions. |
| Huawei | Yes, but… | Both c and a-2 are already in place and can be used. In legacy, a-2 can be used for cases where c is not preferred, i.e. if the storage is limited. The same applies for CHO – we can use mobility info if storage is limited. One new problem in CHO is that there could be additional time between HO prep and HO exec. If any further change is needed, we think a-1 is the best. This follows legacy principles, is forward compatible and has minimal impact- |

Moderator summary: (3/6) companies agree, (3/6) companies disagree. No consensus. Moderator would suggest discussing this issue in the next meeting.

CHO recovery cell ID

In the phase 1 discussion, CATT, Qualcomm and Lenovo state that the FAILURE INDICATION may be initiated with or without RLF report like legacy reestablish case, e.g. for the case that a RLF occurred shortly in a CHO candidate cell after successful handover for CHO recovery. If so, we can continue to discuss whether an explicit CHO recovery cell ID is needed in FAILURE INDICATION message.

Q-B: Companies are invited to provide their views on whether to agree that the FAILURE INDICATION may be initiated with or without RLF report for CHO.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo and Motorola Mobility | Yes | As commented in Q4. |
| Ericsson |  | Seems legit, but maybe a WA is safer at this meeting |
| Samsung |  | We would like to understand more on the detail. For Reestablishment without RLF Report, the failure cell PCI, C-RNTI and shortMAI-C are included in Reestablishment Request message. The RAN node receiving RRCReestablishmentRequest can send FAILURE INDICATION message to the failure node (last serving node). For CHO recovery success without RLF Report, the RAN node receiving recovery cannot know which cell is the failure cell without RLF Report. So the RAN node cannot send FAILURE INDICATION to the last serving node. The RAN node can send FAILURE INDICATION to the source node. But without failure cell id, the source node cannot detect the failure type. So maybe FAILURE INDICATION initiated without RLF report for CHO is not that useful? |
| CATT |  | Although source node cannot know failure cell id, it is still useful to let source MN node be aware of the failure events and also the CHO recovery cell ID. Considering several CHO candidate cells are configured while RLF failed in one candidate and recover successfully in the other cell, source cell should optimize CHO configuration towards selected CHO recovery cell. On the other hand, if we do not consider this scenario and do not report the failure event from the recovery cell to source cell in case RLF is not available, this event could not be counted which is not desired for MRO. |
| Qualcomm |  | Consider the following scenario: - Source cell A configured with CHO candidates B, C. - CHO to cell B failed but CHO recovery success to C - UE didn’t send RLF report to C Can’t C send FAILURE INDICATION to the last serving node A (because that is the node from which UE context was retrieved post CHO)? But agree C can’t send the failed cell B. But perhaps A can still use this FAILURE INDICATION for some optimization. Anyway, this might need more discussion. |
| Huawei | No | We need the RLF report. The reason why we cover the case to have no RLF report in the past is to support legacy UEs. But there is no such agreement for MRO for mob enhancements. In addition, different from reestablishment case, the reception node of the successful CHO recovery cannot know whether this is a normal CHO or a CHO recovery. Besides, due to the delay reporting of the RLF-report, the reception node cannot know the real failure node without RLF-report. |

Moderator summary: Companies have different views on this issue. No consensus, more consideration is needed. Moderator would suggest discussing this issue in the next meeting.

Q-C: Companies are invited to provide their views on whether to include an explicit CHO recovery cell ID in FAILURE INDICATION message in case of without RLF Report, if the answer to Q-B is yes.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo and Motorola Mobility | Yes | As commented in Q4. |
| CATT | Yes |  |

Moderator summary: Depends on Q-B. More consideration is needed. Moderator would suggest discussing this issue in the next meeting.

CHO Cell CGI

If we agree that the FAILURE INDICATION may be initiated with or without RLF report for CHO, we continue to discuss whether to include an explicit CHO recovery Cell CGI in HANDOVER REPORT message in case of without RLF Report.

Q-D: Companies are invited to provide their views on whether to include an explicit CHO recovery Cell CGI in HANDOVER REPORT message in case of without RLF Report.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo and Motorola Mobility | Yes |  |
| Ericsson |  | Seems legit, but maybe a WA is safer at this meeting |
| Samsung |  | See the answer for Q-B. |
| CATT | Yes | For this case: CHO failure->CHO recovery success->RLF occur->RRC Reestablishment attempt If Q-B is agreed, the above case may initiate HANDOVER REPORT message to source node with an explicit CHO recovery Cell CGI after receiving failure indication which is triggered by RRC Reestablishment attempt. |
| Huawei | No | See answer to Q-B, and also… CHO only applies to intra-NR RAN. Both the source node and target node can always decode the RLF Report container. |

Moderator summary: Depends on Q-B. More consideration is needed. Moderator would suggest discussing this issue in the next meeting.

MRO detection mechanism in Stage 2

Based on majority’s view in the phase 1 discussion, for clarity it is proposed:

Proposal 5’: It is proposed to add “For CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell separately” on the basis of legacy MRO detection mechanism in TS 38.300 15.5.2.2.2.

As provided in [4], to capture Proposal 5, the changes can be as following:

15.5.2.2.2	Connection failure due to intra-system mobility

One of the functions of Mobility Robustness Optimization is to detect connection failures that occur due to Too Early or Too Late Handovers, or Handover to Wrong Cell. These problems are defined as follows:

-	Intra-system Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Intra-system Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Intra-system Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

In the definition above, the "successful handover" refers to the UE state, namely the successful completion of the RA procedure.

In case of CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell in the definition above means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell separately.

Q-E: Companies are invited to provide their views on whether the above changes are fine.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo and Motorola Mobility | Yes | Fine to clarify it in stage 2 since the majority thinks it is clearer. |
| Ericsson |  | As stated in Q8, this seems obvious. But ok to capture if the majority thinks that the clarification is needed |
| Samsung | Yes | The clarification is needed |
| CATT | Yes |  |
| Qualcomm | Yes | OK to clarify |
| Huawei | No | Why is this needed? CHO can be considered as HO in the existing text. We so far do not use the terms that are introduced. And we already have the definition of successful HO. |

Moderator summary: Most companies are ok to have the suggested changes for clarification except HW. To go with the majority, moderator would like to capture the changes in stage 2.

Proposal 5: The TP for SON BLCR for TS 38.300 in R3-221292 revised from R3-221089 is agreed.

In the phase 1 discussion, some companies have doubts on the text for iii). Moderator would like to clarify it more clearly.

Now we can see the existing text for “Intra-system Too Late Handover” in the agreed BLCR to 38.300 [8] as following:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO is configured but the CHO execution is not initiated for the UE prior to the connection failure, or if DAPS HO is configured but an RLF is detected in the source cell with successful DAPS HO.

It seems the existing time requirement in “Intra-system Too Late Handover” [8] is only for normal HO, in Section 4.5, CATT suggests the changes as following:

-	Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if CHO is configured but there is no recent CHO execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt), or if DAPS HO is configured but an RLF is detected in the source cell with successful DAPS HO.

Q-F: Companies are invited to provide their views on whether the above changes are fine.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo and Motorola Mobility | Yes | Fine to clarify it in stage 2 for clarity. |
| Ericsson | Yes | Ok |
| Samsung | General fine | The “recent” may need to be deleted. For normal handover, there is no recent HO. For CHO, there is a time start restriction i.e. after CHO is configured. So recent is not needed. But no strong view. The “UE reported timer” is referring to the timer from CHO execution to the failure, right? |
| CATT | Yes | According to the agreement of RAN2, we shall add a time requirement (timer D) for CHO too late detection. |
| Qualcomm | Yes | OK with SS’s comments to remove “recent” and to explicitly clarify “UE reported timer”. I think the timer is from CHO execution till failure. |
| Huawei | 1) see comment 2)Yes | 1) why is the change needed – what is the problem |

Moderator summary:

For the first change, all companies except HW are ok to have it (meanwhile SS/QC propose to remove “recent”); for the second change, all companies are ok to have it.

Proposal 6: The TP for SON BLCR for TS 38.300 in R3-221308 revised from R3-220514 is agreed.

Phase1 discussion

CHO execution condition(s) and candidate cell list

To enable the source node knowing the candidate cell list and CHO execution condition(s), UE-based solution and network-based solution are on table, it is FFS to clarify whether RAN2 agreed RLF-report for CHO is sufficient for MRO purpose before discussing network-based solution.

In [1-3], it is observed that UE RLF-report is not sufficient for MRO purpose especially for the case that a RLF occurred in CHO target cell, considering that the UE forgets/deletes the CHO configuration after completing handover to the CHO target cell. In [4], it is observed that the network CANNOT get all the candidate cells in some scenarios.

However, [5] states that RAN2 agree UE-based solution thus network-based solution is not necessary.

Q1: Companies are invited to provide their views on whether to agree that UE RLF-report is not always sufficient for the source node to know candidate cell list and CHO execution condition(s), e.g. especially for the case that a RLF occurred in CHO target cell after completing handover.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | UE RLF report is not sufficient for MRO purpose due to two reasons: - For RLF case, the UE forgets the CHO configuration after completing the HO to the target cell and if the UE declares RLF in the target cell, then the UE does not include the CHO configuration specific information from the source cell - RAN2 agreement is as follow: In RAN2#113bis-e, RAN2 made the following agreement on the list of candidate cells IDs: 2 Include in the RLF report for CHO the following information: a. Indication of whether a measured neighbour cell included in the existing measResultNeighCells was a CHO candidate cell or not. b. List of candidate cells IDs. Inclusion of a) and b) are subject to the RAN3 reply to the RAN2 LS R2-2102149 And it was confirmed in RAN2#115 Agreements in 113bis are confirmed as: 1 Include in the RLF-report for CHO the following: a. Configured CHO execution condition(s) (A3 and/or A5 event configuration, TTT values) c. Latest radio measurement results of the candidate target cells Try to reuse existing mechanism as much as possible. Agreement a. can be revisited if RAN3 has further progress on it. It could be observed that b. List of candidate cells IDs was not confirmed in RAN2#115. |
| Nokia | Yes | Well, to our understanding, even if the Rel.17 RLF Report is sufficient, then there are still Rel.16 UEs that can be configured with CHO. Also, not sure, but is the report mandatory for Rel.17 UEs? |
| CATT | No | We think UE RLF report is already sufficient to let the source NG-RAN node know the candidate cell list and handover condition. |
| Huawei | Yes | According to RAN2 spec, the UE will clear the CHO configuration upon successful CHO. It is possible that the UE cannot provide the CHO configuration in the RLF-report. |
| Qualcomm | Yes | RAN2 is discussing how to include the candidate CHO cell list during HOF and RLF. And as Samsung pointed out, UE forgets the CHO configuration in the RLF case (i.e., RLF post a successful CHO). Moreover, even if we go for a UE based solution for the candidate cell list, expecting the UE to include the entire CHO execution conditions in RLF report is too much of an overhead. We see the following 3 options further: Option 1: Network based solution for both CHO candidate cell list and CHO execution conditions Option 2: UE based solution for CHO candidate cell list and Network based solution for CHO execution conditions Option 3: UE based solution for both CHO candidate cell list and CHO execution conditions Option 4: No need of CHO execution conditions We prefer Option 1 but are OK to with Option 2 or Option 4 as well. Option 3 is not acceptable to us. |
| Lenovo and Motorola Mobility | Yes | It is common understanding in last RAN2 meeting that the UE forgets the CHO configuration after completing the handover and if the UE declares RLF in the target cell, then the UE does not include the CHO configuration specific information in the RLF report. So, at least for the case that a RLF occurs shortly after a successful CHO execution, the UE does not include the Candidate Cell List and CHO execution condition(s) in the RLF report. |
| ZTE | Yes | It is correct that UE will forgets the CHO configuration in the RLF case. |
| Ericsson | Agree | But so far limited to the case described in the question (RLF occurred in CHO target cell after completing handover). For normal cases, this can still be added by RAN2 |

Moderator summary: (7/8) companies agree UE RLF report is not sufficient for the source node to know candidate cell list and CHO execution condition(s), (1/8) company thinks UE RLF report is sufficient. Based on the majority’s view that UE RLF report is not sufficient for the source node to know candidate cell list and CHO execution condition(s), e.g. for the case that a RLF occurred in CHO target cell after completing handover, moderator would like to propose that network-based solution is needed, e.g. for the case that a RLF occurred in CHO target cell after completing handover.

Proposal 1: Network-based solution is needed, e.g. for the case that a RLF occurred in CHO target cell after completing handover.

If it is agreed that UE RLF-report is not sufficient for any case in CHO procedure, network-based solution is needed. Network-based solutions are summarized as below:

Option a: Derive candidate cell list and CHO execution condition(s) based on Mobility Information.

- Option a-1: Source node transmits the Mobility Information to the target node when CHO is completed, i.e. in the SN STATUS TRANSFER message, and the target node sends the Mobility Information back to the source node via HANDOVER REPORT message. [1]

- Option a-2: Source node transmits the Mobility Information to each candidate target node in the HO request message, and the target node sends the Mobility Information back to the source node via HANDOVER REPORT message. [3]

- Option a-3: Including the Mobility Information in the UE RLF-report. RAN3 asks RAN2 to consider feasibility of adding the Mobility Information to the CHO configuration. [2]

Option b: Source node sends candidate cell list and CHO execution condition(s) to the target node after receiving Handover Success message, e.g. in SN Status Transfer message, and then the target transmits the info back to the source node in HANDOVER REPORT message. [4]

Option c: Source node stores the CHO related configuration. [6]

Q2: Companies are invited to provide their views on which network-based solution(s) is acceptable. It is suggested that multiple acceptable solutions can be provided, if any.

| Company | Option a/b/c | Comments |
| --- | --- | --- |
| Samsung | Option b | Option a-2, two drawbacks: 1) the source node has to send the Mobility Information to each candidate. 2) the information may be not up-to-date e.g. the source updated the UE after Handover Request. Option a-3 has to involve the UE and resume air interface resource but the network has such information. Option c, this option mandates the source node to store the CHO related configuration even after successful handover. Previously RAN3 has agreed that the source mode may release the UE context after successful handover when RAN3 sent LS to RAN2. Comparing Option a-1 and Option b, both needs to add new IE in SN Status Transfer message. The main difference between a-1 and b is whether to include Candidate Cell list and CHO Execution Conditions in an implementation dependent container or add explicit information in Xn message. When Mobility Information was introduced in LTE, it was defined as a container because it includes handover trigger and UE group related information which are highly dependent on implementation. Candidate cell list and CHO execution conditions are different. They are standard parameters which are already transmitted over open interface. In this case, it is easy to include them explicitly in Xn messages. Otherwise, it bring complexity for implementation to map Candidate Cell list and CHO Execution Conditions to an index. Furthermore, RAN3 has agreement that the last serving node is responsible for failure reason detection. With option b, the last serving node can use candidate cell list and CHO execution conditions to help the detection. If Mobility Information, the last serving node cannot understand it. |
| Nokia | a3 or c | Please note, if we send the LS at this meeting, RAN2 will be able to enable the transfer (if feasible) still in Rel.17. This has been proposed because RAN-based solutions have serious drawbacks, while RAN2 enhances the RLF report anyway. |
| CATT |  | If network based solution is adopted, network has to store candidate cell list and CHO execution condition until RLF Report is fetched. Since UE can keep RLF Report for 48 hours, network shall store at least 48 hours in order to wait for the related RLF Report. RLF Report is not a real time report which requires network to wait a long time. Moreover, many RLF Reports may be overwritten by subsequent RLF or handover failure. It means network have to wait 48 hours to guarantee RLF Report is invalid. In one word, we cannot require network to store per UE information for 48 hours. We shall LS RAN2 to include candidate cell list and CHO execution condition in RLF Report for successful handover case. |
| Huawei | Combination of a-1, a-2, c | To be clear, we think that the new solution we need to discuss is a-1. Option c and a-2 is legacy functionality. It already exist today. If a source supports c), no further signaling is needed. If the source node does not want to use c, the node can use mobility info. That means he will use a-2 (as today). The new thing with CHO is that in some cases, the CHO candidates will change between HO preparation and the execution, and in this case we can use a-1. We only need to send this if the mobility info changes between preparation and execution. Compared to other solutions, this is a light weight, forward compatible solution aligned with previous principles. |
| Qualcomm | Option a1 or b | Similar views as Samsung. If agreed, we should send an LS to RAN2 informing that a network-based solution is possible for CHO execution conditions and no need to go for a UE based solution for CHO execution conditions. |
| Lenovo and Motorola Mobility | Option a-1 or Option a-2 | Option a-3 has spec impacts to RAN2. Option b wastes more Xn resources/signaling to transfer SN STATUS TRANSFER message compared with Option a-1, and it is not essential for the last serving node to know candidate cell list and CHO execution condition(s) explicitly. Option c is up to implementation. |
| ZTE | Option a-2 /b + Option c | Option c is already supported but implementation depend. If option c is not supported, then option a-2 or b can be used. |
| Ericsson | c | CHO is a special case. Network signaling based solutions seem too complex for this case. Additionally, RAN2 or RAN3 could discuss solutions to lower the impact of option c on the source node |

Moderator summary:

| Options |  | Companies’ votes to each option |
| --- | --- | --- |
| Option a | Option a-1 | 3 |
|  | Option a-2 | 3 |
|  | Option a-3 | 1 |
| Option b |  | 3 |
| Option c |  | 4 |

No consensus. Continue to discuss the network-based solution in the 2nd round, e.g. Option a-1/a-2/b/c, or combination of at least one of them.

FAILURE INDICATION message

initiating condition

[1] proposes to extend the existing initiating condition “RRC Setup” to be “RRC Setup or Reconfiguration”, since it states that even CHO Recovery procedure is part of the Reestablishment procedure, the UE transmits RRCReconfigurationComplete message instead of RRCRestablishmentRequest message, it cannot be considered as the RRC Reestablishment initiated reporting.

[5-6] propose to introduce a new initiating condition for CHO recovery procedure. [7] provides the TP for TS38.423 to introduce a new initiating condition for CHO recovery in FAILURE INDICATION message.

However, [3] proposes that “RRC Re-establishment” can be reused as the initiating condition in FAILURE INDICATION message for CHO.

Q3-1: Companies are invited to provide their views on whether to introduce a new initiating condition for CHO recovery procedure in FAILURE INDICATION message.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Seems not needed. | Either “RRC Reestab Reporting with RLF Report” or “RRC Setup” can be used. |
| CATT | Yes | We prefer introducing a new initiating condition to reusing RRC Re-establishment for following reason: - In original RRC Reestablishment choice in Failure Indication message, C-RNTI, ShortMAC-I and Failure cell PCI IE is mandatory present which cannot be reused by CHO recovery procedure because NG-RAN will receive RRCReconfigurationComplete message from Uu in CHO recovery procedure and there is no C-RNTI, ShortMAC-I and Failure cell PCI IE in RRCReconfigurationComplete message. - In original RRC Reestablishment choice, only Re-establishment cell CGI IE can be reused. The benefit is limited. - After successful CHO recovery, RLF may occur and RRC Reestablishment procedures may be triggered. If reusing RRC Reestablishment initiating condition for CHO recovery, there may be two RRC Reestablishment procedures in one CHO which may lead to ambiguity. |
| Huawei | Yes | The RRC action at CHO recovery is an RRC reconfiguration. It is not an RRC re-establishment. Also, the condition is a bit misleading in Rel16, since RLF reporting triggered after mobility should also be covered. To solve both, we think the easiest way is to just add this. We can even add this to Rel16 |
| Qualcomm | Yes | Agree with CATT and Huawei. |
| Lenovo and Motorola Mobility |  | We slightly prefer to reuse the existing initiating condition for CHO since CHO recovery is a part of RRC re-establishment procedure, and network can distinguish it from legacy RRC re-establishment based on received CHO specific information in the RLF report e.g. an explicit indicator to indicate the handover type is CHO. For example, for the case that CHO recovery to the CHO candidate cell fails, RRC Reestab Reporting with RLF Report i.e. UE RLF Report Container may be transferred in the FAILURE INDICATION message, or RRC Reestab Reporting without RLF Report may be used in the FAILURE INDICATION message, in this way the mandatory IEs e.g. C-RNTI, ShortMAC-I, Failure cell PCI and Re-establishment cell CGI IE would be transferred. But if the majority supports a new initiating condition due to it is clearer, it is acceptable to us. |
| Ericsson | Neutral | Agree with Lenovo. RRC Reestab can be reused, even if not all the IEs are useful to the source node. Network has all the information it needs to distinguish it from legacy RRC re-establishment |

Moderator summary: (3/6) companies agree to introduce a new initiating condition for CHO recovery procedure, (3/6) companies think the existing initiating condition can be reused for CHO. No consensus. Moderator would suggest leaving this issue to the next meeting.

Q3-2: Companies are invited to provide their views on how to introduce a new initiating condition for CHO recovery procedure in FAILURE INDICATION message (e.g. adopt TP in [1], or TP in [7], or other option), if supporting a new initiating condition.

| Company | Comments |
| --- | --- |
| CATT | We prefer the option in 7 |
| Huawei | adopt TP in [1], consider to have this for rel16 |
| Qualcomm | The one in [7] seems more cleaner |

Moderator summary: Depends on Q3-1. Moderator would suggest leaving this issue to the next meeting.

CHO recovery cell ID

[1] proposes duplicated CHO recovery cell ID is not needed in FAILURE INDICATION message since the RLF report should be always included in the FAILURE INDICATION message for CHO recovery case.

[5] proposes CHO recovery cell ID or RLF Report is needed in FAILURE INDICATION message when introducing a new initiating condition for CHO recovery procedure.

[6] proposes CHO recovery cell ID is needed in FAILURE INDICATION message.

Q4: Companies are invited to provide their views on whether an explicit CHO recovery cell ID (i.e., not the one which may be included in UE RLF Report Container) is needed in FAILURE INDICATION message.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung |  | CHO recovery cell ID is included in RLF report. So the last serving node can get it from RLF report. The question is whether there is scenario that the last serving node cannot get it from RLF report? |
| Nokia | No | Agree with Samsung’s technical comment. |
| CATT | Yes | In our opinion, FAILURE INDICATION message shall be sent even without RLF Report because CHO recovery cell is first selected cell after failure which is similar as reestablish cell in legacy handover. For legacy reestablish case, FAILURE INDICATION message shall be initiated without RLF Report. Considering RLF Report may be overwritten by subsequent RLF or handover failure, if we do not trigger FAILURE INDICATION message for CHO recovery while keep waiting for RLF Report, the optimization for CHO handover failure may be omitted. So, an explicit CHO recovery cell ID shall be included in case of without RLF Report. |
| Huawei | no | The RLF-report is reported for the CHO recovery case which belongs to the initiating condition setup or reconfiguration. Thus, no duplicated CHO recovery cell ID is needed in the Failure Indicator message. |
| Qualcomm | Probably yes | As CATT pointed out, it is possible that FAILURE INDICATION can be sent with or without RLF report; so in the same principle, we should perhaps have an explicit CHO recovery cell ID |
| Lenovo and Motorola Mobility | Probably yes | Similar as legacy RRC reestablishment case, FAILURE INDICATION message may be sent without RLF Report (e.g. the initiating condition “RRC Reestab Reporting without RLF Report” is used), if UE RLF Report Container is not included in FAILURE INDICATION message, an explicit CHO recovery cell ID is needed. |
| Ericsson | Not sure | CATT use-case may need further discussion |

Moderator summary: (3/7) companies agree an explicit CHO recovery cell ID is needed in FAILURE INDICATION message, (1/7) company disagrees, and (3/7) company have some doubts on the scenario. As some companies clarify above, FAILURE INDICATION message may be initiated without RLF report, we would discuss whether to include an explicit CHO recovery cell ID in FAILURE INDICATION message in case of without RLF Report in the 2nd round.

HANDOVER REPORT message

Handover Report Type

[1] proposes no explicit Handover Report Type for CHO is needed in HANDOVER REPORT message, which means supporting reusing the existing Handover Report Type for CHO.

[3-4] support that the existing Handover Report Type e.g. “HO too early” or “HO to wrong cell” can be reused in HANDOVER REPORT for CHO. But [4] also propose to add a new Handover Report Type value “Inappropriate Configuration of a CHO candidate cell” in Handover Report message.

[6] proposes that whether to introduce new Handover report type depends on RAN2’s progress since RAN2 will discuss the case of two successive failures.

Q5-1: Companies are invited to provide their views on whether agree to reuse the existing Handover Report Type e.g. “HO too early” or “HO to wrong cell” in HANDOVER REPORT message for CHO.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung |  | Reusing the existing Handover Report Type e.g. “HO too early” or “HO to wrong cell” for CHO is ok. |
| Nokia | Yes |  |
| CATT | Yes | We believe Handover Report message shall be reused as in legacy handover. Failure node is responsible for analyzing MRO failure type and then trigger handover Report message in order to ask other node make optimization. So, the existing Handover Report Type e.g. “HO too early” or “HO to wrong cell” can be reused. |
| Huawei | Yes | CHO only applies to intra-NR RAN. Both the source node and target node can always decode the RLF Report container. When the reception node receives RLF Report, it triggers FAILURE INDICATION message including the RLF Report. If the target node decides the source node as the right one, it sends the received RLF Report to the source node via HANDOVER REPORT message because the source node can decode it and get more useful information. In this way, there is no need to introduce the explicit Handover Report Type for CHO. |
| Qualcomm | Yes | OK to reuse |
| Lenovo and Motorola Mobility | Yes |  |
| ZTE | Yes |  |
| Ericsson | Yes |  |

Moderator summary: All companies agree to reuse the existing Handover Report Type e.g. “HO too early” or “HO to wrong cell” in HANDOVER REPORT message for CHO.

Proposal 2: Reuse the existing Handover Report Type e.g. “HO too early” or “HO to wrong cell” in HANDOVER REPORT message for CHO.

Q5-2: Companies are invited to provide their views on whether to introduce new Handover Report Type e.g. “Inappropriate Configuration of a CHO candidate cell” in HANDOVER REPORT message for CHO.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | We think a new Handover Report Type “Inappropriate Configuration of a CHO candidate cell” is needed. RAN3 has agreement that the last serving node is responsible for failure reason detection and sends the Handover Type to the source node which triggered the last handover. For CHO handover, the last serving node may detect that the failure reason is inapproriate candidate cell list configuration. It’s beneficial to send the information to the source. |
| Nokia | ? | It depends a bit on the failure type definitions in stage-2 – based on those, if the new type is already added there, does the last serving node has all the information to detect this failure? |
| CATT | No | It is not needed to introduce “Inappropriate Configuration of a CHO candidate cell”. For the following scenario, “source node configured Cell-1, Cell-2, Cell-4 to the UE with CHO configuration. CHO execution failure happened. The UE establish/re-establishes to Cell-3 successfully” we believe it is actually “HO to wrong cell” failure type according to TS 38.300 below: “Intra-system Handover to Wrong Cell: there is a recent handover for the UE prior to the connection failure e.g. the UE reported timer is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the cell UE attempts to re-connect/the cell UE attempts CHO recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.” |
| Huawei | No | As said in Q5-1, the source node can perform root cause analysis based on the RLF-report. No additional information is needed from the HO Report message. |
| Qualcomm | No | Agree with CATT and Huawei. |
| Lenovo and Motorola Mobility | No | Inappropriate Configuration of a CHO candidate cell as well as improper CHO execution condition belong to “HO to wrong cell”, it is not possible for the last serving node to understand whether CHO candidate cell list is appropriate when it does not know the configured candidate cell list and CHO execution condition(s) explicitly. And we also agree with HW that source node can perform root cause analysis. |
| Ericsson | Not needed | Existing Handover Report types covers all the possible scenarios. Also, not sure to understand how the last serving node can perform analysis on the candidate list |

Moderator summary: (5/7) companies disagree to introduce a new Handover Report Type “Inappropriate Configuration of a CHO candidate cell”, (1/7) company doubts whether the last serving node has information to detect this failure, and (1/7) company supports the new type.

Proposal 3: Do not introduce a new Handover Report Type e.g. “Inappropriate Configuration of a CHO candidate cell” in HANDOVER REPORT message for CHO.

CHO Cell CGI

[1] proposes CHO Cell CGI is not needed in HANDOVER REPORT message since the target node will forward the received RLF Report to the source node via HANDOVER REPORT message.

[3] and [6] propose that CHO Cell CGI can be included in the HANDVER REPORT message to represent the CHO candidate cell which is selected for CHO recovery.

Q6: Companies are invited to provide their views on whether an explicit CHO recovery Cell CGI (i.e., not the one which may be included in UE RLF Report Container) is needed in HANDOVER REPORT message.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung |  | RAN2 has the following agreement at RAN2#114-e 7 For CHO, it is confirmed that a new CHOCellID is introduced in the RLF-Report to represent the CHO candidate cell selected after the first connection failure and before the reestablishment. To answer this question, we need to firstly clarify whether there is scenario that the source node cannot get CHO recovery Cell CGI from RLF report. We are open to discuss this. |
| CATT | Yes | As discussed in Q4, RLF Report is not mandatory present, so an explicit CHO recovery Cell CGI is needed in case of without RLF Report. |
| Huawei | No | Comments as in Q5-1. |
| Qualcomm | Probably yes | Might be useful in case RLF report is not present as CATT highlighted. |
| Lenovo and Motorola Mobility | Yes | As commented in Q4, FAILURE INDICATION message may be sent without RLF Report. When UE RLF Report Container is not included in FAILURE INDICATION and HANDOVER REPORT message, an explicit CHO recovery Cell CGI is needed. |
| Ericsson | Not sure | CATT use-case may need further discussion |

Moderator summary: (3/6) companies agree an explicit CHO recovery Cell CGI is needed in HANDOVER REPORT message, (1/6) company disagrees, and (2/6) company has some doubts on the scenario. As some companies clarified in Q4, FAILURE INDICATION message may be initiated without RLF report, we would discuss whether to include an explicit CHO recovery Cell CGI in HANDOVER REPORT message in case of without RLF Report in the 2nd round.

UE RLF Report Container

[1] [3] [4] and [6] support to reuse the existing one UE RLF Report Container in the FAILURE INDICATION message or HANDOVER REPORT message to transfer failure related information for CHO.

Q7: Companies are invited to provide their views on whether agree to reuse the existing one UE RLF Report Container in the FAILURE INDICATION message or HANDOVER REPORT message to transfer failure related information for CHO.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| Nokia | Yes |  |
| CATT | Yes |  |
| Huawei | Yes | Any enhancements on the RLF-report can be transparent in RAN3. The existing container can be reused. |
| Qualcomm | Yes |  |
| Lenovo and Motorola Mobility | Yes |  |
| Ericsson | Yes |  |

Moderator summary: All companies agree to reuse the existing one UE RLF Report Container in the FAILURE INDICATION message or HANDOVER REPORT message to transfer failure related information for CHO.

Proposal 4: Reuse the existing one UE RLF Report Container in the FAILURE INDICATION message or HANDOVER REPORT message to transfer failure related information for CHO.

MRO detection mechanism in Stage 2

[4] proposes to add “For CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell seperatly” to TS 38.300 15.5.2.2.2 to make failure type definition more specific and clearer.

Also, [4] thinks that if Option b is applied i.e. the target node gets the candidate cell list and CHO execution condition(s) from the source node explicitly, for Handover to Wrong cell, the target node may further detect that the failure reason is inappropriate candidate cell list configuration or unproper handover trigger. [4] propose to describe inappropriate candidate cell list configuration in the detection mechanism part, i.e. add “If the first re-establishment attempt cell/the cell UE attempts to re-connect is not in the candidate cell list configured to the UE, the root case of the failure is inappropriate candidate cell configuration” for “Intra-system Handover to Wrong Cell” to TS 38.300 15.5.2.2.2.

[5] proposes to select timer D (i.e. timeConnFailure which starts at CHO execution and stops when the HOF/RLF occurs) to detect CHO failure type, and describe CHO failure type detection separately in TS38.300 like following:

The detailed detection mechanisms for too late CHO, too early CHO and CHO to wrong cell are carried out through the following in the NG-RAN node that served the UE before the reported connection failure:

-	Intra-system Too Late CHO: CHO is configured but there is no recent CHO execution for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).

-	Intra-system Too Early CHO: there is a recent CHO execution for the UE prior to the connection failure e.g. the timer D is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the cell UE attempts to re-connect is the cell that served the UE at the last handover initialisation.

-	Intra-system CHO to Wrong Cell: there is a recent CHO execution for the UE prior to the connection failure e.g. the timer D is smaller than the configured threshold (e.g. Tstore_UE_cntxt), and the first re-establishment attempt cell/the cell UE attempts to re-connect/the cell UE attempts CHO recovery is neither the cell that served the UE at the last handover initialisation nor the cell that served the UE where the RLF happened or the cell that the handover was initialized toward.

The "UE reported timer" above indicates the time elapsed since the last handover initialisation until connection failure or the time elapsed since the CHO execution until connection failure.

From moderator point of view, since we already agreed to reuse the legacy MRO detection mechanism with extensions for CHO in stage 2, we would stick on the agreements and spend no more time on discussing to have separate failure type detection for CHO.

On the other hand, compared the agreed BLCR [8] with the separate CHO failure type detection description in [5], besides whether to describe failure type detection for CHO separately, the significant difference is [5] adds a time requirement for detecting too late failure type in CHO, e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt). For a compromise, moderator suggests discussing whether to add this on the basis of legacy MRO detection mechanism in stage 2.

Q8: Companies are invited to provide their views on whether to add following description i/ii/iii on the basis of legacy MRO detection mechanism in TS 38.300 15.5.2.2.2:

- Add “For CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell seperatly”.

- Add “If the first re-establishment attempt cell/the cell UE attempts to re-connect is not in the candidate cell list configured to the UE, the root case of the failure is inappropriate candidate cell configuration” for “Intra-system Handover to Wrong Cell”.

- Add “e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt)” following “or if CHO is configured but the CHO execution is not initiated for the UE prior to the connection failure” which is already captured in agreed BLCR to 38.300 [8].

| Company | Comments for i/ii/iii |
| --- | --- |
| Samsung | i, yes. We support. At least, the description is right. It is common understanding in RAN3. Right? If so, it’s better to make it clear in stage 2. That’s the main difference for normal handover and CHO from definition point of view. ii. yes. We support. For CHO, clearly there are two failure reasons i.e. inappropriate candidate cell list configuration and inappropriate CHO execution conditions. That’s’ why RAN3 spent a lot of effort to discuss how to get those. Differentiation of the two cases should be captured somewhere in stage 2. iii, we don’t fully understand the proposal from the moderator. See [5], we failed to see the new text for too late failure type in CHO. More clarification is needed. |
| Nokia | i: OK ii: This feels like defining a new failure type without defining it… Technically, it may be all right, but then is shall be a separate failure type. Otherwise, it is still HO to wrong cell. iii: Agree with Samsung, it is somewhat incomprehensive… |
| CATT | For i), for CHO, too late/early/to wrong is actually means CHO execution too late/early/to wrong, we do not have strong opinion. For ii), according to BL CR TS38.300, if the first re-establishment attempt cell/the cell UE attempts to re-connect is not source and target cell, we believe it is HO to wrong cell failure type. ii propose to further identify whether it is in the candidate cell list. If it is in candidate cell list, it will be CHO recovery procedure, otherwise it is RRC reestablishment procedure. We believe different procedures can identify whether it is in the candidate cell list. So, the additional type value is not needed. For iii), we are ok to include “e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt)”, but the original “or if CHO is configured but the CHO execution is not initiated for the UE prior to the connection failure” may also need updated because there may be a previous CHO execution as below. CHO1 CFG->CHO1 execution->CHO1 success->CHO2 CFG->RLF The timer D is the time between CHO1 execution and RLF. So ,we propose as below: “or if CHO is configured but there is no recent CHO execution for the UE prior to the connection failure e.g. the UE report timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt).” |
| Huawei | For i and ii, agree the above comments “From moderator point of view, since we already agreed to reuse the legacy MRO detection mechanism with extensions for CHO in stage 2, we would stick on the agreements and spend no more time on discussing to have separate failure type detection for CHO.” For iii, it is not needed. In the legacy detection description, there has been similar wording. - Intra-system Too Late Handover: there is no recent handover for the UE prior to the connection failure e.g. the UE reported timer is absent or larger than the configured threshold (e.g. Tstore_UE_cntxt). |
| Qualcomm | - Yes - Depends on Q5-2 - OK to clarify it for CHO by explicitly stating CHO configuration and CHO execution |
| Lenovo and Motorola Mobility | Answer to Samsung and Nokia: the new text in [5] for too late failure type in CHO is in section 5 Annex –TP for TS38.300. For i, neutral. It obvious that the Too Late Handover, Too Early Handover and Handover to Wrong Cell means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell in CHO procedure. For ii, not needed. Inappropriate Configuration of a CHO candidate cell is a failure type belonging to “HO to wrong cell”, and it is not possible for the last serving node to understand whether CHO candidate cell list is appropriate when it does not know the configured candidate cell list. It is not necessary to emphasize inappropriate candidate cell list configuration in the detection mechanism part additionally. For iii, neutral. It seems the existing time requirement in “Intra-system Too Late Handover” [8] is only for normal HO, for clarity, iii) is fine, but separate failure type detection for CHO in stage 2 is not needed. |
| Ericsson | i. From a CHO point of view, handover means handover execution i.e. similar to legacy HO execution. So the additional text seems obvious. But ok to clarify this in stage-2 if the majority thinks this is useful ii. Agree with Nokia, it looks like a new failure type. Text is correct, but may bring more confusion than clarification iii. Understand the goal, but proposed text is unclear |

Moderator summary:

For i), (4/7) companies agree, (2/7) companies have no strong opinion, (1/7) company supports no separate failure type detection for CHO. For clarity, moderator would propose to add i) on the basis of legacy MRO detection mechanism in TS 38.300 15.5.2.2.2.

For ii), (3/7) companies disagree, (2/7) company thinks it is still HO to wrong cell if without defining a new failure type and would bring more confusion than clarification, (1/7) company agrees, (1/7) company thinks it depends on Q5-2 but a new Handover Report Type e.g. “Inappropriate Configuration of a CHO candidate cell” is not needed based on the majority’s view. To follow the majority. Moderator would propose that ii) is not needed.

For iii), (2/7) companies agree, (1/7) company is open, (1/7) company disagrees, (3/7) companies think the proposed text is unclear and needs more check. Moderator would suggest discussing it in the 2nd round.

Proposal 5’: It is proposed to add “For CHO, the Too Late Handover, Too Early Handover and Handover to Wrong Cell means Too Late CHO Execution, Too Early CHO Execution and CHO Execution to Wrong Cell separately” on the basis of legacy MRO detection mechanism in TS 38.300 15.5.2.2.2.

MRO for CHO to wrong cell case 4

[5] analyzes the MRO issue for the agreed CHO to wrong cell case 4 as below,

Case 4: the UE receives CHO configuration; the CHO execution fails; the UE attempts to CHO recovery to a CHO candidate cell and successes; a RLF occurs shortly after CHO recovery; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the previously selected target cells.

When RLF occurs after CHO failure and CHO recovery success, the former RLF Report recording CHO failure (e.g. RLF Report A) will be discarded e.g. upon successful delivery. [5] states that for MRO analysis, RLF Report RLF Report A shall be kept and merge with the new RLF Report which is related to the RLF after CHO recovery (e.g. RLF Report B). Based on RAN2 agreements, the RLF Report cannot meet the requirement to analyze CHO failure type for CHO to wrong cell case 4, RAN3 needs to send an LS to RAN2. On the other hand, if RLF Report A and RLF Report B are sent to network separately, wrong decision for failure type detection would be made, thus it is required to take both RLF Report A and B into consideration for failure type detection. [5] proposes RAN3 to ask RAN2 to consider how to collect all RLF Report for CHO to wrong cell case 4 in case that network may fetch UE RLF Report after successful CHO recovery.

From moderator point of view, since this issue is relevant to RAN2 and the maximum number of issues to be discussed in this email discussion is limited, we should first solve the above FFSs concerned by the majority, de-prioritize this issue during this meeting as well as tracking RAN2 progress.

References

R3-220321,	(TP for SON BLCR for 38.423) Mobility enhancements, Huawei

R3-220158, Further consideration on using the Mobility Information in case of CHO, Nokia, Nokia Shanghai Bell

R3-220475, SON Enhancements for CHO, Lenovo, Motorola Mobility, ZTE

R3-220780, TP for TS 38.300: SON enhancements for CHO, Samsung

R3-220514, (TP on MRO for mobility Enhancement for 38.300)Discussion on MRO for mobility Enhancement, CATT

R3-220758, Leftover issue on Mobility enhancement, ZTE

R3-220515, (TP on MRO for 38.423) Failure Indication message enhancement for CHO, CATT

R3-211347, BLCR to 38.300: addition of SON features enhancement, CMCC