3GPP TSG-RAN WG3 Meeting #114-bis-e	R3-221321

E-meeting, 17 – 26 January 2022

Agenda item:	10.2.1.4

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP to SON BL CR to 38.413, NR_ENDC_SON_MDT_enh]  Completion of the work on the SCG UHI

Document for:	Endorsement

# 1	Introduction

At the RAN3 #114 meeting, the work on the SCG UHI progressed substantially. It was agreed to define a new SCG UHI format for reporting from the SN to the MN and a nested structure for reporting between the MNs and from an MN to the SN. However, some aspects are still FFS, and we will address those in this document.

At the offline discussion at the meeting [1], it was agreed to update the NGAP protocol.

# 3	Text proposal to the BL CR to TS 38.413

The proposal below is based on the last endorsed BL CR [R3-216254]. ASN.1 will be added once the changes are agreed.

### First Change

#### 9.2.3.X	Last Visited PSCell Information

The Last Visited PSCell Information contains cell information related to the stay in the PSCell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Last Visited PSCell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Last Visited NG-RAN PSCell Information | M |  | 9.2.3.X1 |  |
| >E-UTRAN Cell |  |  |  |  |
| >>Last Visited E-UTRAN PSCell Information | M |  | 9.2.3.X2 |  |

#### 9.2.3.X1	Last Visited NG-RAN PSCell Information

The Last Visited NG-RAN PSCell Information contains information on the PSCell used and the time the UE accessed the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | M |  | NR CGI 9.3.1.7 |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |

#### 9.2.3.X2	Last Visited E-UTRAN PSCell Information

The Last Visited NG-RAN PSCell Information contains information on the PSCell used and the time the UE accessed the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | M |  | E-UTRA CGI 9.3.1.9 |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. |

### End of Changes