3GPP TSG-RAN3 Meeting # 114bis-e	   R3-221085

# 17th - 26th January 2022

Agenda Item:	24.3

Source:	Ericsson (Moderator)

Title: 	Summary of offline discussion of CG-SDT support (CB: # SDT3_CGbased)

Document for:	Discussion

# Introduction

CB: # SDT3_CGbased

- Turn the WAs into agreement?

- Continue the open issues from last meeting

- Discussion on E1AP impact only for CG-SDT

- Capture agreements and open issues, provide TPs if agreeable

(E/// - moderator)

# To the chair’s notes

Moderator’s conclusions of first round:

- P1: Turn the previous WAs into agreements

- P2: The UE Context Setup procedure is not impacted for CG-SDT configuration exchange

- Take the following WAs:

  - WA1: When the gNB-DU receives the query indication, it should transfer the CG-SDT related resources within the DU to CU RRC Information IE. Introduce an SDT-MACPHY-Config IE to DU to CU RRC Information IE for the gNB-CU to generate the RRC Release message with CG-SDT config;

  - WA2: The gNB-CU notifies gNB-DU to keep SDT RLC config and store CG resource for SDT when UE entering RRC inactive;

  - WA3: The gNB-DU shall keep SDT RLC config and store CG resource for SDT when UE entering RRC inactive.

- P3: gNB-DU shall store which bearers are CG-SDT bearers, the C-RNTI

  - WA4: gNB-DU shall store which bearers are CG-SDT bearers and the RLC configuration

  - WA5: the gNB-DU can be aware of the bearer type (SDT Bearer), but it does not need RAN3 changes (awareness can be implicit).

- P4: When the TAT-SDT expires, the gNB-DU initiates the UE Context Release Request procedure (details to be checked, FFS on new cause).

- P5: Proposal to add a new codepoint int the Bearer Context Status Change IE. Addition to be considered in the E1 output TP of “# SDT4_Others”

- P6: when CG-SDT is configured but the UE selects RA-SDT or non-SDT procedure, the gNB-CU provides the old gNB-DU F1AP UE ID to the gNB-DU. The gNB-DU retrieves the old CG-SDT resource configuration and old UE context based on the old gNB-DU F1AP UE ID.

- TP1: Revise R3-220216, to be agreed as TP for BL CR to TS 38.401. WI Rapporteur designated Huawei as BL CR rapporteur

- TP2: Revise R3-220602, to be agreed as TP for BL CR for TS 38.470. WI Rapporteur designated Lenovo as BL CR rapporteur

Discuss in second round:

- Whether the WAs 1-5 can be agreed

- Check TPs and BL CRs.

# Discussion

The Was from last e-meeting are:

WA: Lower layer configuration for SDT DRBs, F1AP association, and F1 tunnel information are kept in gNB-DU when gNB-CU sends the UE to RRC_INACTIVE.

WA: Once the UE initiates RRC Resume procedure from another cell, the gNB-CU shall indicate to the gNB-DU to release the assigned CG-SDT resource.

WA: New IE is included into the E1AP BEARER CONTEXT MODIFICATION REQUEST message to indicate resume or suspend operation for SDT bearers.

The FFS points are:

FFS on whether to keep other information (e.g., C-RNTI) or maintain full UE context in gNB-DU.

FFS on details of this new IE. FFS on whether to reuse the existing IE.

FFS on whether to use UE Context Setup procedure to exchange CG-SDT query indication and CG-SDT resource configuration.

FFS on whether to use the Initial UL RRC Message Transfer to send the CG-SDT resource configuration.

FFS on whether to exchange the updated tunnel information in UE context release procedure.

FFS on whether there is F1 impact to support the release of CG-SDT resource configuration due to TAT-SDT expiry.

FFS on whether the gNB-DU reconfigures or releases the old CG-SDT resource configuration via the new UE associated logical F1 connection in case of CG-SDT fallback to RA-SDT or non-SDT, and if yes, how to resolve it.

For CG-SDT, whether to include SDT indication and SDT assistant information in F1AP message and whether to include that access happened using CG SDT.

FFS on whether gNB-DU should be aware of which bearers are SDT bearers.

FFS on how to indicate that CG-SDT configuration should be kept in gNB-DU.

To be continued...

## WAs from last e-meeting

Can the following Working Assumptions be turned into agreements? If disagreement, please explain why:

- WA: Lower layer configuration for SDT DRBs, F1AP association, and F1 tunnel information are kept in gNB-DU when gNB-CU sends the UE to RRC_INACTIVE.

- WA: Once the UE initiates RRC Resume procedure from another cell, the gNB-CU shall indicate to the gNB-DU to release the assigned CG-SDT resource.

- WA: New IE is included into the E1AP BEARER CONTEXT MODIFICATION REQUEST message to indicate resume or suspend operation for SDT bearers.

Q1: can the above WA be turned into agreements?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Ericsson | Yes |  |
| ZTE | Yes |  |
| Lenovo | Yes |  |
| Huawei | Yes |  |
| China Telecom | Yes |  |
| Google | Yes |  |
| Intel Corporation | Yes except WA3 | It is true that there should be some mechanism to indicate resume for SDT bearers over E1AP, but not sure whether it should be always done by a new IE. At least for suspend due to SDT, the existing codepoint of the existing Bearer Context Status Change can be re-used as for suspend, anyway all bearers are suspended. |
| CATT | Yes |  |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Yes |  |
| Nokia | Yes |  |

Moderator’s conclusions:

- Turn the WAs into agreements

## Leftover issues from last e-meeting

### FFS on whether to use UE Context Setup procedure to exchange CG-SDT query indication and CG-SDT resource configuration

Companies are invited to discuss if UE Context Setup procedure shall or shall not be used to exchange CG-SDT query indication and CG-SDT resource configuration. One company suggests that it can be used in case of CG SDT fallback to RACH SDT or non-SDT toward the same DU [14].

Q2: Shall the UE Context Setup procedure be used to exchange CG-SDT resource configuration. Yes or No, please explain why?

| Company | Y/N | Comment |
| --- | --- | --- |
| Ericsson | No | The UE context has already been set up in case of CG-SDT |
| ZTE | No | The UE context modification is always needed, it can provide the latest configuration result. |
| Lenovo, Motorola Mobility | No | We think using UE context modification is sufficient already. |
| Huawei | Yes but | This is an enhancement, it is good to have, but we can accept without this. When RRC_INACTIVE UE initiates the RA-SDT under the new gNB-DU within the same gNB-CU, the gNB-CU can send CG-SDT query indication to the new gNB-DU and get the CG-SDT resource configuration via the UE Context Setup procedure. It is beneficial to send the CG-SDT resource configuration in the RRCRelease message to UE at the end of the RA-SDT, without initiating another UE Context Modification procedure between the gNB-CP and the new gNB-DU. |
| China Telecom | No | We also think UE Ctxt modification is sufficient |
| Google | No | UE Context Modification procedure is sufficient |
| Intel Corporation | Yes but | As Huawei mentioned, we saw some benefits, but we are fine without it, if majority thinks not essential. |
| CATT | Yes but | Share the view with HW and Intel, no strong view to have it or not. |
| LGE | Yes but | As in Huawei’s comment, we also think that it is nice to have. For now, it is not harmful to use both the UE Context Setup procedure and the UE Context Modification procedure to exchange CG-SDT query indication and CG-SDT resource configuration. |
| Qualcomm | Semi-neutral | Acknowledge Huawei’s comment here, indeed it is nice to have. |
| Samsung | No | CG-SDT is unnecessary to be determined when UE context is set up. |
| Nokia | Yes | As explained by Huawei, this can be useful in case of RA SDT fallback or short transaction. |

Moderator’s conclusions:

- 6 companies consider the UE Context Setup procedure shall not be used to exchange CG-SDT resource configuration

- 4 companies consider it is a “nice to have” enhancement but can accept without it.

- One company is semi-neutral

- The UE Context Setup procedure is not impacted for CG-SDT configuration exchange

### FFS on how to indicate that CG-SDT configuration should be kept in gNB-DU

The following proposals are on the table:

- When the gNB-DU receives the query indication, it should transfer the CG-SDT related resources within the DU to CU RRC Information IE. It is proposed to introduce an SDT-MACPHY-Config IE to DU to CU RRC Information IE for the gNB-CU to generate the RRC Release message with CG-SDT config;

- The gNB-CU notifies gNB-DU to keep SDT RLC config and store CG resource for SDT when UE entering RRC inactive;

- The gNB-DU shall keep SDT RLC config and store CG resource for SDT when UE entering RRC inactive.

Q3: Companies are invited to provide their views on the above three proposals

| Company | Yes to 1,2,3, all No to 1,2,3, all | Comment |
| --- | --- | --- |
| Ericsson | Yes to 1 no to 2, 3 | the gNB-DU should behave as normal during the existing UE Context Release procedure |
| ZTE | Yes for all | gNB shall store SDT RLC configuration (e.g., keep SDT F1 tunnels) |
| Lenovo, Motorola Mobility | No to 1 Yes to 2, 3 | For 1): We would prefer A new RRC container including CG-SDT resource configuration (i.e. the SDT-MAC-PHY-Config IE is) is included outside the DU to CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message. |
| Huawei | Yes for all | gNB-CU should notify gNB-DU to keep CG context otherwise the CG-SDT cannot work. The “Lower layer configuration for SDT DRBs” to be kept in gNB-DU includes both the RLC-BearerConfig IE and the sdt-MAC-PHY-Config IE for the SDT-DRB as defined in TS 38.331 SDT CR. |
| China Telecom | Yes for all | Agree with Huawei |
| Google | Yes to all | But for 2) it may depend on which F1AP message is used and the notification can be implicitly |
| Intel Corporation | Yes to 1 and 3. No to 2. | For 2), for design simplicity and also reasons below, we prefer to let the full UE context remains in the DU: - It should be the CU's responsibility to choose a right UE for CG SDT who is likely to transfer small data in the same cell. - If DU suffers by keeping CG SDT UE contexts, DU can always initiate UE context release to the CU with the appropriate cause. - If non-SDT data arrives, are we going to indicate DU again or re-establish non-SDT bearers again? - If we decide to keep only SDT bearer related contexts in the DU, are we going to do the same thing for CU-UP? For CU-UP, the baseline assumption is that bearer context remains. The same treatment shall be applied to DU not to make the feature too complicated. |
| CATT | Yes for all | It’s possible to keep the full UE context in gNB-DU, but not really needed, keeping the SDT related context should be enough. |
| LGE | Yes for all | At least, in order to support the CG-SDT, the CG-SDT resource configuration needs to be kept in gNB-DU when sending the UE into RRC_INACTIVE. For a FFS on whether to maintain a full UE context in gNB-DU, we think that there is no need to store the information related to the non-CG-SDT bearer. Anyway, the gNB-CU provides the information of all RBs including SDT bearers and non-SDT bearers to the gNB-DU when sending the UE to RRC_CONNECTED state. |
| Qualcomm | Yes for all | At least in principle; details for 1) can be discussed |
| Samsung | Yes for all | For 2&3, the stage-3 details need further discussion |
| Nokia | OK for 1/ Partly for 2/ 3/ | Question for 2/3/: what is the gain to store RLC config and the F1-U tunnel in DU if in the end the UL data is blocked at CU UP waiting for validation of MAC-I ? |

Moderator’s conclusions:

- Companies views slightly differ, but a a majority steams agreeing them all. It is proposed to take the following WAs:

- WA1: When the gNB-DU receives the query indication, it should transfer the CG-SDT related resources within the DU to CU RRC Information IE. It is proposed to introduce an SDT-MACPHY-Config IE to DU to CU RRC Information IE for the gNB-CU to generate the RRC Release message with CG-SDT config;

- WA2: The gNB-CU notifies gNB-DU to keep SDT RLC config and store CG resource for SDT when UE entering RRC inactive;

- WA3: The gNB-DU shall keep SDT RLC config and store CG resource for SDT when UE entering RRC inactive.

### FFS on whether to keep other information (e.g., C-RNTI) or maintain full UE context in gNB-DU.

Some companies propose that the gNB-DU shall store which bearers are CG-SDT bearers, the C-RNTI (descramble) and the RLC configuration.

Q4: Companies are invited to provide their views on these proposals, whether they can be jointly agreed?

| Company | Y/N | Comment |
| --- | --- | --- |
| Ericsson | No | Dependent on Q3 |
| ZTE | Yes | C-RNTI is needed to decode the upcoming small data and signalling. SDT-RLC configuration is also needed to store the F1 association and F1 tunnels |
| Lenovo, Motorola Mobility | Yes |  |
| Huawei | Yes | DU need to store the C-RNTI and the CS-RNTI. According to RAN2 agreement, UE is configured with CG-SDT resource via the RRCRelease message. If the UE initiates the CG-SDT, the UE needs to monitor PDCCH with C-RNTI for scheduling the subsequent data transmission and CS-RNTI for the scheduling retransmission. Therefore the gNB-DU needs to store the C-RNTI and CS-RNTI for the timely scheduling operation. |
| China Telecom | Yes | C-RNTI is needed for subsequent uplink transmission. RLC configuration is also needed to store…. |
| Google |  | Dependent on the conclusion of proposal 2 of Q3 |
| Intel Corporation | Yes, but | As commented above, it is better to let the full context remain in the DU for design simplicity and forward compatibility. |
| CATT | Yes |  |
| LGE | Yes |  |
| Qualcomm | Yes |  |
| Samsung | Yes | The CG-SDT bearers include SDT-DRBs and SDT-SRB, if any. |
| Nokia | Partly | OK for C-RNTI For RLC configuration: Related to p2 of Q3: what is the gain to store RLC config and the F1-U tunnel in DU if in the end the UL data is blocked at CU UP waiting for validation of MAC-I ? |

Moderator’s conclusions:

- Proposal can be agreed:

- gNB-DU shall store which bearers are CG-SDT bearers, the C-RNTI

- WAs to be taken, since dependent of the previous WAs:

- WA4: gNB-DU shall store which bearers are CG-SDT bearers and the RLC configuration

### FFS gNB-DU should be aware of which bearers are SDT bearers

The moderator proposes to discuss if the gNB-DU shall be aware of SDT bearer for CG-SDT.

Q5: Companies are invited to provide their views on these proposals, whether they can be agreed?

| Company | Y/N | Comment |
| --- | --- | --- |
| Ericsson | No | We don’t see the need to differentiate bearers type in DU. |
| ZTE | Yes | The gNB-DU shall keep the SDT related F1 tunnel and F1 association, and discard non-SDT F1 tunnel and F1 association. But the gNB-DU does not need to differentiate either RA-SDT or CG-SDT. |
| Lenovo, Motorola Mobility | Yes | In order that the gNB-DU allocates the CG-SDT resource according to the QoS requirements of each related DRB, the CG-SDT Query Indication IE should be per DRB basis. |
| Huawei | Yes, but | It can be beneficial that the gNB-DU can configure the matched CG-SDT resource if the gNB-DU is aware of the SDT bearers. However there is no F1 impact, as the non-SDT bearers can be released via UE Context Modification procedure. |
| China Telecom | Yes | As F1 tunnel for SDT bearers shall be kept, gNB-DU shall be aware of which bearers are SDT bearers. |
| Google |  | Dependent on the conclusion of proposal 2 of Q3 |
| Intel Corporation | Yes | SDT bearer is a type of DRB configuration which we think that SDT capability of a certain DRB is rather static and not dynamically changed. It is better to configure from the beginning when setting up a DRB in the DU. With this, CU should be able to further change SDT capability of a DRB when it queries CG SDT via the UE Context Modification procedure. |
| CATT | Yes | F1-tunnel is per DRB level, thus SDT DRB should be known in gNB-DU. |
| LGE | Yes | As mentioned in Q3, when the gNB-CU sends the UE to RRC_INACTIVE state, the gNB-DU needs to keep the RLC bearer configuration and F1 tunnel information only for CG-SDT DRBs. |
| Qualcomm | Yes, however | Dependent on the conclusion of the previous question, as potentially different actions are required for SDT bearers at the DU etc, but one question is whether such indication is implicit in the SDT signalling itself (for example, if the CU notifies DU to store CG resource for SDT, it could specify the bearers at that point) |
| Samsung | Yes | The awareness of SDT bearer is needed since it can determine the context kept after releasing UE to INACTIVE. How to achieve such awareness needs further discussion. |
| Nokia | Partly | To me depends again on p2 Q3 in relation with keeping the F1-U tunnels: what is the gain to the F1-U tunnel in DU if in the end the UL data is blocked at CU UP waiting for validation of MAC-I ? |

Moderator’s conclusions:

- WA5: the gNB-DU can be aware of the bearer type, but it does not need RAN3 changes (awareness can be implicit).

### FFS on whether there is F1 impact to support the release of CG-SDT resource configuration due to TAT-SDT expiry.

When SDT-TAT expires, the kept CG-SDT resource configuration, the UE associated logical F1 connection and F1-U resource should be released. It is proposed that RAN3 selects one or more of the following solutions:

- Option 1: When the TAT-SDT expires, the gNB-DU initiates the UE Context Release Request procedure with a new cause value (e.g. from [7] ‘SDT-TAT expiry’).

- Option 2: gNB-DU provides the SDT-TAT to gNB-CU when providing CG-SDT resource configuration. When TAT-SDT expires, both the gNB-DU and gNB-CU release it locally.

- Option 3: the gNB-CU provides the previous UE F1 AP IDs to the gNB-DU to re-use the tunnel info during e.g. fallback from CG- to RA-SDT.

- Option 4: The gNB-CU does not need to be aware of CG-SDT resource validity. Nothing additional is to be done.

Q6: Companies are invited to provide their views on the above proposed options and comment on their preferred one(s) or why a specific option is discouraged.

| Company | Option(s) | Comment |
| --- | --- | --- |
| Ericsson | Option 3 and 4 |  |
| ZTE | Option 1 | We think TAT-SDT is running in gNB-DU, so the gNB-DU can initiate UE context release procedure when TAT-SDT expires. More, for the F1 tunnel removal, both CU and DU can initiate UE context release in some cases, e.g., the UE moves to another cell or another gNB. |
| Lenovo, Motorola Mobility | Option 1/2 | RAN2 has agreed that UE releases CG-SDT resource configuration when SDT-TAT expiries. Same with UE’s behaviour, the gNB-DU also needs to release the CG-SDT resource configuration due to SDT-TAT expires. Not sure option 3 is related with the FFS. |
| Huawei | Option 1 but FFS whether new cause needed | We understand that the TAT-SDT is separately maintained by the gNB-DU and the UE. The UE (re)starts the timer upon reception of RRCRelease and the TA command. In case of the timer expiration, the UE releases the CG-SDT resource. Correspondingly, the gNB-DU (re)starts the timer after sending RRCRelease including the TAT-SDT configuration or sending the TA command via MAC CE. Upon the timer expiry, the gNB-DU also releases the CG-SDT resource by itself. Therefore, there’s no F1 impact due to the TAT-SDT expiry. |
| China Telecom | Option 1 | Agree with Huawei. TAT-SDT shall be maintained by UE and gNB-DU. In case of timer expired, the gNB-DU can initiated UE context release procedure. |
| Google | Option 4 |  |
| Intel Corporation | Option 1 | Given that DU/UE maintains the timer and the UE just releases CG-SDT resource configuration when the timer expires, it is the same as if CG-SDT is not configured, which is the legacy INACTIVE case where there is no UE context in the last serving DU. It looks reasonable that DU initiates UE context release toward the CU. Whether new cause is needed or not depends on what other actions CU can do based on the cause value included in the release request from DU, which is not clear. We are open for new cause value but also fine without it. |
| CATT | Option 1 | TAT-SDT shall be maintained by UE and gNB-DU, the option 1 is the straightforward way to handle the TAT-SDT expiry case. |
| LGE | Option 1/2 | Agree with Lenovo. |
| Qualcomm | Option 1/2 | Basically option 1 in the sense that the DU should run the timer, and take action as it is mainly its resource that is being used. However it seems that CU needs to know the value of the timer. |
| Samsung | Option 4 Option 3 (although it is not related to FFS, which is covered by Section 3.2.7) | The legacy DU-initiated release procedure can be reused |
| Nokia | Option 1 |  |

Moderator’s conclusions:

- Considering the majority’s views and explanation, it is proposed to agreedoption1:

- When the TAT-SDT expires, the gNB-DU initiates the UE Context Release Request procedure with a new cause value (details to be checked).

### E1 impacts

It is proposed that when the gNB-CU-CP wants to resume the SDT DRB/SRB, it should include the SDT DRB/SRB list to be resumed in the E1AP: BEARER CONTEXT MODIFICATION REQUEST when the Bearer Context Status Change IE is set to “resume”. Upon reception of the resume request together with the SDT DRB/SRB list, the gNB-CU-UP should resume the requested SDT DRB/SRB.

Taking the encoding suggested in [13], it is proposed to build on it for a TP for agreement:

#### 9.3.3.x Bearer Context Status Change for SDT bearers This IE contains bearer context related information to indicate the status of SDT bearers IE/Group Name; Presence; Range; IE type and reference; Semantics description. Bearer Context Status Change for SDT beare; M; ENUMERATED (Suspend, Resume, …); Indicates the status of the Bearer Context. SDT bearers List; 1..<maxnoofPDUSessionResource>. >DRB ID; M; 9.3.1.16. >Continue ROHC; O; ENUMERATED (true, …); See description of drb-ContinueROHC inTS 38.331 [10].

Q7: Companies are invited to provide their views on the proposed new IE for the E1AP: BEARER CONTEXT MODIFICATION REQUEST message.

Other E1-related aspects that can be agreed this meeting can also be raised here to be considered in the TP.

| Company | Comment |
| --- | --- |
| ZTE | agree |
| Lenovo, Motorola Mobility | agree |
| Huawei | No need to introduce a new Bearer Context Status Change for SDT bearers IE, existing IEs can be reused. Continue ROHC is not needed. The legacy Continue ROHC IE in the PDCP Configuration IE-> ROHC Parameters can be reused. Only add new IE for the RB IDs to be resumed in the BEARER CONTEXT MODIFICATION REQUEST. And it is not necessary to differentiate the SDT bearer type. |
| China Telecom | Yes. From our point of view, CU-UP shall be aware of which bearers are SDT bearers. |
| Google | Not sure if the “Continue ROHC” IE should be included here. Should not it be like non-SDT bearers? |
| Intel Corporation | No, sorry. Why do we need to indicate which bearers to resume every time CU-CP requests resume due to SDT? Again, as explained in our paper [14], SDT bearer is a type of DRB configuration which we think that SDT capability of a certain DRB is rather static and not dynamically changed. It is better to configure from the beginning when setting up a bearer in the CU-UP. Then, we only need to indicate "resume for SDT". The existing Bearer Context Status Change IE can be extended to have one more codepoint for resume only for SDT capable bearers. And for suspend due to SDT, the existing codepoint of the existing Bearer Context Status Change can be re-used as for suspend, anyway all bearers are suspended.. |
| CATT | We are fine to use a separate IE Bearer Context Status Change for SDT bearer. On the Continue ROHC, share the view with HW, the legacy Continue ROHC IE in the PDCP Configuration IE-> ROHC Parameters could be reused. |
| LGE | We also think that the gNB-CU-UP needs to be aware of which bearer is used for CG-SDT because the gNB-CU-UP has different actions for SDT bearer and non-SDT bearer, respectively. For example, for the CG-SDT bearer, the gNB-CU-UP needs to keep the F1 DL TNL address. OK to use a new Bearer Context Status Change for SDT bearer IE. |
| Samsung | Agree. Stage-3 details need further discussion. The intention is to resume the data transmission for SDT bearers. So, this new IE only need to indicate the bearers for resuming to the UP. So, there is no need to add a dedicated IE, i.e., “Bearer Context Status Change for SDT beare”. |
| Nokia | Agree in principle. For stage 3 details, there are 2 possible solutions. We tend to prefer the Intel view that instead of a list of DRB IDs added to the E1 Bearer Ctxt Modification Request we could just add a codepoint in the Bearer Context Status Change IE. |

Moderator’s conclusions:

- Consensus for an E1 impact to the E1 Bearer Ctxt Modification Request.

- Proposal to add a new codepoint int the Bearer Context Status Change IE. Addition to be considered in the E1 output TP of “# SDT4_Others”

### FFS on whether the gNB-DU reconfigures or releases the old CG-SDT resource configuration via the new UE associated logical F1 connection in case of CG-SDT fallback to RA-SDT or non-SDT, and if yes, how to resolve it

When CG-SDT is configured but the UE selects to RA-SDT or non-SDT procedure (CG-SDT fallback to RA-SDT or non-SDT), a completely new UE associated logical F1 connection is setup for the UE during RA-SDT or non-SDT procedure. In the legacy behavior, when a new UE associated logical F1 connection is setup, the old UE associated logical F1 connection is released. But it does not allow delta configuration of CG-SDT resource configuration due to the old CG-SDT resource is completely released by the gNB-DU.

In order to support delta configuration of CG-SDT resource configuration in case that CG-SDT is configured but the UE selects to RA-SDT or non-SDT procedure, the following options can be considered:

- Option 1: The gNB-CU stores the old CG-SDT resource configuration. when CG-SDT is configured but the UE selects to RA-SDT or non-SDT procedure, the gNB-CU provides the old CG-SDT resource configuration to the gNB-DU when a new CG-SDT resource configuration is needed.

- Option 2: when CG-SDT is configured but the UE selects RA-SDT or non-SDT procedure, the gNB-CU provides the old gNB-DU F1AP UE ID to the gNB-DU. The gNB-DU retrieves the old CG-SDT resource configuration and old UE context based on the old gNB-DU F1AP UE ID.

- Q8: Companies are invited to provide their views on the above proposed options and comment on their preferred one(s) or why a specific option is discouraged.

| Company | Option(s) | Comment |
| --- | --- | --- |
| Lenovo, Motorola Mobility | Option 2 |  |
| Huawei | Option 2 | We propose to reuse the maintained F1-C/F1-U tunnel in case of CG-SDT access case, and CG-SDT fallback to RA-SDT cases. In case of CG-SDT fall back to RA-SDT or non-SDT at the same gNB, the gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU using the old F1AP association and provide the newly revived new UE DU F1AP ID as an new optional IE, the gNB-DU associates the new C-RNTI to the UE context, discard the new UE DU F1AP ID, discard the old C-RNTI, and if the UE accesses from new cell, the gNB-DU also releases the assigned CG-SDT resources. |
| China Telecom | Option 2 |  |
| Google |  | Prefer using the new F1 connection in case RA-SDT or non-SDT procedure is selected |
| Intel Corporation |  | First, we don't understand the issue why CU cannot retrieve CG-SDT configuration from old F1 to support delta toward new F1 for fallback. In case of CG SDT fallback to RACH SDT or non-SDT toward the same DU, DU doesn't have to know that the old F1 and new F1 belongs to the same UE. CU can know the fallback UE is the same UE who had been configured with CG SDT and do appropriate UE context managements to retrieve CG-SDT configuration from old F1 (before releasing it) and ues it to establish new F1 and provide CG-SDT configuration for delta signalling. |
| CATT | Option 2 |  |
| Samsung | Option 2 |  |
| ZTE | Option 2 |  |
| Nokia | Not sure, depends if my understanding is correct | My understanding is that the new F1 connection is used for RA-SDT (non-SDT) access. Then CU sends UE context setup request to DU over the new F1 connection including the old DU F1AP UE ID. DU identifies the previous UE context and can retrieve the stored CG SDT configuration which is sent in the UE context setup response over the new F1 connection. Is this correct? If yes, we need to consider the CG SDT configuration retrieval in F1 context setup? Or does CU wait for later F1 UE context modification? |

Moderator’s conclusions:

- Majority supports option 2

- when CG-SDT is configured but the UE selects RA-SDT or non-SDT procedure, the gNB-CU provides the old gNB-DU F1AP UE ID to the gNB-DU. The gNB-DU retrieves the old CG-SDT resource configuration and old UE context based on the old gNB-DU F1AP UE ID.

## TP for CG-SDT BLCR to TS 38.401

In this first round, we would suggest focusing on the procedures. It is proposed to discuss if the TP for CG-SDT BLCR to TS 38.401 in [3] can be reviewed to be agreed in the BL CR

Q9: Companies are invited to provide their views on the proposed BL CR in [3], comments on message and text details are welcome. Please state if any other TP can also be merged with it

| Company | Comment |
| --- | --- |
| Ericsson | TP [3] looks fine and can be slightly revised based on outcome of the above questions |
| ZTE | Agree with Ericsson |
| Lenovo, Motorola Mobility | Too early to be agreed. We would prefer to check the TPs in the second round according to the agreements we can made in the 1st round. |
| Huawei | Prefer TP [5] instead of TP [3] TP [3] has some issues, e.g. step 4/5, which message is FFS, and for wrong sequence for the step 12/13 and step 10/11, it is FFS. [ZTE]: Step 4/5 is straightforward to follow legacy procedure, however, FFS is also fine in this meeting. step 12/13 and step 10/11 is not problem, gNB-CU can decide either send F1 message to DU first or send E1 message to gNB-CU-UP first, it does not make sense. |
| China Telecom | Agree with Ericsson |
| Google | It depends on the conclusion to Q3 for steps 4-5. Also we think the DU does not buffer UL data and/or signalling data for the step 8 and then steps 10-11 seems not needed. |
| Intel Corporation | Prefer TP [14] instead of TP in [3] or [5], where the most of the steps are copy/paste. For CG-SDT, it works only in the same DU. We don’t need a completed call flow to understand. As proposed in [14], adding some notes onto Section 8.6.1 is enough. |
| CATT | Prefer to go for the TP [3] as it provides the overall procedure for CG-based SDT configuration, resume and data transmission. |
| LGE | Agree with Lenovo. We can select TP based on progress in Phase 1. |
| Qualcomm | Fine to take this as BL and revise according to first round conclusions. |
| Samsung | The details of TP need further discussion, which can be carried out in Phase 2. Some initial comments from our side: - In the figure, there is no need to include the IE. The detailed IE can be indicated in the text - Step 4: why CG-SDT lower layer config is needed? |
| Nokia | Let us check TP in second round. Some issues: why is RLC config not sent in step 4? steps 10, 11 not needed, etc.. [ZTE]: gNB-DU aleardy stores all the RLC config, when gNB-DU receives the step 4 message, it shall discard non-SDT RLC config but keeps the SDT RLC config. Step 10/11 are needed, but gNB-CU shall verify via I-RNIT，If verification pass, it notifies gNB-DU to send UL data. |

Moderator’s conclusions:

- Moderator added comments from the proponent in [3] regarding comments above on the TP. Based on the comments and arguments, it seems reasonable to agree on this TP

- Revise R3-220216, (TP for CG-SDT BLCR to TS 38.401) to be endorsed as BL CR

## Any other issues?

This e-mail discussions focuses on the proposal that might have spec impacts with agreed TPs, thus the proposals stating the negative, i.e., "this IE shall not be used/included in this message" were not presented. Nevertheless, companies are invited to provide their views if some aspects require discussion or should be captured in the minutes for attention.

Q-extra: any other comments comment would like to raise?

| Company | Comment |
| --- | --- |
| Ericsson | To discuss if st2 TP to 38.470 such as provided in [11] can be considered. Can be revised during second round based on progress. |
| ZTE | Agree with Ericsson |
| Huawei | Agree to Support of reusing the maintained F1-C/F1-U tunnel in case of CG-SDT access case, and CG-SDT fallback to RA-SDT cases. (as discussed in newly added 3.2.7) |
| Nokia | I think that the scenario of fallback to RA-SDT is not yet 100% clear, and the potential use of UE context setup to query the CG SDT configuration in this case. |
| Nokia | Don’t we need to discuss the verification of MAC-I? |

Moderator’s conclusions:

- F1AP Stage 2 TP seems agreeable

- Revise R3-220602, TP to TS 38.470 BL CR: Support of CG-SDT to be endorsed as BL CR for TS 38.470

- Rapporteur designed Lenovo as BL CR rapporteur

# Conclusion

Moderator’s conclusions:

- P1: Turn the previous WAs into agreements

- P2: The UE Context Setup procedure is not impacted for CG-SDT configuration exchange

- Take the following WAs:

  - WA1: When the gNB-DU receives the query indication, it should transfer the CG-SDT related resources within the DU to CU RRC Information IE. Introduce an SDT-MACPHY-Config IE to DU to CU RRC Information IE for the gNB-CU to generate the RRC Release message with CG-SDT config;

  - WA2: The gNB-CU notifies gNB-DU to keep SDT RLC config and store CG resource for SDT when UE entering RRC inactive;

  - WA3: The gNB-DU shall keep SDT RLC config and store CG resource for SDT when UE entering RRC inactive.

- P3: gNB-DU shall store which bearers are CG-SDT bearers, the C-RNTI

  - WA4: gNB-DU shall store which bearers are CG-SDT bearers and the RLC configuration

  - WA5: the gNB-DU can be aware of the bearer type (SDT Bearer), but it does not need RAN3 changes (awareness can be implicit).

- P4: When the TAT-SDT expires, the gNB-DU initiates the UE Context Release Request procedure (details to be checked, FFS on new cause).

- P5: Proposal to add a new codepoint int the Bearer Context Status Change IE. Addition to be considered in the E1 output TP of “# SDT4_Others”

- P6: when CG-SDT is configured but the UE selects RA-SDT or non-SDT procedure, the gNB-CU provides the old gNB-DU F1AP UE ID to the gNB-DU. The gNB-DU retrieves the old CG-SDT resource configuration and old UE context based on the old gNB-DU F1AP UE ID.

- TP1: Revise R3-220216, to be agreed as TP for BL CR to TS 38.401. WI Rapporteur designated Huawei as BL CR rapporteur

- TP2: Revise R3-220602, to be agreed as TP for BL CR for TS 38.470. WI Rapporteur designated Lenovo as BL CR rapporteur

Discuss in second round:

- Whether the WAs 1-5 can be agreed

- Check TPs and BL CRs

# References

R3-220205, Support of CG-based SDT (Nokia, Nokia Shanghai Bell)

R3-220206, (TP for TS 38.473) Support of CG-based SDT (Nokia, Nokia Shanghai Bell)

R3-220216, (TP for CG-SDT BLCR to TS 38.401) Overall procedure for CG-SDT (ZTE, China Telecom)

R3-220217, (TP for CG-SDT BLCR to TS 38.473) Support of CG-SDT (ZTE, China Telecom)

R3-220426, (TP to TS 38.401 BL CR) Support of CG-SDT (Huawei)

R3-220445, Discussion on F1 impacts for supporting CG-based SDT (Ericsson)

R3-220498, On CG based SDT (Lenovo, Motorola Mobility)

R3-220552, (TP for CG-SDT BL CR to TS 38.463) Support of CG-SDT (LG Electronics)

R3-220553, (TP for CG-SDT BL CR to TS 38.473) Support of CG-SDT (LG Electronics)

R3-220567, (TP to CG-SDT BL CR of TS38.473) Discussion on CG-based small data transmission (Samsung, ZTE)

R3-220602, TP to TS 38.470 BL CR: Support of CG-SDT (Ericsson LM)

R3-220814, E1 impact on SDT (China Telecom Corporation Ltd.)

R3-220815, TP to TS38.463 on the support of SDT in E1 interface (China Telecom Corporation Ltd.)

R3-220842, (TP for CG-SDT BL CR for TS 38.473, 38.463, 38.401) Discussion on CG based SDT (Intel Corporation)