3GPP TSG-RAN WG3 Meeting #114bis-e	R3-221125

E-meeting, 17-26 January 2022

Title: 	Supporting Redcap UEs over Xn interface

Source: 	Huawei, ZTE

Agenda item:	11.3

Document Type:	other

- Introduction

This TP captures the agreements in CB # RedCap3_eDRX

5. Text proposal to TS 38.423

Change Begins for TS 38.423

8.2.5	RAN Paging

8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

The RAN Paging procedure is triggered by the NG-RAN node1 by sending the RAN PAGING message to the NG-RAN node2, in which the necessary information e.g. UE RAN Paging Identity should be provided.

If the Paging Priority IE is included in the RAN PAGING message, the NG-RAN node2 may use it to prioritize paging.

If the Assistance Data for RAN Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 38.300 [9].

If the UE Radio Capability for Paging IE is included in the RAN PAGING message, the NG-RAN node2 may use it to apply specific paging schemes.

If the Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 may use it according to TS 36.304 [34]. When available, NG-RAN node1 may include the Extended UE Identity Index Value IE in the RAN PAGING message towards an ng-eNB (e.g. NG-RAN node2).

When available, the NG-RAN node1 shall include the Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the UE Specific DRX IE in the RAN PAGING message towards the NG-RAN node2. If the UE specific DRX IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 36.304 [34].

When available, the NG-RAN node1 shall include the NR Paging eDRX Information IE in the RAN PAGING message towards the NG-RAN node2. If the NR Paging eDRX Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the NR Paging eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

<Unchanged Text Omitted>

9.1.1.7	RAN PAGING

This message is sent by the NG-RAN node1 to NG-RAN node2 to page a UE.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE UE Identity Index Value | M |  |  |  | YES | reject |
| >Length-10 |  |  |  |  |  |  |
| >>Index Length-10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [33] and TS 36.304 [34]. | – |  |
| UE RAN Paging Identity | M |  | 9.2.3.43 |  | YES | ignore |
| Paging DRX | M |  | 9.2.3.66 | Includes the RAN paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | ignore |
| RAN Paging Area | M |  | 9.2.3.38 |  | YES | reject |
| Paging Priority | O |  | 9.2.3.44 |  | YES | ignore |
| Assistance Data for RAN Paging | O |  | 9.2.3.41 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.3.91 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.141 | Coded as specified in TS 36.304 [34]. | YES | ignore |
| Paging eDRX Information | O |  | 9.2.3.142 |  | YES | ignore |
| UE specific DRX | O |  | 9.2.3.143 | Includes the UE specific paging cycle as defined in TS 36.304 [34] and 38.304 [33]. | YES | Ignore |
| NR Paging eDRX Information | O |  | 9.2.3.X |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.2.3.Y |  | YES | ignore |

<Unchanged Text Omitted>

9.2.3.X	NR  Paging eDRX Information

This IE indicates the NR Paging eDRX parameters for RRC_IDLE as defined in TS 38.304 [33].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR Paging eDRX Cycle Idle | M |  | ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …) | TeDRX defined in TS 38.304 [33]. Unit: [number of hyperframes]. |
| NR Paging Time Window | O |  | ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, s17, s18, s19, s20, s21, s22, s23, s24, s25, s26, s27, s28, s29, s30, s31, s32, …) | Unit: [1.28 seconds] |

9.2.3.Y	Paging eDRX Information for RRC INACTIVE

This IE indicates the NR Paging eDRX parameters for RRC_INACTIVE as defined in TS 38.304 [33].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR Paging eDRX Cycle Inactive | M |  | ENUMERATED (hfquarter, hfhalf, hf1, …) | TeDRX defined in TS 38.304 [33]. Unit: [number of hyperframes]. |

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

AMF-Region-Information,

AMF-UE-NGAP-ID,

AS-SecurityInformation,

AssistanceDataForRANPaging,

BitRate,

Cause,

CellAndCapacityAssistanceInfo-EUTRA,

CellAndCapacityAssistanceInfo-NR,

CellAssistanceInfo-EUTRA,

CellAssistanceInfo-NR,

CHOinformation-Req,

CHOinformation-Ack,

CHO-MRDC-EarlyDataForwarding,

CHO-MRDC-Indicator,

CPTransportLayerInformation,

TNLA-To-Add-List,

TNLA-To-Update-List,

TNLA-To-Remove-List,

TNLA-Setup-List,

TNLA-Failed-To-Setup-List,

CriticalityDiagnostics,

XnUAddressInfoperPDUSession-List,

DAPSResponseInfo-List,

DataTrafficResourceIndication,

DeliveryStatus,

DesiredActNotificationLevel,

DRB-ID,

DRB-List,

DRB-Number,

DRBsSubjectToDLDiscarding-List,

DRBsSubjectToEarlyStatusTransfer-List,

DRBsSubjectToStatusTransfer-List,

DRBToQoSFlowMapping-List,

E-UTRA-CGI,

ExpectedUEActivityBehaviour,

ExpectedUEBehaviour,

ExtendedUEIdentityIndexValue,

FiveGCMobilityRestrictionListContainer,

GlobalCell-ID,

GlobalNG-RANNode-ID,

GlobalNG-RANCell-ID,

GUAMI,

InterfaceInstanceIndication,

I-RNTI,

LocationInformationSNReporting,

LocationReportingInformation,

LowerLayerPresenceStatusChange,

LTEUESidelinkAggregateMaximumBitRate,

LTEV2XServicesAuthorized,

MR-DC-ResourceCoordinationInfo,

ServedCells-E-UTRA,

ServedCells-NR,

ServedCellsToUpdate-E-UTRA,

ServedCellsToUpdate-NR,

MAC-I,

MaskedIMEISV,

MDT-Configuration,

MDTPLMNList,

MobilityRestrictionList,

NG-RAN-Cell-Identity,

NG-RANnodeUEXnAPID,

NR-CGI,

NE-DC-TDM-Pattern,

NRUESidelinkAggregateMaximumBitRate,

NRV2XServicesAuthorized,

PagingDRX,

PagingeDRXInformation,

PagingPriority,

PartialListIndicator,

PLMN-Identity,

PDCPChangeIndication,

PDUSessionAggregateMaximumBitRate,

PDUSession-ID,

PDUSession-List,

PDUSession-List-withCause,

PDUSession-List-withDataForwardingFromTarget,

PDUSession-List-withDataForwardingRequest,

PDUSessionResourcesAdmitted-List,

PDUSessionResourcesNotAdmitted-List,

PDUSessionResourcesToBeSetup-List,

PDUSessionResourceChangeRequiredInfo-SNterminated,

PDUSessionResourceChangeRequiredInfo-MNterminated,

PDUSessionResourceChangeConfirmInfo-SNterminated,

PDUSessionResourceChangeConfirmInfo-MNterminated,

PDUSessionResourceSecondaryRATUsageList,

PDUSessionResourceSetupInfo-SNterminated,

PDUSessionResourceSetupInfo-MNterminated,

PDUSessionResourceSetupResponseInfo-SNterminated,

PDUSessionResourceSetupResponseInfo-MNterminated,

PDUSessionResourceModificationInfo-SNterminated,

PDUSessionResourceModificationInfo-MNterminated,

PDUSessionResourceModificationResponseInfo-SNterminated,

PDUSessionResourceModificationResponseInfo-MNterminated,

PDUSessionResourceModConfirmInfo-SNterminated,

PDUSessionResourceModConfirmInfo-MNterminated,

PDUSessionResourceModRqdInfo-SNterminated,

PDUSessionResourceModRqdInfo-MNterminated,

PDUSessionType,

PC5QoSParameters,

QoSFlowIdentifier,

QoSFlowNotificationControlIndicationInfo,

QoSFlows-List,

RANPagingArea,

ResetRequestTypeInfo,

ResetResponseTypeInfo,

RFSP-Index,

RRCConfigIndication,

RRCResumeCause,

SCGConfigurationQuery,

SecurityIndication,

S-NG-RANnode-SecurityKey,

SpectrumSharingGroupID,

SplitSRBsTypes,

S-NG-RANnode-Addition-Trigger-Ind,

S-NSSAI,

TargetCellList,

TAISupport-List,

Target-CGI,

TimeToWait,

TraceActivation,

UEAggregateMaximumBitRate,

UEContextID,

UEContextInfoRetrUECtxtResp,

UEContextKeptIndicator,

UEHistoryInformation,

UEIdentityIndexValue,

UERadioCapabilityForPaging,

UERadioCapabilityID,

UERANPagingIdentity,

UESecurityCapabilities,

UPTransportLayerInformation,

UserPlaneTrafficActivityReport,

XnBenefitValue,

RANPagingFailure,

TNLConfigurationInfo,

MaximumCellListSize,

MessageOversizeNotification,

NG-RANTraceID,

MobilityInformation,

InitiatingCondition-FailureIndication,

HandoverReportType,

TargetCellinEUTRAN,

C-RNTI,

UERLFReportContainer,

Measurement-ID,

RegistrationRequest,

ReportCharacteristics,

CellToReport,

ReportingPeriodicity,

CellMeasurementResult,

UEHistoryInformationFromTheUE,

MobilityParametersInformation,

MobilityParametersModificationRange,

RACHReportInformation,

IABNodeIndication,

SNTriggered,

SCGIndicator,

UESpecificDRX,

NRPagingeDRXInformation,

NRPagingeDRXInformationforRRCINACTIVE

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

id-ActivatedServedCells,

id-ActivationIDforCellActivation,

id-AdditionalDRBIDs,

id-AMF-Region-Information,

id-AMF-Region-Information-To-Add,

id-AMF-Region-Information-To-Delete,

id-AssistanceDataForRANPaging,

id-AvailableDRBIDs,

id-Cause,

id-cellAssistanceInfo-EUTRA,

id-cellAssistanceInfo-NR,

id-CellAndCapacityAssistanceInfo-EUTRA,

id-CellAndCapacityAssistanceInfo-NR,

id-ConfigurationUpdateInitiatingNodeChoice,

id-UEContextID,

id-CriticalityDiagnostics,

id-XnUAddressInfoperPDUSession-List,

id-DesiredActNotificationLevel,

id-DRBsSubjectToStatusTransfer-List,

id-ExpectedUEBehaviour,

id-ExtendedUEIdentityIndexValue,

id-FiveGCMobilityRestrictionListContainer,

id-GlobalNG-RAN-node-ID,

id-GUAMI,

id-indexToRatFrequSelectionPriority,

id-List-of-served-cells-E-UTRA,

id-List-of-served-cells-NR,

id-LocationInformationSN,

id-LocationInformationSNReporting,

id-LocationReportingInformation,

id-LTEUESidelinkAggregateMaximumBitRate,

id-LTEV2XServicesAuthorized,

id-MAC-I,

id-MaskedIMEISV,

id-MDT-Configuration,

id-MDTPLMNList,

id-MN-to-SN-Container,

id-MobilityRestrictionList,

id-M-NG-RANnodeUEXnAPID,

id-new-NG-RAN-Cell-Identity,

id-newNG-RANnodeUEXnAPID,

id-NRUESidelinkAggregateMaximumBitRate,

id-NRV2XServicesAuthorized,

id-oldNG-RANnodeUEXnAPID,

id-OldtoNewNG-RANnodeResumeContainer,

id-PagingDRX,

id-PagingeDRXInformation,

id-PagingPriority,

id-PartialListIndicator-EUTRA,

id-PartialListIndicator-NR,

id-PCellID,

id-PDUSessionResourceSecondaryRATUsageList,

id-PDUSessionResourcesActivityNotifyList,

id-PDUSessionResourcesAdmitted-List,

id-PDUSessionResourcesNotAdmitted-List,

id-PDUSessionResourcesNotifyList,

id-PDUSessionToBeAddedAddReq,

id-PDUSessionToBeReleased-RelReqAck,

id-procedureStage,

id-RANPagingArea,

id-requestedSplitSRB,

id-RequiredNumberOfDRBIDs,

id-ResetRequestTypeInfo,

id-ResetResponseTypeInfo,

id-RespondingNodeTypeConfigUpdateAck,

id-RRCResumeCause,

id-selectedPLMN,

id-ServedCellsToActivate,

id-servedCellsToUpdate-E-UTRA,

id-ServedCellsToUpdateInitiatingNodeChoice,

id-servedCellsToUpdate-NR,

id-sourceNG-RANnodeUEXnAPID,

id-SpareDRBIDs,

id-S-NG-RANnodeMaxIPDataRate-UL,

id-S-NG-RANnodeMaxIPDataRate-DL,

id-S-NG-RANnodeUEXnAPID,

id-TAISupport-list,

id-Target2SourceNG-RANnodeTranspContainer,

id-targetCellGlobalID,

id-targetNG-RANnodeUEXnAPID,

id-TimeToWait,

id-TNLA-To-Add-List,

id-TNLA-To-Update-List,

id-TNLA-To-Remove-List,

id-TNLA-Setup-List,

id-TNLA-Failed-To-Setup-List,

id-TraceActivation,

id-UEContextInfoHORequest,

id-UEContextInfoRetrUECtxtResp,

id-UEContextKeptIndicator,

id-UEContextRefAtSN-HORequest,

id-UEHistoryInformation,

id-UEIdentityIndexValue,

id-UERANPagingIdentity,

id-UESecurityCapabilities,

id-UserPlaneTrafficActivityReport,

id-XnRemovalThreshold,

id-PDUSessionAdmittedAddedAddReqAck,

id-PDUSessionNotAdmittedAddReqAck,

id-SN-to-MN-Container,

id-RRCConfigIndication,

id-SplitSRB-RRCTransfer,

id-UEReportRRCTransfer,

id-PDUSessionReleasedList-RelConf,

id-BearersSubjectToCounterCheck,

id-PDUSessionToBeReleasedList-RelRqd,

id-ResponseInfo-ReconfCompl,

id-initiatingNodeType-ResourceCoordRequest,

id-respondingNodeType-ResourceCoordResponse,

id-PDUSessionToBeReleased-RelReq,

id-PDUSession-SNChangeRequired-List,

id-PDUSession-SNChangeConfirm-List,

id-PDCPChangeIndication,

id-PC5QoSParameters,

id-SCGConfigurationQuery,

id-UEContextInfo-SNModRequest,

id-requestedSplitSRBrelease,

id-PDUSessionAdmitted-SNModResponse,

id-PDUSessionNotAdmitted-SNModResponse,

id-admittedSplitSRB,

id-admittedSplitSRBrelease,

id-PDUSessionAdmittedModSNModConfirm,

id-PDUSessionReleasedSNModConfirm,

id-s-ng-RANnode-SecurityKey,

id-PDUSessionToBeModifiedSNModRequired,

id-S-NG-RANnodeUE-AMBR,

id-PDUSessionToBeReleasedSNModRequired,

id-target-S-NG-RANnodeID,

id-S-NSSAI,

id-MR-DC-ResourceCoordinationInfo,

id-RANPagingFailure,

id-UERadioCapabilityForPaging,

id-PDUSessionDataForwarding-SNModResponse,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-NE-DC-TDM-Pattern,

id-InterfaceInstanceIndication,

id-S-NG-RANnode-Addition-Trigger-Ind,

id-SNTriggered,

id-DRBs-transferred-to-MN,

id-TNLConfigurationInfo,

id-MessageOversizeNotification,

id-NG-RANTraceID,

id-FastMCGRecoveryRRCTransfer-SN-to-MN,

id-FastMCGRecoveryRRCTransfer-MN-to-SN,

id-RequestedFastMCGRecoveryViaSRB3,

id-AvailableFastMCGRecoveryViaSRB3,

id-RequestedFastMCGRecoveryViaSRB3Release,

id-ReleaseFastMCGRecoveryViaSRB3,

id-CHOinformation-Req,

id-CHOinformation-Ack,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-DAPSResponseInfo-List,

id-CHO-MRDC-EarlyDataForwarding,

id-CHO-MRDC-Indicator,

id-MobilityInformation,

id-InitiatingCondition-FailureIndication,

id-UEHistoryInformationFromTheUE,

id-HandoverReportType,

id-HandoverCause,

id-SourceCellCGI,

id-TargetCellCGI,

id-ReEstablishmentCellCGI,

id-TargetCellinEUTRAN,

id-SourceCellCRNTI,

id-UERLFReportContainer,

id-NGRAN-Node1-Measurement-ID,

id-NGRAN-Node2-Measurement-ID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReport,

id-ReportingPeriodicity,

id-CellMeasurementResult,

id-NG-RANnode1CellID,

id-NG-RANnode2CellID,

id-NG-RANnode1MobilityParameters,

id-NG-RANnode2ProposedMobilityParameters,

id-MobilityParametersModificationRange,

id-RACHReportInformation,

id-IABNodeIndication,

id-UERadioCapabilityID,

id-SCGIndicator,

id-UESpecificDRX,

id-PDUSessionExpectedUEActivityBehaviour,

id-NRPagingeDRXInformation,

id-NRPagingeDRXInformationforRRCINACTIVE,

<Skip unchanged part>

-- **************************************************************

--

#### -- RAN PAGING

--

-- **************************************************************

RANPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{RANPaging-IEs}},

...

}

RANPaging-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue				CRITICALITY reject		TYPE UEIdentityIndexValue							PRESENCE mandatory}|

{ ID id-UERANPagingIdentity					CRITICALITY ignore		TYPE UERANPagingIdentity							PRESENCE mandatory}|

{ ID id-PagingDRX							CRITICALITY ignore		TYPE PagingDRX										PRESENCE mandatory}|

{ ID id-RANPagingArea						CRITICALITY reject		TYPE RANPagingArea									PRESENCE mandatory}|

{ ID id-PagingPriority						CRITICALITY ignore		TYPE PagingPriority									PRESENCE optional }|

{ ID id-AssistanceDataForRANPaging			CRITICALITY ignore		TYPE AssistanceDataForRANPaging					PRESENCE optional }|

{ ID id-UERadioCapabilityForPaging			CRITICALITY ignore		TYPE UERadioCapabilityForPaging					PRESENCE optional }|

{ ID id-ExtendedUEIdentityIndexValue		CRITICALITY ignore		TYPE ExtendedUEIdentityIndexValue				PRESENCE optional }|

{ ID id-PagingeDRXInformation				CRITICALITY ignore		TYPE PagingeDRXInformation						PRESENCE optional }|

{ ID id-UESpecificDRX						CRITICALITY ignore		TYPE UESpecificDRX									PRESENCE optional }|

{ ID id-NRPagingeDRXInformation				CRITICALITY ignore		TYPE NRPagingeDRXInformation					     PRESENCE optional }|

{ ID id-NRPagingeDRXInformationforRRCINACTIVE	CRITICALITY ignore		TYPE NRPagingeDRXInformationforRRCINACTIVE		  PRESENCE optional }

,

...

}

NRCellPRACHConfig ::= OCTET STRING

NRPagingeDRXInformation ::= SEQUENCE {

nrpaging-eDRX-Cycle-Idle		NRPaging-eDRX-Cycle-Idle,

nrpaging-Time-Window	NRPaging-Time-Window											OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {NRPagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

NRPagingeDRXInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRPaging-eDRX-Cycle-Idle ::= ENUMERATED {

hfquarter, hfhalf, hf1, hf2, hf4, hf6,

hf8, hf10, hf12, hf14, hf16,

hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NRPaging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

s17, s18, s19, s20, s21, s22,

s23, s24, s25, s26, s27, s28,

s29, s30, s31, s32,

...

}

NRPagingeDRXInformationforRRCINACTIVE ::= SEQUENCE {

nrpaging-eDRX-Cycle-Inactive		NRPaging-eDRX-Cycle-Inactive,

iE-Extensions			ProtocolExtensionContainer { { NRPagingeDRXInformationforRRCINACTIVE-ExtIEs} }	OPTIONAL,

...

}

NRPagingeDRXInformationforRRCINACTIVE-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

NRPaging-eDRX-Cycle-Inactive ::= ENUMERATED {

hfquarter, hfhalf, hf1,

...

}

NG-RAN-Cell-Identity ::= CHOICE {

nr						NR-Cell-Identity,

e-utra					E-UTRA-Cell-Identity,

choice-extension		ProtocolIE-Single-Container { {NG-RAN-Cell-Identity-ExtIEs} }

}

NG-RAN-Cell-Identity-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

<Skip unchanged part>

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR																			ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 9

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List																	ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority																	ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest															ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 18

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 19

id-LocationReportingInformation																		ProtocolIE-ID ::= 20

id-MAC-I																							ProtocolIE-ID ::= 21

id-MaskedIMEISV																						ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 23

id-MN-to-SN-Container																				ProtocolIE-ID ::= 24

id-MobilityRestrictionList																			ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 27

id-UEReportRRCTransfer																				ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer																ProtocolIE-ID ::= 30

id-PagingDRX																						ProtocolIE-ID ::= 31

id-PCellID																							ProtocolIE-ID ::= 32

id-PDCPChangeIndication																				ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck																	ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm																ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse																	ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck																	ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse																ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf																	ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm																	ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList															ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList																	ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List																	ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List																	ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq																		ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired																ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd																ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq																	ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired																ProtocolIE-ID ::= 51

id-RANPagingArea																					ProtocolIE-ID ::= 52

id-PagingPriority																					ProtocolIE-ID ::= 53

id-requestedSplitSRB																				ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease																			ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo																				ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo																			ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck																ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse															ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl																			ProtocolIE-ID ::= 60

id-RRCConfigIndication																				ProtocolIE-ID ::= 61

id-RRCResumeCause																					ProtocolIE-ID ::= 62

id-SCGConfigurationQuery																			ProtocolIE-ID ::= 63

id-selectedPLMN																						ProtocolIE-ID ::= 64

id-ServedCellsToActivate																			ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey																			ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR																				ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 71

id-SN-to-MN-Container																				ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer																				ProtocolIE-ID ::= 74

id-TAISupport-list																					ProtocolIE-ID ::= 75

id-TimeToWait																						ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 77

id-targetCellGlobalID																				ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID																			ProtocolIE-ID ::= 80

id-TraceActivation																					ProtocolIE-ID ::= 81

id-UEContextID																						ProtocolIE-ID ::= 82

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest																		ProtocolIE-ID ::= 85

id-UEContextKeptIndicator																			ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest																		ProtocolIE-ID ::= 87

id-UEHistoryInformation																				ProtocolIE-ID ::= 88

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 89

id-UERANPagingIdentity																				ProtocolIE-ID ::= 90

id-UESecurityCapabilities																			ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport																	ProtocolIE-ID ::= 92

id-XnRemovalThreshold 																				ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel																		ProtocolIE-ID ::= 94

id-AvailableDRBIDs 																					ProtocolIE-ID ::= 95

id-AdditionalDRBIDs 																				ProtocolIE-ID ::= 96

id-SpareDRBIDs 																						ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs 																			ProtocolIE-ID ::= 98

id-TNLA-To-Add-List																					ProtocolIE-ID ::= 99

id-TNLA-To-Update-List																				ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List																				ProtocolIE-ID ::= 101

id-TNLA-Setup-List																					ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List																		ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck																	ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL																		ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LocationInformationSN																			ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 113

id-MaxIPrate-DL																						ProtocolIE-ID ::= 114

id-SecurityResult																					ProtocolIE-ID ::= 115

id-S-NSSAI																							ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add																	ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete																	ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected																ProtocolIE-ID ::= 120

id-RANPagingFailure																					ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-DRB-IDs-takenintouse																				ProtocolIE-ID ::= 133

id-SplitSessionIndicator																			ProtocolIE-ID ::= 134

id-CNTypeRestrictionsForEquivalent																	ProtocolIE-ID ::= 135

id-CNTypeRestrictionsForServing																		ProtocolIE-ID ::= 136

id-DRBs-transferred-to-MN																			ProtocolIE-ID ::= 137

id-ULForwardingProposal																				ProtocolIE-ID ::= 138

id-EndpointIPAddressAndPort 																		ProtocolIE-ID ::= 139

id-IntendedTDD-DL-ULConfiguration-NR																ProtocolIE-ID ::= 140

id-TNLConfigurationInfo																				ProtocolIE-ID ::= 141

id-PartialListIndicator-NR																			ProtocolIE-ID ::= 142

id-MessageOversizeNotification																		ProtocolIE-ID ::= 143

id-CellAndCapacityAssistanceInfo-NR																	ProtocolIE-ID ::= 144

id-NG-RANTraceID																					ProtocolIE-ID ::= 145

id-NonGBRResources-Offered																			ProtocolIE-ID ::= 146

id-FastMCGRecoveryRRCTransfer-SN-to-MN																ProtocolIE-ID ::= 147

id-RequestedFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 148

id-AvailableFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 149

id-RequestedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 150

id-ReleaseFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN																ProtocolIE-ID ::= 152

id-ExtendedRATRestrictionInformation																ProtocolIE-ID ::= 153

id-QoSMonitoringRequest																				ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer															ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA																		ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA																ProtocolIE-ID ::= 157

id-CHOinformation-Req																				ProtocolIE-ID ::= 158

id-CHOinformation-Ack																				ProtocolIE-ID ::= 159

id-targetCellsToCancel																				ProtocolIE-ID ::= 160

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= 161

id-procedureStage																					ProtocolIE-ID ::= 162

id-DAPSRequestInfo																					ProtocolIE-ID ::= 163

id-DAPSResponseInfo-List																			ProtocolIE-ID ::= 164

id-CHO-MRDC-Indicator																				ProtocolIE-ID ::= 165

id-OffsetOfNbiotChannelNumberToDL-EARFCN															ProtocolIE-ID ::= 166

id-OffsetOfNbiotChannelNumberToUL-EARFCN															ProtocolIE-ID ::= 167

id-NBIoT-UL-DL-AlignmentOffset																		ProtocolIE-ID ::= 168

id-LTEV2XServicesAuthorized																			ProtocolIE-ID ::= 169

id-NRV2XServicesAuthorized																			ProtocolIE-ID ::= 170

id-LTEUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 171

id-NRUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 172

id-PC5QoSParameters																					ProtocolIE-ID ::= 173

id-AlternativeQoSParaSetList																		ProtocolIE-ID ::= 174

id-CurrentQoSParaSetIndex																			ProtocolIE-ID ::= 175

id-MobilityInformation 																				ProtocolIE-ID ::= 176

id-InitiatingCondition-FailureIndication															ProtocolIE-ID ::= 177

id-UEHistoryInformationFromTheUE																	ProtocolIE-ID ::= 178

id-HandoverReportType																				ProtocolIE-ID ::= 179

id-HandoverCause																					ProtocolIE-ID ::= 180

id-SourceCellCGI																					ProtocolIE-ID ::= 181

id-TargetCellCGI																					ProtocolIE-ID ::= 182

id-ReEstablishmentCellCGI																			ProtocolIE-ID ::= 183

id-TargetCellinEUTRAN																				ProtocolIE-ID ::= 184

id-SourceCellCRNTI																					ProtocolIE-ID ::= 185

id-UERLFReportContainer																				ProtocolIE-ID ::= 186

id-NGRAN-Node1-Measurement-ID																		ProtocolIE-ID ::= 187

id-NGRAN-Node2-Measurement-ID																		ProtocolIE-ID ::= 188

id-RegistrationRequest																				ProtocolIE-ID ::= 189

id-ReportCharacteristics																				ProtocolIE-ID ::= 190

id-CellToReport																								ProtocolIE-ID ::= 191

id-ReportingPeriodicity																					ProtocolIE-ID ::= 192

id-CellMeasurementResult																				ProtocolIE-ID ::= 193

id-NG-RANnode1CellID																						ProtocolIE-ID ::= 194

id-NG-RANnode2CellID																				ProtocolIE-ID ::= 195

id-NG-RANnode1MobilityParameters																	ProtocolIE-ID ::= 196

id-NG-RANnode2ProposedMobilityParameters															ProtocolIE-ID ::= 197

id-MobilityParametersModificationRange																ProtocolIE-ID ::= 198

id-TDDULDLConfigurationCommonNR																		ProtocolIE-ID ::= 199

id-CarrierList																						ProtocolIE-ID ::= 200

id-ULCarrierList																					ProtocolIE-ID ::= 201

id-FrequencyShift7p5khz																				ProtocolIE-ID ::= 202

id-SSB-PositionsInBurst																				ProtocolIE-ID ::= 203

id-NRCellPRACHConfig																				ProtocolIE-ID ::= 204

id-RACHReportInformation																			ProtocolIE-ID ::= 205

id-IABNodeIndication																				ProtocolIE-ID ::= 206

id-Redundant-UL-NG-U-TNLatUPF																		ProtocolIE-ID ::= 207

id-CNPacketDelayBudgetDownlink																		ProtocolIE-ID ::= 208

id-CNPacketDelayBudgetUplink																		ProtocolIE-ID ::= 209

id-Additional-Redundant-UL-NG-U-TNLatUPF-List														ProtocolIE-ID ::= 210

id-RedundantCommonNetworkInstance																	ProtocolIE-ID ::= 211

id-TSCTrafficCharacteristics																		ProtocolIE-ID ::= 212

id-RedundantQoSFlowIndicator																		ProtocolIE-ID ::= 213

id-Redundant-DL-NG-U-TNLatNG-RAN																	ProtocolIE-ID ::= 214

id-ExtendedPacketDelayBudget																		ProtocolIE-ID ::= 215

id-Additional-PDCP-Duplication-TNL-List																ProtocolIE-ID ::= 216

id-RedundantPDUSessionInformation																	ProtocolIE-ID ::= 217

id-UsedRSNInformation																				ProtocolIE-ID ::= 218

id-RLCDuplicationInformation																		ProtocolIE-ID ::= 219

id-NPN-Broadcast-Information																		ProtocolIE-ID ::= 220

id-NPNPagingAssistanceInformation																	ProtocolIE-ID ::= 221

id-NPNMobilityInformation																			ProtocolIE-ID ::= 222

id-NPN-Support																						ProtocolIE-ID ::= 223

id-MDT-Configuration																				ProtocolIE-ID ::= 224

id-MDTPLMNList																						ProtocolIE-ID ::= 225

id-TraceCollectionEntityURI																			ProtocolIE-ID ::= 226

id-UERadioCapabilityID																				ProtocolIE-ID ::= 227

id-CSI-RSTransmissionIndication																		ProtocolIE-ID ::= 228

id-SNTriggered  																					ProtocolIE-ID ::= 229

id-DLCarrierList																					ProtocolIE-ID ::= 230

id-ExtendedTAISliceSupportList																		ProtocolIE-ID ::= 231

id-cellAssistanceInfo-EUTRA																			ProtocolIE-ID ::= 232

id-ConfiguredTACIndication																			ProtocolIE-ID ::= 233

id-secondary-SN-UL-PDCP-UP-TNLInfo																	ProtocolIE-ID ::= 234

id-pdcpDuplicationConfiguration																		ProtocolIE-ID ::= 235

id-duplicationActivation																			ProtocolIE-ID ::= 236

id-NPRACHConfiguration																				ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency																	ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated													ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SFN-Offset																						ProtocolIE-ID ::= 242

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-PagingeDRXInformation																			ProtocolIE-ID ::= 245

id-CHO-MRDC-EarlyDataForwarding																		ProtocolIE-ID ::= 246

id-SCGIndicator																						ProtocolIE-ID ::= 247

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-NRPagingeDRXInformation                                                                        ProtocolIE-ID ::= xxx

id-NRPagingeDRXInformationforRRCINACTIVE                                                          ProtocolIE-ID ::= yyy

END

-- ASN1STOP

Change Ends for TS 38.423