3GPP TSG-RAN WG3 Meeting #114bis-e	R3-221256

17-26 Jan 2022

Agenda item:	13.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for BL CR for TS 38.401) Inter donor-DU re-routing

Document for:	Discussion and Decision

This contribution proposes Stage-2 TP for inter-Donor-DU routing, per CB: # 1306_IAB_Multi-hop.

Proposed TP for BL CR for TS 38.401:

## 8.x1	IAB Inter-Donor-DU routing

When an IAB-donor-DU is configured with the information to support inter-donor-DU routing, the IAB-donor-DU may identify the re-routed UL IP packet based on the source IP address field of the UL packet, and forward the re-routed UL IP packet to the peer IAB-donor-DU via a tunnel. In the intra-donor-CU scenario for UL inter-donor-DU rerouting, the IAB-donor-DU and its peer IAB-donor-DU are controlled by the same IAB-donor-CU. In the inter-donor-CU case, the IAB-donor-DU and its peer IAB-donor-DU are controlled by different IAB-donor-CUs.

Editor’s Note: it is FFS on the tunnel type.