3GPP TSG-RAN WG3 Meeting #114bis-e	R3-221318

Online, January 17th – 26th 2022

Title:	 [Draft] Support for RAN Visible QoE Measurements

Response to:	-

Release:	Rel-17

Study Item:	NR_QOE

Source:	Ericsson [To be RAN3]

To:	CT1

Cc:	RAN2-

Contact Person:

Name: 		Filip Barac

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:      3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall description

In the legacy QoE management framework (defined for UTRA and E-UTRA), the QoE measurement configuration and the QoE measurement report are delivered from the network to the UE and from the UE to the network, respectively, inside containers (i.e., XML files) whose contents are not visible to the RAN. CT1 has, in the specification TS 27.007, defined the AT commands +CAPPLEVMC and +CAPPLEVMR for delivering the QoE configuration and report containers between the UE AS and the UE Application layer.

In the Rel-17 RAN2/3 Work Item on “NR QoE management and optimizations for diverse services”, RAN3 agreed to support the concept RAN visible QoE measurements, where:

- The RAN assembles a RAN visible QoE measurement configuration and delivers it to the UE AS, which then delivers it to the UE Application layer.

- The UE Application layer delivers the RAN visible QoE measurement report to the UE AS, which then delivers the report to the RAN. The RAN can use the corresponding reports for various types of optimizations.

To enable RAN visible QoE measurements, the RAN visible QoE measurement configuration and RAN visible QoE report need to be delivered between the UE AS and the UE Application layer separately from the XML files containing the legacy QoE measurement configuration and legacy QoE measurement report, either as a separate field in the existing AT commands or in newly defined commands.

In addition to the RAN visible QoE measurements, the ID of the PDU session used to carry the data for the QoE measurement session also needs to be reported to the RAN.

The above requires to be supported in AT commands.

2. Actions:

To 3GPP CT1:

Given that it is necessary to deliver the RAN visible QoE configuration, RAN visible QoE report between the UE AS and the UE Application layer, RAN3 respectfully asks CT1 to specify the corresponding AT commands. RAN2 will provide the detailed content of the RVQoE configuration and report, once they complete their work.

3. Date of next TSG RAN WG3 meeting:

RAN3#115-e                         February 21st  - March 3rd 2022		Online