3GPP TSG-RAN WG3 #114bis-e																	   R3-221392

17-26 Jan 2022

Online

Agenda Item:	22.2.5

Source:	ZTE, Qualcomm Incorporated, Samsung, Huawei, Lenovo, Nokia, Nokia Shanghai Bell

Title:	(TP for BL CR for 38.413) Multicast group paging

Document for:	Approval

- TP to TS38.413

----------Start of the First Change--------------

### 8.5.X	Multicast Group Paging

#### 8.5.X.1	General

The purpose of the Multicast Group Paging procedure is to enable the AMF to notify CM-IDLE UEs which have joined an MBS Session about its activation.

The procedure uses non-UE associated signalling.

#### 8.5.X.2	Successful Operation

Figure 8.5.X.2-1: Multicast Group Paging

The AMF initiates the Multicast Group Paging procedure by sending the MULTICAST GROUP PAGING message to the NG-RAN node.

At the reception of the MULTICAST GROUP PAGING message, the NG-RAN node shall perform multicast group paging of the MBS Session identified by the MBS Session ID IE utilising information provided by the AMF.

If the Paging DRX IE is included in the MULTICAST GROUP PAGING message, the NG-RAN node shall use it according to TS 38.304 [12].

If the MBS Service Area Information IE is included in the MULTICAST GROUP PAGING message, the NG-RAN node shall take it into account during multicast group paging, as specified in TS 23.247 [xx].

#### 8.5.1.3	Abnormal Conditions

Void.

*****************Next changes*******************

#### 9.2.4.X	MULTICAST GROUP PAGING

This message is sent by the AMF to trigger multicast group paging for a multicast MBS session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.x |  | YES | ignore |
| MBS Service Area Information | O |  | (FFS) |  | YES | ignore |
| Multicast Group Paging Area List | M |  |  |  | YES | ignore |
| > Multicast Group Paging Area Item |  | 1..<maxnoofPagingAreas> |  |  | - |  |
| >> Multicast Group Paging Area | M |  | 9.3.1.y |  | - |  |
| >> UE Paging List |  | 0..1 |  |  |  |  |
| >>> UE Paging Item |  | 1..<maxnoofUEsforPaging> |  |  | - |  |
| >>>> UE Identity Index Value | M |  | 9.3.3.23 |  | - |  |
| >>>> Paging DRX | O |  | 9.3.1.90 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingAreas | Maximum no. of MBS Area for multicast group paging. Value is FFS. |
| maxnoofUEsforPaging | Maximum no. of UEs for multicast group paging. Value is FFS. |

9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | FFS |  |

9.3.1.y	Multicast Group Paging Area

This IE contains a list of TAIs corresponding to the multicast group paging area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Area TAI List |  | 1..<maxnoofTAIforPaging> |  |  |
| >>TAI | M |  | 9.3.3.11 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIforPaging | Maximum no. of TAIs for multicast group paging. Value is 16. |

----------End of the Change--------------