3GPP TSG-RAN3 Meeting #114bis-e	R3-220088

Electronic meeting, 17- 26 January 2022

3GPP TSG-CT WG4 Meeting #107-e	C4-216388

# E-Meeting, 15th – 23rd November 2021

Title:	Reply LS On Source IP address clarifications

Response to:	LS On Source IP address clarifications (C4-216373 / R3-216139)

Release:	Rel-16

# Work Item:	     TEI16

Source:	CT4

To:	RAN3

cc:	CT1, SA2

Contact Person:	Frank Yong Yang

E-mail Address:	frank.yong.yang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

CT4 thanks RAN3 for the LS On Source IP address clarifications.

As specified in 3GPP TS 29.281, a GTP-U tunnel is considered as a unidirectional tunnel where the destination IP address and "the TEID in the GTP-U header is used to de-multiplex traffic incoming from remote tunnel endpoints so that it is delivered to the User plane entities in a way that allows multiplexing of different users, different packet protocols and different QoS levels".

There is no specific requirement specified for a sending GTP-U entity regarding the source IP address(es) used when sending GTP-U packets towards a destination GTP-U F-TEID. A receiving GTP-U entity should be prepared to receive GTP-U packets from different source IP addresses. Clause 4.3.0 in 3GPP TS 29.281 documents a few example scenarios where a GTP-U endpoint may receive GTP-U packets from multiple remote GTP-U endpoints; this list is not meant to be exhaustive.

Therefore, CT4 would like to provide the following response to the question:

RAN3 would like to ask CT4 whether DL forwarding traffic contained within a GTP-U DL forwarding tunnel can be transmitted by more than one source IP address.

As per the above GTP-U principles, DL forwarding traffic contained within a GTP-U DL forwarding tunnel could be transmitted by more than one source IP address. CT4 however would like to ask RAN3 to clarify what are the intended use cases for such a scenario.

2. Actions:

To RAN3

ACTION: 	CT4 kindly asks RAN3 to take the above into account and answer the question.

3. Date of Next CT4 Meetings:

3GPP TSG CT4#107e-bis		01/2022	E-Meeting

3GPP TSG CT4#108e			02/2022	E-Meeting