3GPP TSG-RAN WG3 Meeting #115	R3-221834

E-meeting, 21 Feb – 03 Mar 2022

Title: 	(TP for SON BLCR for 38.423) Mobility enhancements

Source: 	Huawei

Agenda item:	10.2.6

Document Type:	other

- Introduction

In RAN3#114b-e meeting, the CHO was discussed during Mobility Enhancement Optimization. There are some progresses and some open issues.

For CHO:

Network-based solution is needed, e.g. for the case that a RLF occurred in CHO target cell after completing handover.

Reuse the existing Handover Report Type e.g. “HO too early” or “HO to wrong cell” in HANDOVER REPORT message for CHO.

Do not introduce a new Handover Report Type e.g. “Inappropriate Configuration of a CHO candidate cell” in HANDOVER REPORT message for CHO.

Reuse the existing one UE RLF Report Container in the FAILURE INDICATION message or HANDOVER REPORT message to transfer failure related information for CHO.

which network-based solution is adopted, e.g. Option a-1/a-2/b/c, or combination of at least one of them;

whether to introduce a new initiating condition for CHO recovery procedure in FAILURE INDICATION message;

whether the FAILURE INDICATION message may be initiated without RLF report for CHO, if yes, whether to include an explicit CHO recovery cell ID in FAILURE INDICATION message and whether to include an explicit CHO recovery Cell CGI in HANDOVER REPORT message, in case of without RLF Report.

In this paper, we will discuss the CHO related open issues and give our considerations on the potential solutions.

- Discussion

2.1	Network-based solution

Network-based solution is needed, e.g. for the case that a RLF occurred in CHO target cell after completing handover.

which network-based solution is adopted, e.g. Option a-1/a-2/b/c, or combination of at least one of them;

Network-based solutions are summarized as below [1]:

Option a: Derive candidate cell list and CHO execution condition(s) based on Mobility Information.

- Option a-1: Source node transmits the Mobility Information to the target node when CHO is completed, i.e. in the SN STATUS TRANSFER message, and the target node sends the Mobility Information back to the source node via HANDOVER REPORT message. [1]

- Option a-2: Source node transmits the Mobility Information to each candidate target node in the HO request message, and the target node sends the Mobility Information back to the source node via HANDOVER REPORT message. [3]

- Option a-3: Including the Mobility Information in the UE RLF-report. RAN3 asks RAN2 to consider feasibility of adding the Mobility Information to the CHO configuration. [2]

Option b: Source node sends candidate cell list and CHO execution condition(s) to the target node after receiving Handover Success message, e.g. in SN Status Transfer message, and then the target transmits the info back to the source node in HANDOVER REPORT message. [4]

Option c: Source node stores the CHO related configuration. [6]

For RLF in legacy MRO functionality, the source node forwarded the mobility information to the target node. Upon reception of the RLF report, the target node can use the source C-RNTI and the time information provided in the RLF report from the UE to check the context. The UE context can include the mobility information and source C-RNTI. The target node sends back to the source node the UE context via HANDOVER REPORT message for the source node to perform optimization. In our understanding, solution a-2 has been supported. In case of CHO, it is possible that the mobility information may include part of the CHO related configuration because the source node can send several CHO requests before the final CHO configuration sent to UE is determined.

For RLF in source or HOF in target, the RLF report is directly forwarded to the source node in legacy MRO functionality. That is, if the source node can realize the connection failure, whether to store the HO related configuration depends on the implementation of source node. In these cases, the source node cannot get back the mobility information from the target node. To perform optimization based on the RLF report, it is possible that the source node has stored the related handover configuration. In this way, solution c has been supported.

Observation 1: Both solution a-2 and solution c are supported in legacy MRO functionality for CHO.

If the source node decides to support the CHO candidate optimisation, the retrieval of the CHO configuration related information is also possible from the failure cell (target node) to the source node if RLF happens shortly after handover. This could be done for the target node in two ways:

- using the source CRNTI that was used in the preparation of the last successful handover to identify a specific UE context, or

- using the Mobility Information IE. This used by the source node to form different types of groups, thereby reducing the need to store the full UE context.

In our understanding, the CHO configuration including the CHO candidate cell list and corresponding execution conditions is kind of UE context. It is reasonable for the source node implements to form different types of groups for CHO configurations, thereby reducing the need to store the full UE context. As stated in the above section, the source node could create different mobility information groups also taking the assigned CHO candidate cells and/or corresponding CHO execution conditions into account. With this assumption, we prefer to use the mobility information for the retrieval of the UE context after successful CHO.

Proposal 1: Solution a-1 should be optimized for CHO.

Figure 1 NW-based mobility information delivery

2.2 Enhancements in FAILURE INDICATION message

whether to introduce a new initiating condition for CHO recovery procedure in FAILURE INDICATION message;

Currently, the initiating conditions in FAILURE INDICATION message include the RRC Reestablishment initiated reporting and RRC Setup initiated reporting. In our understanding, for the RRC Reestablishment, it refers to the case that the reception node performs the RRC Reestablishment procedure with the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| CHOICE Initiating condition | M |  |  |  | YES | reject |
| >RRC Reestab |  |  |  |  |  |  |
| >>CHOICE RRC Reestab Initiated Reporting | M |  |  |  | – |  |
| >>>RRC Reestab Reporting without RLF Report |  |  |  |  |  |  |
| >>>>Failure cell PCI | M |  | 9.2.2.10 | Physical Cell Identifier | – |  |
| >>>>Re-establishment cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>>>C-RNTI | M |  | BIT STRING (SIZE (16)) | C-RNTI contained in the RRCRe-establishment Request message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | – |  |
| >>>>ShortMAC-I | M |  | BIT STRING (SIZE (16)) | ShortMAC-I contained in the RRCRe-establishment Request message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | – |  |
| >>>RRC Reestab Reporting with RLF Report |  |  |  |  |  |  |
| >>>>UE RLF Report Container | M |  | 9.2.2.59 | nr-RLF-Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10]) or RLF-Report-r9 IE contained in the UEInformationResponse message (TS 36.331 [14]) | – |  |
| >RRC Setup |  |  |  |  |  |  |
| >>CHOICE RRC Setup Initiated Reporting | M |  |  |  | – |  |
| >>>RRC Setup Reporting with RLF Report |  |  |  |  |  |  |
| >>>>UE RLF Report Container | M |  | 9.2.2.59 | nr-RLF-Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10]) or RLF-Report-r9 IE contained in the UEInformationResponse message (TS 36.331 [14]) | – |  |

For the CHO Recovery procedure, though it is part of the Reestablishment procedure as described in RAN2, the UE transmits RRCReconfigurationComplete message to the target cell instead of RRCRestablishmentRequest message. This cannot be considered as the RRC Reestablishment initiated reporting.

In addition, in Rel-16, we includes two types of initiating conditions in the FAILURE INDICATION message, covering reestablishment and setup cases. Actually, there is another case for UE to report the RLF report. For example, if the RLF report is not reported at the first cell where the UE re-enters connected state and the UE is handed over to the target cell, the UE finally reports the RLF report to the target cell. This is the RRCReconfiguration case.

Therefore, new initiating condition should be introduced to cover the RRCReconfiguration case including all HO types. For this case, the UE just sends the RLF report to the new cell. It is similar as the setup case. Consequently, we prefer to extend the second initiating condition to cover the RRCReconfiguration case.

Proposal 2: Reword existing initiating condition: RRC Setup, or reconfiguration.

Besides, in NR, RRC_INACTIVE state was introduced. Correspondingly, it was agreed for the UE to perform the RLF reporting after resumption. If possible, it is also to consider resumption as the new initiating condition.

2.3 No CHO recovery cell ID

whether the FAILURE INDICATION message may be initiated without RLF report for CHO, if yes, whether to include an explicit CHO recovery cell ID in FAILURE INDICATION message and whether to include an explicit CHO recovery Cell CGI in HANDOVER REPORT message, in case of without RLF Report.

For the reestablishment case, the UE provides the source cell information, e.g., source PCI and source C-RNTI, to the reception node. In this way, although there is no RLF report, the reception node can know the RLF occurred in the last serving cell. The reception node triggers the FAILURE INDICATION message and sets the initiating condition as RRC Reestab Reporting without RLF Report.

However, for CHO recovery success without RLF Report, the reception node receives RRCReconfigurationComplete message. There is no information in the RRCReconfigurationComplete message to indicate whether it is triggered due to CHO recovery. It is impossible for the node to differ the CHO recovery case from the normal CHO execution one. The reception node cannot trigger the FAILURE INDICATION message to the source node.

Observation 2: The node where the UE performs successful CHO recovery cannot initiate the FAILURE INDICATION message without RLF report.

In other word, only in case that the reception node receives RLF report, it can trigger the FAILURE INDICATION message to the failure node based on the contents of RLF report. Since we support delay-reporting of the RLF report. The failure node can be different from the source one. Technically speaking, the reception node has got the RLF report from the UE, it is an intuitive idea to always include the RLF report in the FAILURE INDICATION message. As a result, it is not needed to include the duplicated CHO recovery cell ID into the message.

Proposal 3: CHO recovery cell ID is not needed in FAILURE INDICATION message.

According to the description in section 9.2.3.4 in stage 2 specification [2], the CHO only applies to intra-NR RAN. And, the RLF Report for CHO will always be encoded into NR RRC format. That is, both the source node and target node can always decode the RLF Report container.

When the reception node receives RLF Report, it triggers FAILURE INDICATION message including the RLF Report. If the target node decides the source node as the right one, it sends HANDOVER REPORT message. Technically speaking, the target node will also forward the received RLF Report to the source node via HANDOVER REPORT message because the source node can decode it and get more useful information. In this way, there is no need to introduce the CHO Cell CGI in HANDOVER REPORT message.

Proposal 4: CHO Cell CGI is not needed in HANDOVER REPORT message.

- Conclusion

In this paper, we mainly discuss SON for mobility enhancement optimization, and we have the proposals:

Observation 1: Both solution a-2 and solution c are supported in legacy MRO functionality for CHO.

Observation 2: The node where the UE performs successful CHO recovery cannot initiate the FAILURE INDICATION message without RLF report.

Proposal 1: Solution a-1 should be optimized for CHO.

Proposal 2: Reword existing initiating condition: RRC Setup or reconfiguration.

Proposal 3: CHO recovery cell ID is not needed in FAILURE INDICATION message.

Proposal 4: CHO Cell CGI is not needed in HANDOVER REPORT message.

- Reference

- R3-221294, Summary of Offline Discussion on SON Enhancements for CHO

- TS 38.300

- Annex 1 Add condition - TP on TS 38.423

Start of the first change

#### 9.1.3.16	FAILURE INDICATION

This message is sent by NG-RAN node2 to indicate an RRC re-establishment attempt or a reception of an RLF Report from a UE that suffered a connection failure at NG-RAN node1.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| CHOICE Initiating condition | M |  |  |  | YES | reject |
| >RRC Reestab |  |  |  |  |  |  |
| >>CHOICE RRC Reestab Initiated Reporting | M |  |  |  | – |  |
| >>>RRC Reestab Reporting without RLF Report |  |  |  |  |  |  |
| >>>>Failure cell PCI | M |  | 9.2.2.10 | Physical Cell Identifier | – |  |
| >>>>Re-establishment cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>>>C-RNTI | M |  | BIT STRING (SIZE (16)) | C-RNTI contained in the RRCRe-establishment Request message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | – |  |
| >>>>ShortMAC-I | M |  | BIT STRING (SIZE (16)) | ShortMAC-I contained in the RRCRe-establishment Request message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | – |  |
| >>>RRC Reestab Reporting with RLF Report |  |  |  |  |  |  |
| >>>>UE RLF Report Container | M |  | 9.2.2.59 | nr-RLF-Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10]) or RLF-Report-r9 IE contained in the UEInformationResponse message (TS 36.331 [14]) | – |  |
| >RRC Setup or Reconfiguration |  |  |  |  |  |  |
| >>CHOICE RRC Setup or Reconfiguration Initiated Reporting | M |  |  |  | – |  |
| >>>RRC Setup or Reconfiguration Reporting with RLF Report |  |  |  |  |  |  |
| >>>>UE RLF Report Container | M |  | 9.2.2.59 | nr-RLF-Report-r16 IE contained in the UEInformationResponse message (TS 38.331 [10]) or RLF-Report-r9 IE contained in the UEInformationResponse message (TS 36.331 [14]) | – |  |

Start of the next change

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[…]

InitiatingCondition-FailureIndication ::= CHOICE {

rRCReestab					RRCReestab-initiated,

rRCSetuporReconfiguration					RRCSetuporReconfiguration-initiated,

choice-extension				ProtocolIE-Single-Container { {InitiatingCondition-FailureIndication-ExtIEs} }

}

InitiatingCondition-FailureIndication-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

[…]

RRCSetuporReconfiguration-initiated ::= SEQUENCE {

rRRCSetuporReconfiguration-Initiated-Reporting	RRCSetuporReconfiguration-Initiated-Reporting,

uERLFReportContainer    UERLFReportContainer			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RRCSetuporReconfiguration-initiated-ExtIEs} } OPTIONAL,

...

}

RRCSetuporReconfiguration-initiated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RRCSetuporReconfiguration-Initiated-Reporting ::= CHOICE {

rRCSetuporReconfiguration-reporting-with-UERLFReport				RRCSetuporReconfiguration-Initiated-Reporting-with-UERLFReport,

choice-extension				ProtocolIE-Single-Container { {RRCSetuporReconfiguration-Initiated-Reporting-ExtIEs} }

}

RRCSetuporReconfiguration-Initiated-Reporting-ExtIEs XNAP-PROTOCOL-IES ::= {

...

}

RRCSetuporReconfiguration-Initiated-Reporting-with-UERLFReport ::= SEQUENCE {

uERLFReportContainer	UERLFReportContainer,

iE-Extensions			ProtocolExtensionContainer { {RRCSetuporReconfiguration-Initiated-Reporting-with-UERLFReport-ExtIEs} } OPTIONAL,

...

}

RRCSetuporReconfiguration-Initiated-Reporting-with-UERLFReport-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

End of the last change

6 - Annex 2 - Mobilityinfo in SN status transfer - TP on TS 38.423

9.1.1.4	SN STATUS TRANSFER

This message is sent by the source NG-RAN node to the target NG-RAN node to transfer the uplink/downlink PDCP SN and HFN status during a handover or for dual connectivity.

Direction:	source NG-RAN node  target NG-RAN node(handover), 
NG-RAN node from which the DRB context is transferred  NG-RAN node to which the DRB context is transferred (RRC connection re-establishment or dual connectivity).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated for handover at the source NG-RAN node and for dual connectivity at the NG-RAN node from which the DRB context is transferred. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated for handover at the target NG-RAN node and for dual connectivity at the NG-RAN node to which the DRB context is transferred. | YES | reject |
| DRBs Subject To Status Transfer List | M |  | 9.2.1.14 |  | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |

9.3.4	PDU Definitions

-- **************************************************************

--

#### -- SN STATUS TRANSFER

--

-- **************************************************************

SNStatusTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{SNStatusTransfer-IEs}},

...

}

SNStatusTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-targetNG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-DRBsSubjectToStatusTransfer-List		CRITICALITY ignore		TYPE DRBsSubjectToStatusTransfer-List		PRESENCE mandatory}|

{ ID id-MobilityInformation				CRITICALITY ignore	TYPE MobilityInformation								PRESENCE optional},

...

}