3GPP TSG-RAN WG3 #115	R3-222056

Online, 21 February – 3 March, 2022

Agenda Item:	16.2.1

Source:	Ericsson

Title:	Congestion control for UE onboarding

Document for:	Discussion

# 1	Introduction

At RAN3#114bis, the following was agreed with regard to the onboarding indication, see RAN3#114bis chairman Notes [1]:

DU ultimately decides whether to set the onboarding indication in SIB1.

RAN2 agreed on the following, see RP-213095 [2]:

RAN2#114-e: [029] Toggling the 1-bit onboarding indication in SIB1 allows to control congestion due to onboarding request. [029] RAN2 confirms that onboarding does not impact the cell reselection procedure. RAN2#116-e: Cell selection (in 38304) is not affected by “on-boarding support” indicator. Suitability criteria of a SNPN cell is not affected by “on-boarding support” indicator. Assumption that NAS will anyway allow access for onboarding only if the cell/SNPN supports onboarding

In accordance with the email discussion summary on NPN access control, see R3-221056 [2], the following is still FFS:

Whether CU sends “congestion assistance information” to assist DU in the setting of the onboarding indication in SIB1. Whether DU informs CU how it has set the onboarding indication in SIB1 in a given cell so that the CU can check when UE accesses for onboarding in that cell that this accessing is aligned with the SIB1 broadcast.

In this contribution, we will discuss above open issues.

# 2		Discussion

## 2.1	Congestion assistance information from CU to DU

As indicated in R3-220853 [4], DU can derive the cell congestion level from the Network Access Rate Reduction message to decide whether to block the onboarding access in some cells. Therefore, it is not necessary to add any additional congestion assistance information.

There is no need to specify additional cell access control related information from the CU to the DU.

## 2.2	Onboarding indication from DU to CU

R3-220055 [5] proposes that the DU informs the CU about the onboarding indication broadcast in SIB1 in the F1 SETUP REQUEST message, such that the CU can verify whether the UE was allowed to access when it performs RRC connection setup including the onboarding request indicator in msg5.

In principle, the UE should not be allowed to access the cell when onboarding is disabled in the cell in accordance with SIB1 broadcast. However, once the UE has selected an SNPN which supports onboarding, the UE is no longer required to check the onboarding enabled indication when performing subsequent cell (re)selection and thus, there will be cases when the UE will attempt to access the cell even if the onboarding indication is not broadcast.

UEs may select cells which do not broadcast the onboarding indication and thus, setup an RRC connection even if the cell is congested.

Even though the gNB may as a consequence decide to release the UE with an appropriate wait timer, this would however create additional signaling overhead in the congested cell and contradicts with the intention of using the onboarding indication for congestion control. Instead of solving this issue after successful RRC connection setup, the cell access control should be addressed by RAN2 to avoid such situations.

Ask RAN2 to reconsider their agreements on congestion control for UE onboarding, see draft LS in the annex.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	UEs may select cells which do not broadcast the onboarding indication and thus, setup an RRC connection even if the cell is congested.

Based on the discussion in the previous sections we propose the following:

Proposal 1	There is no need to specify additional cell access control related information from the CU to the DU.

Proposal 2	Ask RAN2 to reconsider their agreements on congestion control for UE onboarding, see draft LS in the annex.

# 4	References

- RAN3#114bis chairman Notes, " Agenda”, RAN3 chairman, 2022-01-27

- RP-213095, “Status Report to TSG”, China Telecom, 2021-12

- R3-212685, “Summary of Offline Discussion – NPN Cell Access Control”, Nokia, Nokia Shanghai Bell – Moderator, RAN3#114bis, 2022-01

- R3-220853, “Left issues on ehanced NPN”, ZTE, RAN3#114bis, 2022-01

- R3-220655, “(TP for NG_RAN_PRN_enh BLCR for TS 38.300) Supporting enhanced private network”, Huawei, RAN3#114bis, 2022-01

# Annex:	Draft LS

Title:	LS on congestion control for UE onboarding

Release:	Release 17

Work Items:	NG_RAN_PRN_enh

Source:	Ericsson [To be RAN3]

To:	RAN WG2

CC:

Contact Person:

Name:		Alexander Vesely

E-mail Address:	alexander dot vesely at ericsson dot com

Attachments:	None

1. Overall Description:

RAN3 was discussing whether the functions in the gNB are able to rely on the onboarding request indication provided by the UE in msg5 which should imply that the cell has enabled onboarding and was wondering whether, following latest RAN2 agreements, the UE would act in accordance with the original intended cell congestion control mechanism for onboarding.

While network solutions could be specified, RAN3 believes that this issue should be rather solved by reconsidering latest RAN2 agreements and prefers RAN2 to address potential congestion in the cell.

2. Actions:

To RAN2: 		RAN3 kindly asks RAN2 to reconsider their agreements on congestion control for UE onboarding.