3GPP TSG-RAN WG3 Meeting #115-e	R3-222189

21 Feb – 3 Mar 2022

# Online

Agenda Item:	10.3.2.1

Source:	ZTE

Title:	(TP for MDT BL CR for TS38.413) Introduce PDCP Excess Packet Delay

Document for:	Discussion and agreement

TP for TS38.413

Start of Change

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, …) |  |  |  |
| M6 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  |  |  |
| M6 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity,…) | Number of reports | Yes | Ignore |
| M6 Delay Threshold | O |  | (FFS)ENUMERATED (ms0.25,ms0.5,ms1,ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500, ms750, …) |  | Yes | Ignore |

Next Change

9.4.5	Information Element Definitions

--unchanged part

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-M6ReportAmount		CRITICALITY ignore	EXTENSION ReportAmountMDT		PRESENCE optional		}

{ ID id-m6delay-threshold	CRITICALITY ignore	EXTENSION M6delay-threshold		PRESENCE optional		}

...

}

M6delay-threshold ::= ENUMERATED { ms0.25,ms0.5,ms1,ms30, ms40, ms50, ms60, ms70, ms80, ms90, ms100, ms150, ms300, ms500, ms750, ...}

M6report-Interval ::= ENUMERATED {

ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30,

...

}

Next Change

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-m6delay-threshold         															                     ProtocolIE-ID ::= 2xx

END

-- ASN1STOP

End of Change