3GPP TSG-RAN WG3 Meeting #115-e	R3-222237

E-meeting, 21 Feb – 3 Mar 2022

Title: 	End markers from 5GC during handover

Source: 	Huawei

Agenda item:	9.3.5.1

Document Type:	Discussion

1. Introduction

At previous RAN3#114bis meeting, it was discussed the end marker transmission for the 5GS intra-system handover based on [1].  In this document, we provide our views on this issue.

2. Discussion

The discussion is encompassed about the following highlighted sentence whether the GTP-U end marker packets per PDU session are transmitted together with the QFI or not. There were some discussions at previous meeting to minute in the chairman notes that this is not allowed.

<Excerpted from section 9.2.3.2.3 Data Forwarding of TS 38.300> Handling of end marker packets: - The source NG-RAN node receives one or several GTP-U end marker packets per PDU session from the UPF and replicates the end marker packets into each data forwarding tunnel when no more user data packets are to be forwarded over that tunnel. - End marker packets sent via a data forwarding tunnel are applicable to all QoS flows forwarded via that tunnel. After end marker packets have been received over a forwarding tunnel, the target NG-RAN node can start taking into account the packets of QoS flows associated with that forwarding tunnel received at the target NG-RAN node from the NG-U PDU session tunnel.

We think that for intra-5GS handover, there is no restrictions on whether the GTP end markers are transmitted together with the QFI or not, as long as it is pretty clear that the GTP-U end marker packets are per PDU session.

In addition, it is already allowed to embed the QFI into the end marker for the 5GC to EPC inter-system handover as follows.

<Excerpted from section 9.3.2.5 Data Forwarding for the User Plane of TS 38.300> - Handling of end marker packets: - The source NG-RAN node receives one or several end marker packets per PDU session from the UPF. When there are no more data packets to be forwarded for QoS flows mapped to an E-RAB, the source NG-RAN node sends one or several end markers including one QFI (by means of the PDU Session User Plane protocol TS 38.415 [30]) of those QoS flows mapped to that E-RAB and sends the end marker packets to the UPF over the PDU session tunnel. From the included QFI in the end markers and its mapping to an EPS bearer ID, the UPF knows which EPS bearer tunnel it needs to forward the end-markers to the SGW. The QFI is removed in the end marker packets sent to the SGW.

Then technically speaking, for intra-system handover, it is also allowed to carry the QFI in the end markers, which is already supported in TS 38.415. Hence there is no need to restrict the scenario. On the other hand, if it is to be restricted, this may have negative impact on the on-site and real deployments, and also lead to more IoT test at this late stage.

- No restrictions on whether the GTP-U end markers from the 5GC are embedded with the QFI or not during handover.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- No restrictions on whether the GTP-U end markers from the 5GC are embedded with the QFI or not during handover.

4. Reference

- R3-220754 Clarification for handling of end marker packets_R15 (ZTE)