3GPP TSG-RAN WG3 #117-e	R3-224996

Online, 15th - 24th August 2022

Agenda Item:	9.2.7

Source:	Ericsson (moderator)

Title:	CB: # 21_R17MBS_XnF1E1 - Summary of email discussion

Document for:	Discussion

# Introduction

This document summarizes the offline discussions on Rel-17 MBS Corrections, guided by the chair’s summary below

CB: # 21_R17MBS_XnF1E1

- Incomplete multicast call flow and other CRs to TS 38.401

- Definition of MBS related contexts on NG-RAN interfaces

- MRB Type (with bearer type change) and RLC Mode on F1AP

- MRB ID range on NG-RAN interfaces, MRB ID change

- F1-U Tunnel establishment for the per UE data over the ptp leg (e.g., triggering, CU/DU roles)

- Update to F1-U Context Descriptor

- Data Forwarding between MBS supporting nodes (e.g., at MBS session level)

- Mobility between Supporting node and non-supporting node

- Broadcast service and Unicast service co-existence

- Misc (e.g., Address management for shared CU UP, codepoints update, F1/E1AP ID, IE namings on HFN/SN for PDCP initialisation, PDCP count "wrap around", MBS specific cause values on network interfaces)

- Capture agreements and provide CRs if agreeable, split the work

(E/// - moderator)

Summary of offline disc R3-224996

# For the Chairman’s Notes

[to be added]

# Discussion first round

## General alignment of definitions, missing definitions, etc.

This topic is dealt with in R3-224467 [17] - R3-224473 [22], including CR for 38.401 [18], CR for XnAP [19], CR for F1AP [20], CR for E1AP [21], CR for 38.472 [22], CR for 37.482 [23].

Q1: Given the fact that the content of those documents were submitted to all companies participating in MBS discussions in the past begin of July, the moderator suggests to collect further comments but to assume that these changes are in general agreeable. Please provide your comments below.

| Company | Comment |
| --- | --- |

## Control of F1-U ptp retransmission tunnel resources

This topic is dealt with in R3-224475 [24] and R3-224476 [25].

Q2:The moderator proposes to take

(1) agree on CR for F1AP in R3-224476 [25]

(2) agree on the following parts of R3-224407 [8], CR for 38.401: section 6.1.5 and ptp retransmission parts of section 6.4. Please provide your comments below

| Company | Comment |
| --- | --- |

## Control of F1-U “catch up” tunnel resources, i.e. for ptp F1-U tunnels that provide data forwarded on Xn-U

R3-224327 [1], R3-224328 [2] and R3-224329 [3] propose to allow establishment of ptp F1-U tunnels to deliver PDCP PDUs to the DU as received via the Xn-U forwarding tunnel. Different than the re-structuring of the F1-U Context Descriptor in R3-224471 [20] and R3-224472 [21] it is proposed to keep an explicit UE reference.

Q3:The moderator suggest to

(1) first comment on the overall concept, scenario and requirement of this approach

(2) comment to an approach, suggested by the moderator, given (1) can be answered positively, where the F1-U Context descriptor, upon request from the CU, follows approach in in R3-224471 [20] and R3-224472 [21] and includes yet another “discriminator” for this new kind of tunnel e.g. “ptp forwarding tunnel” (with the semantics that no PDCP Status Report is configured) and including the first PDCP SN to be sent on that tunnel and by that allowing a “direct” communication between the DU and the CU-UP) w/o the necessity to “intercept” the F1-U MC Distribution Setup procedure at the CU by retrieving UE context data.

and kindly asks to provide comments below.

| Company | Comment |
| --- | --- |

## Introducing support of MBS Session level Xn-U data forwarding

Papers R3-224449 [15] (discussion) and R3-224450 [16] (XnAP CR) propose to introduce MBS Session level Xn-U data forwarding.

Q4: The moderator kindly asks to provide below

(1) general comments on the proposal

(2) comments on the CR, if applicable/if any

| Company | Comment |
| --- | --- |

## Control of Xn-U forwarding tunnel resources for MC MBS traffic in E1AP

E1AP CRs in R3-224860 [39], R3-224329 [3] and R3-224648 [30] introduce missing E1AP signaling to configure Xn-U forwarding resources towards supporting and non-supporting gNBs (which requires QFI mapping information).

Q5.1: The moderator proposes to follow principles outlined in R3-224860 [30} and R3-224329 [3], which rather configures the MC MBS Session Context than a UE Session Context. The moderator suggests to take [R3-224860 [40] as baseline (which include the forwarding and mapping information into the existing “Setup/Modify” List IEs. Please provide your comments below:

| Company | Comment |
| --- | --- |

Q5.2: The moderator also proposes to discuss renaming the MBS Initial HFN and Reference PDCP SN IE (E1AP, see [30]) or alternatively modifying the IE semantics bearing in mind, that the value carried in this IE may be only an estimate. Corresponding proposal for XnAP in [40] to be included in the discussion.

| Company | Comment |
| --- | --- |

Q5.3: The moderator also proposes to discuss changes proposed in [30] regarding PDCP SN wrap around. Please provide your comments:

| Company | Comment |
| --- | --- |

Q5.4: And finally, sorry for that, the moderator proposes to discuss proposals regarding provision of mapping information to support duplication avoidance, as proposed in [30]. Please provide your comments:

| Company | Comment |
| --- | --- |

## Re-structuring TS 38.401 message flow for multicast

There are a couple of attempts to either correct the current version of the mc message flow in TS 38.401 in its current shape as in R3-224334 [6] or to disentangle the flow and split it up into different stages/scenarios as shown in R3-224477 [26].

Q6: The moderator suggests to

(1) follow the approach in R3-224477 [26] and discuss it

(2) then discuss whether and how to introduce further details as suggested in R3-224334 [6] and R3-224933 [42] and R3-224943/5040 [44].

and kindly asks to provide comments below:

| Company | Comment |
| --- | --- |

## Support of MRB ID Change on E1/F1

R3-224672 [35] (discussion), R3-224673 [36] (CR F1AP) and R3-224674 [37] (CR E1AP).

Q7: (1) The moderator understands that MRB ID change in case of inter-DU mobility is covered, while these papers propose to support MRB ID Change in case of intra-DU mobility. If this is confirmed, the moderator wonders whether this is necessary, as the scope of the MRB ID was defined to be the DU (an F1 interface instance) and we could assume that the MRB ID allocation does not need to be changed.

(2) The E1AP CR also proposes to change the semantics of the E1AP maxnoofMRBs.

The moderator kindly asks to please provide comments below.

| Company | Comment |
| --- | --- |

## Corrections for shared NG-U termination

Q8.1: Documents R3-224442 [10] - R3-224444 [12] propose to correct the handling of shared NG-U terminations on E1 and NG to explicate the case where F1/NG Distribution Setup procedure instances refer to already established NG-U terminations. Please provide your comments below,

| Company | Comment |
| --- | --- |

Q8.2: R3-224447 [13] and R3-224448 [14] suggest introducing a third codepoint to allow a combination of the first two codepoints in one step. Please provide your comments below.

| Company | Comment |
| --- | --- |

## Corrections for MC MBS session admission control

R3-224942 [43] and R3-224943 [44] deals with admission control in general.

Q9: The moderator would like to take these papers to highlight that the RAN3 decision that admission control [for MRB radio resources] takes place at multicast session activation is not captured anywhere in stage 2/3. The moderator proposes to task the authors of [43] and [44] to capture that agreement in an appropriate place in 38.401 (as a first proposal). Please provide your comments below.

| Company | Comment |
| --- | --- |

## Correction to Group Paging in F1

Q10: R3-224669 (discussion paper in the other CB) and F1AP CR in R3-224671 [34] propose to specify the DU action in case the UE Identity List for Paging IE is not included. Please provide your comments below:

| Company | Comment |
| --- | --- |

## Introducing MBS cause values in E1AP and F1AP

Q11: Documents R3-224665 [31] - R3-224668 [32] propose including MBS specific cause values. Please provide your comments below:

| Company | Comment |
| --- | --- |

## DU and CU role for deciding the MRB bearer type applied in the UE RRC configuration

The moderator saw that there are a couple of papers (R3-224332 [4], R3-224406 [7], ff) that contain proposals that the CU ultimately determines the MRB bearer type, which on the one hand side may come from confusing the fact that the CU terminates the RRC protocol, while the DU provides, per Rel-15 agreement the lower layer configuration which is transparently incorporated in the final RRC Reconfiguration message, and partly from the fact that the CU would request the DU to setup e.g. ptp retransmission resources, however, the DU still remains in charge of determining the MRB bearer type, as it denotes lower layer configuration.

Q12: The moderator suggests to close the discussion with the above understanding and tasks the author of R3-224407 [8] to provide a resulting CR for TS 38.401 capturing that understanding in chapter 7.7.1 or any other suitable place. Please provide your comments below

| Company | Comment |
| --- | --- |

## Broadcast and Unicast Co-existence correction for F1AP

Q13: Please provide comments to R3-224333 [5], the F1AP CR.

| Company | Comment |
| --- | --- |

## E1AP ASN.1 Correction for MCBearerContextToModify

Q14: Please provide comments to R3-224644 [27].

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

R3-224327	Consideration on Multicast Data Forwarding and F1-U Tunnel aspects (Huawei, CBN, China Unicom, Nokia, Nokia Shanghai Bell), discussion

R3-224328	Multicast Data Forwarding and F1-U tunnel aspects (Huawei, CBN, China Unicom, Nokia, Nokia Shanghai Bell), CR0977r, TS 38.473 v17.1.0, Rel-17, Cat. F

R3-224329	Multicast Data Forwarding and F1-U tunnel aspects (Huawei, CBN, China Unicom, Nokia, Nokia Shanghai Bell),CR0028r, TS 37.483 v17.1.0, Rel-17, Cat. F

R3-224332	Other leftover issues on F1 and E1 for MBS (Huawei, CBN, Qualcomm Incorporated, Lenovo), discussion

R3-224333	Correction on Broadcast and Unicast co-existence (Huawei, CBN, Qualcomm Incorporated, CATT, Lenovo), CR0978r, TS 38.473 v17.1.0, Rel-17, Cat. F

R3-224334	Correction on Multicast Session Establishment (Huawei, CBN, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, CATT, Lenovo), CR0240r, TS 38.401 v17.1.1, Rel-17, Cat. F

R3-224406	F1-U tunnels related issues (Lenovo), discussion

R3-224407	Correction on F1-U tunnels for multicast MRB (Lenovo), CR0243r, TS 38.401 v17.1.0, Rel-17, Cat. F

R3-224408	Correction on F1-U tunnels for multicast MRB (Lenovo), CR0983r, TS 38.473 v17.1.0, Rel-17, Cat. F

R3-224442	Correction of address management for shared CU UP (Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Orange), discussion

R3-224443	Correction of address management for shared CU UP (Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Orange), CR0862r, TS 38.413 v17.1.1, Rel-17, Cat. F

R3-224444	Correction of address management for shared CU UP (Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Orange), CR0029r, TS 37.483 v17.1.0, Rel-17, Cat. F

R3-224447	Correction of shared CU UP codepoints (Nokia, Nokia Shanghai Bell, Ericsson, Orange), discussion

R3-224448	Correction of shared CU UP codepoints (Nokia, Nokia Shanghai Bell, Ericsson, Orange), CR0030r, TS 37.483 v17.1.0, Rel-17, Cat. F

R3-224449	Correction of MBS data forwarding (Nokia, Nokia Shanghai Bell, Huawei, Orange), discussion

R3-224450	Correction of MBS data forwarding (Nokia, Nokia Shanghai Bell, Huawei, Orange), CR0863r, TS 38.423 v17.1.0, Rel-17, Cat. F

R3-224467	General open issues - discussion (Ericsson, Qualcomm, Verizon Wireless, AT&T, China Unicom), discussion

R3-224468	Further Corrections for NR MBS (Ericsson, Qualcomm, Verizon Wireless, AT&T, China Unicom), CR0244r, TS 38.401 v17.1.1, Rel-17, Cat. F

R3-224470	Further Corrections for NR MBS (Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm, Verizon Wireless, AT&T, China Unicom), CR0867r, TS 38.423 v17.1.0, Rel-17, Cat. F

R3-224471	Further Corrections for NR MBS (Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm, Verizon Wireless, AT&T, China Unicom), CR0984r, TS 38.473 v17.1.0, Rel-17, Cat. F

R3-224472	Further Corrections for NR MBS (Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm, Verizon Wireless, AT&T, China Unicom), CR0031r, TS 37.483 v17.1.0, Rel-17, Cat. F

R3-224473	Corrections for MBS-associated signalling (Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm, CATT, Verizon Wireless, AT&T, China Unicom), CR0022r, TS 38.472 v17.0.0, Rel-17, Cat. F

R3-224474	Corrections for MBS-associated signalling (Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm, CATT, Verizon Wireless, AT&T, China Unicom), CR0002r, TS 37.482 v17.1.0, Rel-17, Cat. F

R3-224475	Corrections for ptp retransmission topics and overall example message flow restructuring (Ericsson, Verizon Wireless, AT&T, China Unicom), discussion

R3-224476	Corrections for the establishment of F1-U ptp retransmission tunnels (Ericsson, Verizon Wireless, AT&T, China Unicom), CR0985r, TS 38.473 v17.1.0, Rel-17, Cat. F

R3-224477	Corrections to the example message flow for multicast MBS Context establishmen (Ericsson, Verizon Wireless, AT&T, China Unicom), CR0245r, TS 38.401 v17.1.1, Rel-17, Cat. F

R3-224644	E1AP ASN.1 correction on MCBearerContextToModify (CATT,Nokia, Nokia Shanghai Bell,Huawei,ZTE,Ericsson,Samsung,Lenovo), CR0032r, TS 37.483 v17.1.0, Rel-17, Cat. F

R3-224645	Introduction of ongoing broadcast service in XnAP (CATT,Nokia, Nokia Shanghai Bell), CR0878r, TS 38.423 v17.1.0, Rel-17, Cat. F

R3-224647	Discussion on three issues on E1AP (CATT), discussion

R3-224648	Correction on three issues on TS 37.483 (CATT), CR0033r, TS 37.483 v17.1.0, Rel-17, Cat. F

R3-224665	Introduction of MBS specific cause values (Huawei, CBN, Qualcomm Incorporated), discussion

R3-224667	Introduction of MBS specific cause values (Huawei, CBN, Qualcomm Incorporated), CR1000r, TS 38.473 v17.1.0, Rel-17, Cat. F

R3-224668	Introduction of MBS specific cause values (Huawei, CBN, Qualcomm Incorporated), CR0034r, TS 37.483 v17.1.0, Rel-17, Cat. F

R3-224671	Correction on Multicast Group Paging (Huawei, CBN, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Lenovo), CR1001r, TS 38.473 v17.1.0, Rel-17, Cat. F

R3-224672	Multicast MRB ID change over F1 and E1 interfaces (Huawei, CBN, Qualcomm Incorporated, Lenovo), discussion

R3-224673	Correction on MRB ID Change (Huawei, CBN, Qualcomm Incorporated, Lenovo), CR1002r, TS 38.473 v17.1.0, Rel-17, Cat. F

R3-224674	Correction on MRB ID Change (Huawei, CBN, Qualcomm Incorporated, Lenovo), CR0035r, TS 37.483 v17.1.0, Rel-17, Cat. F

R3-224859	Discussion for the MBS open issues (Samsung), discussion

R3-224860	Correction for the MBS multicast data forwarding (Samsung), CR0037r, TS 37.483 v17.1.0, Rel-17, Cat. F

R3-224861	Correction for the MRB progress information (Samsung), CR0892r, TS 38.423 v17.1.0, Rel-17, Cat. F

R3-224919	Discussion on Multicast MBS Session Context Establishment (Google Inc.), discussion

R3-224933	Clarification to Multicast MBS Session Context Establishment (Google Inc.), CR0254r, TS 38.401 v17.1.1, Rel-17, Cat. F

R3-224942	Issues found in Rel-17 NR MBS with discussions and draft CR to F1AP (ZTE), other

R3-224943 (will be revised in R3-225040)	Correction to 38.401 on admission control of multicast session for NR MBS (ZTE, CMCC, Lenovo), CR0256r, TS 38.401 v17.1.1, Rel-17, Cat. F