3GPP TSG-RAN WG3 Meeting #117bis-e	R3-225354

Online, 10th – 18th October 2022

Agenda Item:	16.2

Source:	Ericsson

Title:	Authorization for U2U relay

Document for:	Discussion, Decision

# 1	Introduction

In RAN3#117-e meeting, one of the open issues for Relay and Remote UE authorization is below.

Whether the 5G ProSe Authorized IE could be extended to include the U2U relay authorization.

SA2 sent an LS in [1] to ask for RAN WGs’ opinion. In this contribution, we would provide the analysis.

# 2	Discussion

SA2 has discussed the ProSe authorization information for UE-to-UE Relay sent from AMF to NG-RAN node in Rel-18. SA2 is seeking for RAN WGs’ inputs whether such information is needed or not.

One or more of the followings might be included for UE-to-UE Relay operation.

1)	whether the UE is authorized to act as a 5G ProSe Layer-2 UE-to-UE Relay;

2)	whether the UE is authorized to act as a 5G ProSe Layer-3 UE-to-UE Relay;

3)	whether the UE is authorized to act as a 5G ProSe Layer-2 U2U UE;

4)	whether the UE is authorized to act as a 5G ProSe Layer-3 U2U UE.

SA2 Question 1: Whether the "5G ProSe authorised" information needs to be enhanced to include the authorization information for UE-to-UE Relay operation?

SA2 Question 2: If the answer to Q1 is yes, which bullet(s) need to be included?

To answer Q1, first we should clarify under which scenario the network needs to have control of U2U relay UE. As captured in TR 38.836, the coverage scenarios have been discussed and supported. The same applies for U2N relaying, which means out of coverage, in coverage, and etc.

In case of UE-to-UE relay, the data traffic is sent directly between the Source UE and Destination UE, i.e., the gNB is not involved. By changing this basic principle, that means dedicated signalings between the network and U2U relay UE needs to be supported, though this is not the intention when introducing U2U relay. Furthermore it is sufficient to leave for a UE decision to determine whether the UE or its peer UE to be authorized for U2U relay. The UE may base on provisioned information to autonomously make the decision.

The answer to Q1 is No, i.e., the authorization information for U2U relay operation does not have be included as part of 5G ProSe authorization.

Q1 should be evaluated in RAN2 at the same time. Q2 is more for RAN3 on the signaling impacts, i.e., NGAP, XnAP, and F1AP. Based on the proposed answer to Q1, we don’t think RAN3 needs to add the corresponding authorization info.

Reply to SA2 that RAN3 does not expect any change in multiple specifications to support network controlled U2U relay operation.

# 3 Conclusion

In this paper we propose:

Proposal 1	The answer to Q1 is No, i.e., the authorization information for U2U relay operation does not have be included as part of 5G ProSe authorization.

Proposal 2	Reply to SA2 that RAN3 does not expect any change in multiple specifications to support network controlled U2U relay operation.

# 4	References

R3-225322, LS on ProSe Authorization information related to UE-to-UE Relay operation to NG-RAN, SA2