3GPP TSG-RAN WG3 Meeting #117-bis-e	R3-225564

# Online, 10 - 18 October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0889 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of inter-system handover to HeNB (alternative solution) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, AT&T, Verizon Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2022-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 5G to 4G inter-system handover is supported since Rel-15. However, the Target ID IE in the HANDOVER REQUEST message is erroneously encoded as Global ng-eNB ID rather than Global eNB ID. The difference between the two is as follows: - eNB ID (in E-UTRAN) supports four different ID lengths: 20-bit macro, 18-bit short macro, 21-bit long macro, and 28-bit home eNB ID lengths. - ng-eNB ID (in NG-RAN) supports three different ID lengths: the same as eNB ID except the 28-bit home eNB ID (since there is no HeNB in the NG-RAN architecture). The consequence of this error is that 5G to 4G handover to a target HeNB is unintentionally precluded by stage 3 specifications, even though it is supported since Rel-15 by stage 2 specifications as confirmed by LS from SA2 in R3-225323. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Home eNB ID is introduced as a new choice in the Global ng-eNB ID IE, in order to fill the gap caused by the erroneous encoding of the E-UTRAN choice in the Target ID IE. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol point of view. The impact can be considered isolated because the change affects only inter-system handover to HeNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G to 4G handover is not possible to a HeNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.8, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modification

#### 9.3.1.8	Global ng-eNB ID

This IE is used to globally identify an ng-eNB (see TS 38.300 [8]). This IE is also used to globally identify an eNB (see TS 36.300 [17]) in case of handover to E-UTRAN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| CHOICE ng-eNB ID | M |  |  |  | - |  |
| >Macro ng-eNB ID |  |  |  |  |  |  |
| >>Macro ng-eNB ID | M |  | BIT STRING (SIZE(20)) | Equal to the 20 leftmost bits of the E-UTRA Cell Identity IE contained in the E-UTRA CGI IE of each cell served by the ng-eNB. | - |  |
| >Short Macro ng-eNB ID |  |  |  |  |  |  |
| >>Short Macro ng-eNB ID | M |  | BIT STRING (SIZE(18)) | Equal to the 18 leftmost bits of the E-UTRA Cell Identity IE contained in the E-UTRA CGI IE of each cell served by the ng-eNB. | - |  |
| >Long Macro ng-eNB ID |  |  |  |  |  |  |
| >>Long Macro ng-eNB ID | M |  | BIT STRING (SIZE(21)) | Equal to the 21 leftmost bits of the E-UTRA Cell Identity IE contained in the E-UTRA CGI IE of each cell served by the ng-eNB. | - |  |
| >Home eNB ID |  |  |  |  | YES | reject |
| >>Home eNB ID | M |  | BIT STRING (SIZE(28)) | Equal to the Cell Identity IE contained in the E-UTRAN CGI IE (see TS 36. 423 [40] subclause 9.3.1.9) of the cell served by the eNB. Applicable for handover to E-UTRAN. | - |  |

Next Modification

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

** Unchanged text skipped **

id-GUAMIType,

id-HomeENB-ID,

id-IncludeBeamMeasurementsIndication,

** Unchanged text skipped **

NgENB-ID ::= CHOICE {

macroNgENB-ID			BIT STRING (SIZE(20)),

shortMacroNgENB-ID		BIT STRING (SIZE(18)),

longMacroNgENB-ID		BIT STRING (SIZE(21)),

choice-Extensions		ProtocolIE-SingleContainer { {NgENB-ID-ExtIEs} }

}

NgENB-ID-ExtIEs NGAP-PROTOCOL-IES ::= {

{ID id-HomeENB-ID		CRITICALITY reject	TYPE BIT STRING (SIZE(28))	PRESENCE mandatory },

...

}

Last Modification

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

** Unchanged text skipped **

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HomeENB-ID											ProtocolIE-ID ::= xxx

END