3GPP TSG-RAN3 #117bis-e	R3-225615

# 10 - 18 Oct 2022, online

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IoT NTN about constructing the Mapped Cell ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson, ZTE, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2022-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN3#116e, we get a common understanding in NR NTN as below, and this common understanding should also be applied to IoT NTN. It’s common understanding in RAN3 that gNB is always responsible for constructing the Mapped Cell ID regardless of whether the UE location is not available or not as specified in Section 16.14.5. Therefore, eNB can always construct the Mapped Cell ID for a NB-IoT UE, no matter NB-IoT UE using CP CIoT EPS optimization only or UP CIoT EPS optimization. In current text, "if available" already covers all cases, including the various UE types. Therefore, listing UE types is unnecessary and potentially confusing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the limitation that eNB can construct the Mapped Cell ID only for specfic kinds of UE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact under functional point of view, since it only removes the inaccurate limitation for eNB to construct the Mapped Cell ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current limitation regarding NB-IoT UE that supports S1-U data transfer or User Plane CIoT EPS optimisation is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 23.21.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<< START OF CHANGE >>>>>>

### 23.21.6	Signalling

The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported in the following cases:

-	The Cell Identity indicated by the eNB to the Core Network as part of the User Location Information, or as E-UTRAN CGI in the related S1AP messages;

-	The Cell Identity used for Paging Optimization in S1 interface;

-	The Cell Identity used for PWS.

For a BL UE or a UE in enhanced coverage, the Cell Identity included within the target identification of the handover messages allows identifying the correct target cell.

The mapping between Mapped Cell ID(s) and geographical area(s) is configured in the RAN and Core Network.

NOTE 1:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate different geographical areas (e.g. overlapping and/or with different dimensions).

The eNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., depending on operator's policy) or up to implementation.

NOTE 2:	As described in TS 23.401 [17], the User Location Information may enable the MME to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The eNB reports the broadcasted TAC(s) of the selected PLMN to the MME. In case the eNB knows the UE's location information, the eNB may determine the TAI the UE is currently located in and provide that TAI to the MME.

<<<<<< END OF CHANGE >>>>>>