3GPP TSG-RAN WG3 Meeting #117e-bis	R3-225754

E-meeting, 10 – 18 Oct, 2022

Agenda item:	13.4

Source:	Xiaomi

Title:	Discussion on mitigation of interference

Document for:	Discussion and Decision

1	Introduction

In this contribution, we’d like to discuss the mobility enhancement based on the last RAN3 agreements as below

- PCI space partitioning via OAM configuration can be used in some cases for avoidance of PCI collisions.

- From RAN3 perspective, existing mechanism can be used for PCI collision detection in mobile IAB scenario. Further enhancement is FFS.

- RAN3 to discuss whether mobile IAB needs any enhancements to the existing mechanisms for PCI collision avoidance and/or optimization.

- From RAN3 perspective, no enhancements are needed for RACH collision avoidance unless requested by other WGs.

2	Discussion

2.2	PCI Optimization in mobile IAB

According to the agreement, RAN3 should discuss the enhancement for mobile IAB based on the existing PCI poetization function.

- From RAN3 perspective, existing mechanism can be used for PCI collision detection in mobile IAB scenario. Further enhancement is FFS.

- RAN3 to discuss whether mobile IAB needs any enhancements to the existing mechanisms for PCI collision avoidance and/or optimization.

7.8	PCI Optimisation Function

The PCI Optimization Function in non-split gNB case is specified in TS 38.300 [2].

In split gNB architecture, the OAM configures a PCI for each NR cell to the gNB-DU.

For centralized PCI assignment in split gNB architecture, the gNB-CU detects PCI conflict of NR cells and reports the NR cells suffering PCI conflict to OAM directly. The OAM is in charge of reassigning a new PCI for the NR cell subject to PCI conflict.

For distributed PCI assignment in split gNB architecture, the OAM assigns a list of PCIs for each NR cell and sends the configured PCI list to the gNB-CU. If the gNB-CU detects PCI conflict, the gNB-CU may select a new PCI value from the preconfigured PCI list for the NR cell and send it to the gNB-DU by either F1 Setup procedure or gNB-CU configuration update procedure.

Existing PCI optimization function in TS 38.401 is shown below

If the IAB-donor and IAB-node have the same OAM, existing mechanism is sufficient, while if the IAB-donor and IAB-node have different OAMs, e.g. in roaming scenario, existing mechanism is not sufficient,

In case of the IAB-donor and IAB-node with different OAMs,

- For centralized PCI assignment, IAB-donor CU detects PCI conflict of NR cells, IAB-donor should notify the IAB-node, and then the IAB-node that have connection with OAM can reports the PCI conflict to OAM. After that, the OAM can reassign new PCI for the cells in the IAB-node to avoid PCI conflict.

- For distributed PCI assignment, OAM assigns a list of PCIs for the cells and send the configured PCI list to the IAB-node, if the IAB-donor detects PCI conflict, either IAB-donor or IAB-node can be responsible for selecting a new PCI value from the preconfigured PCI list.

Observation 1, according to the existing mechanism, IAB-donor can detect the PCI conflict, while IAB-donor may not be aware of the cell configuration or pre-configurations for the IAB-node via OAM, since IAB-donor may connect to different OAM from IAB-node.

Proposal 1, RAN3 discuss whether the IAB-donor need to notify the detected PCI conflict info to IAB-node, so that the IAB-node can select a proper PCI in the reconfigured PCI list or notify the OAM to reassign PCI to avoid the conflict.

Proposal 2, RAN3 discuss whether the IAB-node should notify the preconfigured PCI list via OAM to IAB-donor, so that the IAB-donor can select a proper PCI for the IAB-node considering the detected PCI conflict.

2.1	PCI space partitioning

In last RAN3 meeting, PCI space partitioning had been discussed for the avoidance of PCI collisions and RAN3 agreed that it can be used in some cases. In our understanding, if mobile IABs are widely deployed in the future, we don’t think the PCI space partitioning via OAM is enough to avoid PCI collisions.

Even through there’re 1008 PCI values, some physical layer signals (SS, DMRS & SRS) are designed based on PCI, avoid interference should also consider PCI Mod3 (for PSS) interference, PCI Mod4 (for DMRS) interference and PCI Mod30 (for DMRS/SRS) interference, PCI partitioning via cannot satisfy the requirements for mobile IAB deployment.

On the other hand, the PCI collisions can still happen even with known trajectory of the IAB node in advance, it can still be impacted by the mobile IABs-nodes, it may have interference for the IAB-node without known trajectory

Observation 2, PCI space partitioning via OAM cannot avoid the PCI collisions considering the interference cannot be predicted in advance when mobile IAB-nodes are widely deployed.

In our view, the solution to avoid the PCI collision in real-time can be dynamic PCI change or frequency change, considering the PCI mod3 is hard to be avoided.

Proposal 3, both dynamic PCI change and frequency change can be considered for the avoidance of PCI collision in mobile IAB scenario.

2.2	Assistance info for the avoidance of PCI collision

In our view, exchange some assistance info in advance may be useful for the avoidance of PCI collision and the following information should be considered.

- The pre-configurations in the mobile IAB-node, e.g. the pre-configured frequency and list of PCI.

- The PCI configuration of the neighbours, knowing the neighbour information can be helpful to configure the new PCI resource to avoid interference, since the interferences are from the neighbour.

- The PCI conflict detection info, it would be useful to consider the detected info, so that the new configuration can avoid the collision.

- IAB-node movement info, since IAB node is mobile, it would be useful if known the location or trajectory of the IAB node, so that the new configuration can avoid the interference in the road of a mobile IAB-node.

Proposal 4, RAN3 consider the following information for PCI/frequency reconfiguration for the avoidance of PCI collisions,

- The pre-configurations in the mobile IAB-node

- The PCI configuration of the neighbours

- The PCI conflict detection info

- IAB-node movement info

2.3	RACH resource configuration conflict

- From RAN3 perspective, no enhancements are needed for RACH collision avoidance unless requested by other WGs.

RAN3 agreed as above there is no enhancement for RACH collision during UE migration, while for the RACH configuration conflict, it face the same issue as PCI conflict and can be treated in the same way.

Proposal 5, the enhancement for PCI conflict can also be used for RACH resource configuration conflict.

3	Conclusion

In this contribution, we had the following observations and proposals:

Observation 1, according to the existing mechanism, IAB-donor can detect the PCI conflict, while IAB-donor may not be aware of the cell configuration or pre-configurations for the IAB-node via OAM, since IAB-donor may connect to different OAM from IAB-node.

Proposal 1, RAN3 discuss whether the IAB-donor need to notify the detected PCI conflict info to IAB-node, so that the IAB-node can select a proper PCI in the reconfigured PCI list or notify the OAM to reassign PCI to avoid the conflict.

Proposal 2, RAN3 discuss whether the IAB-node should notify the preconfigured PCI list via OAM to IAB-donor, so that the IAB-donor can select a proper PCI for the IAB-node considering the detected PCI conflict.

Observation 2, PCI space partitioning via OAM cannot avoid the PCI collisions considering the interference cannot be predicted in advance when mobile IAB-nodes are widely deployed.

Proposal 3, both dynamic PCI change and frequency change can be considered for the avoidance of PCI collision in mobile IAB scenario.

Proposal 4, RAN3 consider the following information for PCI/frequency reconfiguration for the avoidance of PCI collisions,

- The pre-configurations in the mobile IAB-node

- The PCI configuration of the neighbours

- The PCI conflict detection info

- IAB-node movement info

Proposal 5, the enhancement for PCI conflict can also be used for RACH resource configuration conflict.

4	References