3GPP TSG-RAN WG3 #117bis-e                                                                                      R3-225794

Online, 10th - 18th October, 2022

Agenda Item:	16.3

Source:	CATT

Title:	Discussion on Service Continuity Enhancements for SL relay

Document for:	Discussion

- Introduction

In last meeting, some agreements for service continuity enhancements for U2N relay were achieved [1]:

Reuse the existing network procedures to support single-hop L2 U2N Relay in Rel-18.

Source gNB decides to trigger path switching for the L2N remote UE.

Current signaling can support Scenario C, i.e., intra-gNB indirect to indirect path switch.

Regarding the support of lossless data delivery during path switch, RAN3 would wait for RAN2’s progress first.

RAN3 focuses on the XnAP and possible F1AP impacts to support the basic scenarios.

FFS on which node should decide the new path type, i.e., either indirect or direct.

RAN3 continues analyzing the following options for selection of target Relay UE.

- Option 1: source gNB selects one target Relay UE and sends the ID related information to the target gNB

- Option 2: source gNB sends a list of candidate target Relay UE information to the target gNB for selection

- Option 3: source gNB provides also the measurement information of Remote UE to the target gNB for selection of target Relay UE

In this contribution, we further discuss the service continuity and give us proposals.

- Discussion

a) Inter-gNB indirect-to-direct path switching

For the handover preparation/execution stage, it is same as the normal UE inter-CU handover. After switching, target CU sends a UE context release message to source CU. Source CU may send UE context modification request to gNB-DU and also send a RRC reconfiguration to source relay UE to release remote UE context e.g., release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE. Note that the source gNB can send the RRC reconfiguration message to the source relay UE to release the remote UE context at any time after the source gNB sends the RRC reconfiguration message to the remote UE.

Observation 1: Source gNB can send the RRC reconfiguration message to the source relay UE to release the remote UE context at any time after the source gNB sends the RRC reconfiguration message to the remote UE.

Observation 2: Target CU sends RRC reconfiguration conveyed in UE context modification request for relay UE and UE context release request for DU to release remote UE context.

Proposal 1: Take Figure 1 as baseline for inter-gNB indirect-to-direct path switching.

Figure 1 inter-gNB indirect-to-direct path switching

b) Inter-gNB direct-to-indirect path switching

In the last meeting, there are three options for selection of target Relay UE.

Option 1: source gNB selects one target Relay UE and sends the ID related information to the target gNB

Option 2: source gNB sends a list of candidate target Relay UE information to the target gNB for selection

Option 3: source gNB provides also the measurement information of Remote UE to the target gNB for selection of target Relay UE

In legacy, source gNB is responsible for selecting a target cell, and target gNB make the final decision of target cell. Specifically, source gNB provides one target cell in Xn handover request message and a list of candidate target cells on each frequency in candidateCellInfoList in HandoverPreparationInformation.

candidateCellInfoList

A list of the best cells on each frequency for which measurement information was available

Observation 3: In legacy, source gNB is responsible for selecting a target cell and target gNB make the final decision of target cell.

When coming to the selection of target relay UE, we can follow the same principle. Source gNB has the PC5 measurement results. It is able to select one target relay UE and a list of candidate target relay UEs. From RAN3 perspective, we need to introduce target relay UE L2ID in handover request message. However, whether introduce a list of candidate target relay UE in HandoverPreparationInformation should be discussed in RAN2.

Proposal 2: Introduce one target relay UE L2ID in Xn handover request message. Whether introduce a list of candidate target relay UEs in HandoverPreparationInformation should be discussed in RAN2.

Another issue is the authorized information. If we agree source gNB sends target relay UE L2ID to target gNB, source gNB does not know whether the target relay UE has been authorized. If the source gNB decides handover the remote UE to a unauthorized target relay UE, the handover procedure will failure.

Also in 23.700-33 6.23.1 Solution #23: Session Continuity for path switching for L2 U2N Relay [2]

In this solution, the source gNB determines whether to switch to a direct cell or a L2 U2N relay UE. If the source gNB determines to switch to a L2 U2N relay UE, the source gNB selects the target L2 U2N relay UE for remote UE taken the authorized PLMN list for L2 U2N remote UE into account.

To avoid handover failure, source gNB should know the authorized information of target relay UE. There are two alternatives:

- Alt 1:Target relay UE reports the authorized information to remote UE via measurement procedure and remote UE further sends the authorized information to source gNB via measurement report.

- Alt 2: The authorized information can be informed to source gNB by CN or target gNB.

For Alt1, it will impact air interface and the authorized information may be tampered by false relay UE. Compared with Alt1, Alt2 is a safer solution but it will impact the NG/Xn interface. RAN3 should discuss how to inform the authorized information of target relay UE to source gNB.

Proposal 3: Source gNB should know the authorized information of target relay UE.

Figure 2 inter-gNB direct-to-indirect path switching

Proposal 4: Take Figure 2 as baseline for inter-gNB direct-to-indirect path switching.

c) Intra-gNB indirect-to-indirect path switching

GNB-CU decides switch remote UE to a target relay UE which is camped in the same gNB. GNB-CU triggers UE context setup/modification procedure for target relay UE, and also triggers UE context setup procedure for remote UE. After switching, gNB-CU sends UE context modification to convey RRC reconfiguration for source relay UE to release remote UE context, and also sends UE context release to release remote UE context from DU side.

Figure 3 intra-gNB indirect-to-indirect path switching

Proposal 5: Take Figure 3 as baseline for intra-gNB indirect-to-indirect path switching.

d) Inter-gNB indirect-to-indirect path switching

This procedure of inter-gNB indirect-to-indirect path switching is similar as the inter-gNB direct-to-indirect path switching. The difference is that source gNB should send RRC reconfiguration to source relay UE to release remote UE context after remote UE connects to target relay UE. This RRC reconfiguration message for relay UE can be sent any time after source gNB sends RRC reconfiguration message to remote UE based on gNB implementation.

Figure 4 inter-gNB indirect-to-indirect path switching

Observation 4: Compared with inter-gNB direct-to-indirect path switching, the procedure of inter-gNB indirect-to-indirect path switching should add a RRC reconfiguration message for source relay UE to release remote UE context after source gNB sends RRC reconfiguration message to remote UE.

Proposal 6: Take Figure 4 as baseline for inter-gNB indirect-to-indirect path switching.

- Conclusion

Observation 1: Source gNB can send the RRC reconfiguration message to the source relay UE to release the remote UE context at any time after the source gNB sends the RRC reconfiguration message to the remote UE, without the target gNB indication

Observation 2: Target CU sends RRC reconfiguration conveyed in UE context modification request for relay UE and UE context release request for DU.

Proposal 1: Take Figure 1 as baseline for inter-gNB indirect-to-direct path switching.

Observation 3: In legacy, source gNB is responsible for selecting a target cell and target gNB make the final decision of target cell.

Proposal 2: Introduce one target relay UE L2ID in Xn handover request message. Whether introduce a list of candidate target relay UEs in HandoverPreparationInformation should be discussed in RAN2.

Proposal 3: Source gNB should know the authorized information of target relay UE.

Proposal 4: Take Figure 2 as baseline for inter-gNB direct-to-indirect path switching.

Proposal 5: Take Figure 3 as baseline for intra-gNB indirect-to-indirect path switching.

Observation 4: Compared with inter-gNB direct-to-indirect path switching, the procedure of inter-gNB indirect-to-indirect path switching should add a RRC reconfiguration message for source relay UE to release remote UE context after source gNB sends RRC reconfiguration message to remote UE.

Proposal 6: Take Figure 4 as baseline for inter-gNB indirect-to-indirect path switching.

- Reference

[1] RAN3#117e, Chair notes.

[2] 23.700-33, Study on system enhancement for Proximity based Services.

- Annex for 38.423

============ Start of Change ==============

8.2.1	Handover Preparation

8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

If the Target Relay UE L2ID IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to configure target Relay UE for subsequent path switching.

============ Next Change ==============

9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| GUAMI | M |  | 9.2.3.24 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >NG-C UE associated Signalling reference | M |  | AMF UE NGAP ID 9.2.3.26 | Allocated at the AMF on the source NG-C connection. | – |  |
| >Signalling TNL association address at source NG-C side | M |  | CP Transport Layer Information 9.2.3.31 | This IE indicates the AMF’s IP address of the SCTP association used at the source NG-C interface instance. Note: If no UE TNLA binding exists at the source NG-RAN node, the source NG-RAN node indicates the TNL association address it would have selected if it would have had to create a UE TNLA binding. | – |  |
| >UE Security Capabilities | M |  | 9.2.3.49 |  | – |  |
| >AS Security Information | M |  | 9.2.3.50 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.3.17 |  | – |  |
| >PDU Session Resources To Be Setup List |  | 1 | 9.2.1.1 | Similar to NG-C signalling, containing UL tunnel information per PDU Session Resource; and in addition, the source side QoS flow DRB mapping | – |  |
| >RRC Context | M |  | OCTET STRING | Either includes the HandoverPreparationInformation message as defined in subclause 10.2.2. of TS 36.331 [14], or the HandoverPreparationInformation-NB message as defined in subclause 10.6.2 of TS 36.331 [14], if the target NG-RAN node is an ng-eNB, or the HandoverPreparationInformation message as defined in subclause 11.2.2 of TS 38.331 [10], if the target NG-RAN node is a gNB. | – |  |
| >Location Reporting Information | O |  | 9.2.3.47 | Includes the necessary parameters for location reporting. | – |  |
| >Mobility Restriction List | O |  | 9.2.3.53 |  | – |  |
| >5GC Mobility Restriction List Container | O |  | 9.2.3.100 |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.107 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.3.108 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| >MBS Session Information List | O |  | 9.2.1.36 |  | YES | ignore |
| >5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.2.3.107 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| >UE Slice Maximum Bit Rate List | O |  | 9.2.3.167 |  | YES | ignore |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| UE History Information | M |  | 9.2.3.64 |  | YES | ignore |
| UE Context Reference at the S-NG-RAN node | O |  |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | – |  |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >Target NG-RAN node UE XnAP ID | C-ifCHOmod |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| NR V2X Services Authorized | O |  | 9.2.3.105 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.2.3.106 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.3.109 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source NG-RAN node provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| Time Synchronisation Assistance Information | O |  | 9.2.3.153 |  | YES | ignore |
| QMC Configuration Information | O |  | 9.2.3.156 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.2.3.159 |  | YES | ignore |
| 5G ProSe PC5 QoS Parameters | O |  | 9.2.3.160 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| Target Relay UE L2ID | O |  | BIT STRING (SIZE(24)) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

============ Next Change ==============

9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

XnAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ActivationIDforCellActivation,

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

SRB-ID

TargetRelayUEL2ID

FROM XnAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

XNAP-PRIVATE-IES,

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES,

XNAP-PROTOCOL-IES-PAIR

FROM XnAP-Containers

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

id-TargetRelayUEL2ID

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

HandoverRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-sourceNG-RANnodeUEXnAPID			CRITICALITY reject	TYPE NG-RANnodeUEXnAPID							PRESENCE mandatory}|

{ ID id-Cause								CRITICALITY reject	TYPE Cause										PRESENCE mandatory}|

{ ID id-targetCellGlobalID					CRITICALITY reject	TYPE Target-CGI									PRESENCE mandatory}|

{ ID id-GUAMI								CRITICALITY reject	TYPE GUAMI										PRESENCE mandatory}|

{ ID id-UEContextInfoHORequest				CRITICALITY reject	TYPE UEContextInfoHORequest						PRESENCE mandatory}|

{ ID id-TraceActivation						CRITICALITY ignore	TYPE TraceActivation							PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore	TYPE MaskedIMEISV								PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore	TYPE UEHistoryInformation						PRESENCE mandatory}|

{ ID id-UEContextRefAtSN-HORequest			CRITICALITY ignore	TYPE UEContextRefAtSN-HORequest				PRESENCE optional }|

{ ID id-CHOinformation-Req					CRITICALITY reject	TYPE CHOinformation-Req							PRESENCE optional }|

{ ID id-NRV2XServicesAuthorized				CRITICALITY ignore	TYPE NRV2XServicesAuthorized					PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized			CRITICALITY ignore	TYPE LTEV2XServicesAuthorized					PRESENCE optional }|

{ ID id-PC5QoSParameters					CRITICALITY ignore	TYPE PC5QoSParameters							PRESENCE optional }|

{ ID id-MobilityInformation					CRITICALITY ignore	TYPE MobilityInformation						PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore	TYPE UEHistoryInformationFromTheUE			PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject	TYPE IABNodeIndication							PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore	TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-TimeSynchronizationAssistanceInformation	CRITICALITY ignore	TYPE TimeSynchronizationAssistanceInformation		PRESENCE optional }|

{ ID id-QMCConfigInfo						CRITICALITY ignore	TYPE QMCConfigInfo								PRESENCE optional }|

{ ID id-FiveGProSeAuthorized				CRITICALITY ignore	TYPE FiveGProSeAuthorized						PRESENCE optional }|

{ ID id-FiveGProSePC5QoSParameters			CRITICALITY ignore	TYPE FiveGProSePC5QoSParameters				PRESENCE optional },

{ ID id-TargetRelayUEL2ID					CRITICALITY ignore	TYPE TargetRelayUEL2ID							PRESENCE optional }|

...

}

============ Next Change ==============

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

#### -- T

TABasedMDT ::= SEQUENCE {

tAListforMDT		TAListforMDT,

iE-Extensions		ProtocolExtensionContainer { {TABasedMDT-ExtIEs} } OPTIONAL,

...

}

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

TargetRelayUEL2ID ::=  BIT STRING(SIZE(24))

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

#### -- IEs

--

-- **************************************************************

#### id-ActivatedServedCells

//////////////////////////////////////////////////////////////skip unchanged//////////////////////////////////////////////////////////////

id-TargetRelayUEL2ID													ProtocolIE-ID ::= xxx

END

-- ASN1STOP

============ End of Change ==============