3GPP TSG-RAN WG3 #117bis-e	R3-225933

Online, 10-18 October 2022

Agenda Item:	19.2

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: # UAV_Identification

Document for:	Approval

# Introduction

CB: # UAV_Identification

- How to support subscription based aerial UE identification, Re-use LTE mechanism?

- Whether to consider UAV supporting on MRDC, inter-RAT mobility?

- Capture agreements and provide CRs if agreeable.

(Nok - moderator)

Summary of offline disc R3-225933

The first round of discussion is set to deadline on 11th October (Tuesday) 11:00 UTC.

The final round of discussion is set to deadline on 17th October (Monday) 08:00 UTC.

# For the Chairman’s Notes

TBD.

# Discussion

This is the first meeting in RAN3 where UAV for NR (NR_UAV) work item is discussed in RAN3. One of the objectives is defined as follows:

2. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2] Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

Most of the contributions submitted indicate similar proposals.

- Provide Aerial Subscription Information from (a) AMF to gNB via NGAP, and (b) between gNBs via XnAP

- Base the Aerial UE Subscription Information IE with similar codepoints as in the LTE solution (ENUMERATED (allowed, not allowed, …)

- Over NGAP, Introduce the Aerial Subscription Information in Initial Context Setup, UE Context Modification, Handover Request, Path Switch Request Acknowledge (*) messages

- Over XnAP, Introduce the Aerial Subscription Information in Handover Request, Retrieve UE Context Response (*) messages

- (*) Indicates messages which were not included as part of all of the proposals

Additionally, some companies made remark that further RAN3 changes may be needed for inter-RAT mobility and Dual-Connectivity is supported by additional RAN2 progress.

Likewise, some draft text proposals were made available within the submissions for Stage 2 (TS 38.300) and Stage 3 (TS 38.413 and TS 38.423).

At this initial meeting, the moderator proposes to capture the high-level agreements to be the basis for the future Stage 2 and Stage 3 specifications changes, and to derive the corresponding BL CRs at the next RAN3#118 meeting.

## Companies to provide valuable comments for the possible solutions

Q1: Do you agree to introduce Aerial UE Subscription Information IE with codepoints ENUMERATED (allowed, not allowed, …) over NGAP and XnAP. If not, please indicate the reason.

| Company | Comment |
| --- | --- |
| Nokia | Yes |

Moderator Summary : TBD

Q2: Do you agree to introduce Aerial UE Subscription Information IE over NGAP over the following messages? If not, please indicate the reason.

- INITIAL CONTEXT SETUP

- UE CONTEXT MODIFICATION

- HANDOVER REQUEST

- PATH SWITCH REQUEST ACKNOWLEDGE

| Company | Comment |
| --- | --- |
| Nokia | Yes, agree to introduce in all the mentioned NGAP messages |

Moderator Summary : TBD

Q3: Do you agree to introduce Aerial UE Subscription Information IE over XnAP over the following messages? If not, please indicate the reason.

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

| Company | Comment |
| --- | --- |
| Nokia | Yes, agree to introduce in all the mentioned XnAP messages |

Moderator Summary : TBD

# Conclusion, Recommendations [if needed]

TBU

# References

| R3-225374 | UAV Support over NG-RAN interface (NEC) | discussion |
| --- | --- | --- |
| R3-225572 | On subscription-based UAV UE identification (Nokia, Nokia Shanghai Bell) | discussion |
| R3-225618 | Subscription-based Aerial-UE Identification for NR (CATT) | other |
| R3-225694 | Initial discussion on NR support for UAV (Huawei) | discussion |
| R3-225804 | Subscription-based aerial-UE identification for NR UAV (CMCC) | discussion |
| R3-225855 | NR support for UAV WI (Ericsson) | discussion |
| R3-225856 | Introduction of Aerial authorization information (Ericsson, AT&T, NTT DOCOMO, INC, Qualcomm Incorporated, Intel Corporation) | CR0618r2, TS 38.413 v17.2.0, Rel-18, Cat. B |
| R3-225875 | Discussion on NR support for UAV (ZTE) | discussion |