3GPP TSG-RAN WG3 Meeting #118	R3-226227

14 November – 18 November 2022

Toulouse, France

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1037 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additions for L1/L2 triggered mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2 |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to fulfil the objectives of the WID in RP-221799 (NR_Mob_enh2) a number of changes are required. This CR implements the changes in the affected procedures between gNB-DU, and gNB-CU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new IEs in UE CONTEXT SETUP REQUEST message, UE CONTEXT SETUP RESPONSE message, UE CONTEXT MODIFICATION REQUEST message, UE CONTEXT MODIFICATION RESPONSE message, UE CONTEXT MODIFICATION REQUIRED message, UE CONTEXT MODIFICATION CONFIRM message to support L1/L2 triggered mobility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | L1/L2 triggered mobility based procedures cannot be supported on F1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1, 8.3.4, 8.3.5, 9.2.2.1, 9.2.2.2, 9.2.2.7, 9.2.2.8, 9.2.2.10 and 9.2.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: R3-225351 |  |  |  |  |  |  |  |  |

<<<<<< NEXT CHANGE >>>>>>

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB,DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. The gNB-CU shall perform RRC Reconfiguration or RRC connection resume as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the SpCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly.

If the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall use the provided value from the gNB-CU.

If the UL Configuration IE in DRB to Be Setup Item IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB. If the Additional Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup the indicated RLC entities for the indicated SRB. If the SDT RLC Bearer Configuration IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT SRB indicated by the SRB ID IE. If the SRB Mapping Info IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the SRB Mapping Info IE for the SRB identified by the SRB ID IE and the Uu Relay RLC channel identified by the SRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of SRB data to Uu Relay RLC channel.

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

If the BH RLC Channel To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE as follows:

-	if the IP to layer2 Traffic Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the IP to layer2 Traffic Mapping Info To Add IE, if present, for the egress BH RLC channel identified by the BH RLC CH ID IE, and shall remove the previously stored mapping information as indicated by the IP to layer2 Mapping Traffic Info To Remove IE, if present. The gNB-DU shall use the mapping information stored for the mapping of IP traffic to layer 2, as specified in TS 38.340 [30].

-	if the BAP layer BH RLC channel Mapping Info IE is included, the gNB-DU shall store the mapping information  contained in the BAP layer BH RLC channel Mapping Info To Add IE, if present, for the egress or ingress BH RLC channel identified by the BH RLC CH ID IE, and shall remove the previously stored mapping information as indicated by the BAP layer BH RLC channel Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored when forwarding traffic on BAP sublayer, as specified in TS 38.340 [30].

If two UL UP TNL Information IEs are included in UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT SETUP RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT SETUP REQUEST message for a DRB, the gNB-DU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT SETUP RESPONSE message and setup one or two additional RLC entities for the indicated DRB. The gNB-CU and the gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2].

If Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activating/deactivating CA based PDCP duplication for the DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when activating/deactivating CA based PDCP duplication for the DRB with more than two RLC entities.

If DC Based Duplication Configured IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. If DC Based Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activating/deactivating DC based PDCP duplication for this DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT SETUP REQUEST message for a DRB, the gNB-DU shall, if supported, take it into account when activating/deactivating DC based PDCP duplication for the DRB with more than two RLC entities. If the Primary Path Indication IE is included in the RLC Duplication Information IE, the gNB-DU shall, if supported, take it into account when performing DC based PDCP duplication for the DRB with more than two RLC entities.

If UL PDCP SN length IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. If the Additional RRM Policy Index IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall, if supported, contain the Additional RRM Policy Index IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20]. The gNB-DU shall, if supported, store the received Additional RRM Policy Index in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is available at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT SETUP REQUEST. The gNB-DU may use it for RRM purposes.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested DRBs, SRBs, BH RLC channels, Uu RLC channels, PC5 Relay RLC channels, and SL DRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to Setup List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of BH RLC channels which are successfully established shall be included in the BH RLC Channel Setup List IE;

-	A list of BH RLC channels which failed to be established shall be included in the BH RLC Channel Failed to be Setup List IE;

-	A list of SL DRBs which are successfully established shall be included in the SL DRB Setup List IE;

-	A list of SL DRBs which failed to be established shall be included in the SL DRB Failed to Setup List IE.

-	A list of Uu Relay RLC channels which are successfully established shall be included in the Uu RLC Channel Setup List IE;

-	A list of Uu Relay RLC channels which failed to be established shall be included in the Uu RLC Channel Failed to be Setup List IE;

-	A list of PC5 Relay RLC channels which are successfully established shall be included in the PC5 RLC Channel Setup List IE;

-	A list of PC5 Relay RLC channels which failed to be established shall be included in the PC5 RLC Channel Failed to be Setup List IE;

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB or SL DRB or a BH RLC channel or a Uu RLC channel or a PC5 Relay RLC channel, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

For EN-DC operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15].

For NG-RAN operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the DRB Information IE.

For DC operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE at the gNB acting as secondary node. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

For sidelink operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE if the gNB-CU receives sidelink related UE information from UE. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8].

If the HandoverPreparationInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU of the gNB acting as master node shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. The gNB-CU shall only initiate the UE Context Setup procedure for handover or secondary node addition when at least one DRB is setup for the UE, or at least one BH RLC channel is set up for IAB-MT. If the HandoverPreparationInformation IE containing the sidelink related UE information is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8].

If the received CU to DU RRC Information IE does not include source cell group configuration, the gNB-DU shall generate the cell group configuration using full configuration. Otherwise, delta configuration is allowed.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall deduce that changes to the measurements configuration need to be applied. If the measObjectToAddModList IE is included in the MeasConfig IE, then the frequencies added in such IE are to be activated. Then the gNB-DU shall decide if measurement gaps are needed or not and, if needed, the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message. If the measObjectToRemoveList IE is included in the MeasConfig IE, the gNB-DU shall ignore it.

If the NeedForGapsInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT SETUP REQUEST message shall be ignored. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The UEAssistanceInformationEUTRA IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformationEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring LTE sidelink resources for the UE.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the Masked IMEISV IE is contained in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be set up with an appropriate cause value for each SCell failed to setup.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT SETUP REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT SETUP RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

If the Full Configuration IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

If the C-RNTI IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

The UE Context Setup Procedure is not used to configure SRB0.

If the UE CONTEXT SETUP REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE via SRB1.

If the Notification Control IE is included in the DRB to Be Setup List IE contained in the UE CONTEXT SETUP REQUEST message and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store the received UL PDU Session Aggregate Maximum Bit Rate and use it when enforcing uplink traffic policing for non-GBR Bearers for the concerned UE as specified in TS 23.501 [21].

The gNB-DU shall store the received gNB-DU UE Aggregate Maximum Bit Rate Uplink and use it for non-GBR Bearers for the concerned UE.

If the UE CONTEXT SETUP REQUEST message contains the QoS Flow Mapping Indication IE, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the UE CONTEXT SETUP REQUEST message contains the New gNB-CU UE F1AP ID IE, the gNB-DU shall, if supported, replace the value received in the gNB-CU UE F1AP ID IE by the value of the New gNB-CU UE F1AP ID and use it for further signalling.

If the RAN UE ID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store and replace any previous information received.

If the Trace Activation IE is included in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, initiate the requested trace function as described in TS 32.422 [29].

In particular, the gNB-DU shall, if supported:

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT and Trace", initiate the requested trace session and MDT session as described in TS 32.422 [29];

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT Only", initiate the requested MDT session as described in TS 32.422 [29] and the gNB-DU shall ignore Interfaces To Trace IE, and Trace Depth IE. If the Management Based MDT PLMN List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [29].

For each QoS flow whose DRB has been successfully established and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [21].

If the UE CONTEXT SETUP REQUEST message contains the Configured BAP Address IE, the gNB-DU shall, if supported, store this BAP address configured for the corresponding child IAB-node and use it as specified in TS 38.340 [30].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Setup List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30].

If the F1-C Transfer Path IE is included in UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account.

If the NR V2X Services Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the LTE V2X Services Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the NR UE Sidelink Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for NR V2X services.

If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for LTE V2X services.

If the PC5 Link Aggregate Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for NR V2X services as defined in TS 23.287 [40].

If the TSC Traffic Characteristics IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

If the Conditional Inter-DU Mobility Information IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider that the request concerns a conditional handover or conditional PSCell addition or conditional PSCell change for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Target gNB-DU UE F1AP ID IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU shall replace the existing prepared conditional handover or conditional PSCell addition or conditional PSCell change identified by the Target gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the Serving NID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall combine the Serving NID IE with the Serving PLMN IE to identify the serving NPN, and may take it into account for UE context establishment.

If the Estimated Arrival Probability IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If for a given E-RAB for EN-DC operation the ENB DL Transport Layer Address IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If for a given Qos flow for NG-RAN operation the PDCP Terminating Node DL Transport Layer Address IE is included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the F1-C Transfer Path NRDC IE is included in UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account.

If the MDT Polluted Measurement Indicator IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account as specified in TS 38.401 [4].

If the SCG Activation Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU may use it to configure SCG resources as specified in TS 37.340 [7] , and if supported, shall include the SCG Activation Status IE in the UE CONTEXT SETUP RESPONSE message. If the SCG Activation Request IE in the UE CONTEXT SETUP REQUEST message is set to “Activate SCG”, the gNB-DU shall activate the SCG resources and set the SCG Activation Status IE in the UE CONTEXT SETUP RESPONSE message to “SCG Activated”.

If the Old CG-SDT Session Info IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, retrieve the old CG-SDT resource configuration and old UE context based on the indicated gNB-CU F1AP UE ID and gNB-DU F1AP UE ID.

If the 5G ProSe Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for 5G ProSe services.

If the 5G ProSe PC5 Link Aggregate Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for 5G ProSe services as defined in TS 23.304 [44].

If the Uu RLC Channel To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the Uu Relay RLC channel configurations for a L2 U2N Relay UE.

If the PC5 RLC Channel To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the PC5 Relay RLC channel configurations for a L2 U2N Remote UE.

If the Path Switch Configuration IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to configure the path switch from direct path to indirect path as specified in TS 38.401 [4].

If the MUSIM-GapConfig IE is contained in the CU to DU RRC Information IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, decide to use this IE for MUSIM gap configuration or select another one based on the received UEAssistanceInformation IE. If gNB-DU selects a different MUSIM gap configuration from received UEAssistanceInformation IE, then it shall include the selected MUSIM gap information to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

If MUSIM-GapConfig IE is not contained in the CU to DU RRC Information IE, then gNB-DU shall, if supported, send the selected MUSIM gap configuration based on the received UEAssistanceInformation IE, to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message. When MUSIM-GapConfig IE is received, the gNB-CU should use this value.

If the gNB-DU UE Slice Maximum Bit Rate List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store and use the information for the uplink traffic policing for each concerned slice as specified in TS 23.501 [21].

If the Multicast MBS Session Setup List IE is contained in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, store and use the information for configuring MBS Session Resources, if applicable.

If the UE Multicast MRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for configuring MBS Session Resources, if applicable. And if the MBS PTP Retransmission Tunnel Required IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Retransmission F1-U tunnel. If the MBS PTP Forwarding Tunnel Required Information IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Forwarding F1-U tunnel. If the Source MRB ID IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the DU shall, if supported, use it to identify the MRB configuration as provided to the UE in the source cell and take it into account for configuring MBS Session Resources.

If the InterFrequencyConfig-NoGap IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the ul-GapFR2-Config IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the Candidate L1/L2 Cell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider that these cells are candidate cells recommended for L1/L2 triggered mobility and will take them under consideration.

If the L1/L2 Cell List IE is included in the UE CONTEXT SETUP RESPONSE message the gNB-CU will understand that the cells are configured for L1/L2 triggered mobility.

If the L1/L2 Cell Failed To Setup List IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the corresponding Cell(s) for L1/L2 triggered mobility failed to be set up with an appropriate cause value for each Cell failed to setup.

If the Candidate L1/L2 SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider it as a list of candidate SCells recommended for L1/L2 triggered mobility and will take them under consideration.

If the L1/L2 SCell List IE is included in the UE CONTEXT SETUP RESPONSE message the gNB-CU will understand that the SCells are configured for L1/L2 triggered mobility.

If the L1/L2 SCell Failed To Setup List IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) for L1/L2 triggered mobility failed to be set up with an appropriate cause value for each SCell failed to setup.

Editor’s note: The Candidate L1/L2 SCell To Be Setup List IE, the L1/L2 SCell List IE and the L1/L2 SCell Failed To Setup List IE are FFS

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: UE Context Setup Request procedure: unsuccessful Operation

If the gNB-DU is not able to establish an F1 UE context, or cannot even establish one bearer it shall consider the procedure as failed and reply with the UE CONTEXT SETUP FAILURE message. If the Conditional Inter-DU Mobility Information IE was included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT SETUP FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT SETUP REQUEST message, it shall reply with the UE CONTEXT SETUP FAILURE message with an appropriate cause value. Further, if the Candidate SpCell List IE is included in the UE CONTEXT SETUP REQUEST message and the gNB-DU is not able to accept the SpCell ID IE, the gNB-DU shall, if supported, include the Potential SpCell List IE in the UE CONTEXT SETUP FAILURE message and the gNB-CU should take this into account for selection of an opportune SpCell. The gNB-DU shall include the cells in the Potential SpCell List IE in a priority order, where the first cell in the list is the one most desired and the last one is the one least desired (e.g., based on load conditions). If the Potential SpCell List IE is present but no Potential SpCell Item IE is present, the gNB-CU should assume that none of the cells in the Candidate SpCell List IE are acceptable for the gNB-DU.

If the gNB-DU is not able to accept any of the cells in the Candidate L1/L2 Cell To Be Setup List IE, it shall reply with the UE CONTEXT SETUP FAILURE message with an appropriate cause value.

#### 8.3.1.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value. If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT SETUP REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

In case of "CHO-replace" when the Target gNB-DU UE F1AP ID IE is included, if the candidate cell in the SpCell ID IE included in the UE CONTEXT SETUP REQUEST message was not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore this candidate cell.

<<<<<< NEXT CHANGE >>>>>>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

If the SpCell ID IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace any previously received value and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the ServCellIndex IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take this into account for the indicated SpCell. If the SpCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message and the indicated SCell(s) are already setup, the gNB-DU shall replace any previously received value. If the SCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the SCell To Be Removed List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of SCells to be removed.

If the DRX Cycle IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall use the provided value from the gNB-CU. If the DRX configuration indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and set to "release", the gNB-DU shall release DRX configuration.

If the SL DRX Cycle list IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the provided value from the gNB-CU for the indicated RX UE of this UE. If the SL DRX configuration indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and set to "release", the gNB-DU shall, if supported, release SL DRX configuration for the indicated RX UE of this UE.

If the SRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4], and replace any previously received value. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB if the value is set to be "true", or delete the RLC entity of secondary path if the value is set to be "false". If the Additional Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup the indicated RLC entities for the indicated SRB. If the SRB Mapping Info IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the SRB Mapping Info IE for the SRB identified by the SRB ID IE and the Uu Relay RLC channel identified by the SRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of SRB data to Uu Relay RLC channel.

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4]. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE, if present, for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

If the BH RLC Channel To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE following the behaviour described for the UE Context Setup procedure.

If the BH RLC Channel To Be Modified List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Modified Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE following the behaviour described for the UE Context Setup procedure.

If the BH RLC Channel To Be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall release the BH RLC channels in the list.

If two UL UP TNL Information IEs are included in UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT MODIFICATION RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT MODIFICATION RESPONSE message and setup one or two additional RLC entities for the indicated DRB. The gNB-CU and the gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2].

If Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activating/deactivating CA based PDCP duplication for the DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for the DRB with more than two RLC entities.

If DC Based Duplication Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. Otherwise, the gNB-DU shall regard that DC based PDCP duplication is de-configured for this DRB id the value is set to be "false", and it should stop PDCP duplication activation/deactivation by MAC CE. If DC Based Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activating/deactivating DC based PDCP duplication for this DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall, if supported, take it into account when activating/deactivating DC based PDCP duplication for the DRB with more than two RLC entities. If the Primary Path Indication IE is included in the RLC Duplication Information IE, the gNB-DU shall, if supported, take it into account when performing DC based PDCP duplication for the DRB with more than two RLC entities.

For a certain DRB which was allocated with two GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the UE Context Modification procedure, the gNB-DU shall consider that the CA based PDCP duplication for the concerned DRB is de-configured. If such UE Context Modification procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the UL Configuration IE in DRB to Be Setup Item IE or DRB to Be Modified Item IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the RRC Reconfiguration Complete Indicator IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider the ongoing reconfiguration procedure involving changes of the L1/L2 configuration at the gNB-DU signalled to the gNB-CU via the CellGroupConfig IE for MR-DC operation or standalone operation has been successfully performed when such IE is set to ‘true’; otherwise (when such IE is set to ‘failure’), the gNB-DU shall consider the ongoing reconfiguration procedure has been failed and it shall continue to use the old L1/L2 configuration.

If DL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If UL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If the RLC Failure Indication IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU should consider that the RLC entity indicated by such IE needs to be re-established when the CA-based packet duplication is active, and the gNB-DU may include the Associated SCell List IE in UE CONTEXT MODIFICATION RESPONSE by containing a list of SCell(s) associated with the RLC entity indicated by the RLC Failure Indication IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE. If the UE CONTEXT MODIFICATION REQUEST message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the Transmission Action Indicator IE, the gNB-DU shall stop or restart (if already stopped) data transmission for the UE, according to the value of this IE. It is up to gNB-DU implementation when to stop or restart the UE scheduling.

For EN-DC operation, if the DRB to Be Setup List IE is present in the UE CONTEXT MODIFICATION REQUEST message the gNB-CU shall include the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15]. For NG-RAN operation, the gNB-CU shall include the DRB Information IE in the UE CONTEXT MODIFICATION REQUEST message.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall deduce that changes to the measurements’ configuration need to be applied. The gNB-DU shall take the received info, e.g. the measObjectToAddModList IE, and/or the measObjectToRemoveList IE into account, when generating measurement gap and when deciding if a measurement gap is needed or not.

If the NeedForGapsInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

For DC operation, if the gNB-CU includes the CG-Config IE in the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may initiate low layer parameters coordination taking this information into account.

For sidelink operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE if the gNB-CU receives sidelink related UE information from UE. If the CG-ConfigInfo IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information as defined in TS 38.331 [8].

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT MODIFICATION REQUEST message shall be ignored.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT MODIFICTION REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. If the Additional RRM Policy Index IE is received from an MeNB, the UE CONTEXT MODIFICATION REQUEST message shall , if supported, contain the Additional RRM Policy Index IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20]. The gNB-DU shall, if supported, store the received Additional RRM Policy Index in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is modified at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT MODIFICATION REQUEST. The gNB-DU may use it for RRM purposes.

If the UE CONTEXT MODIFICATION REQUEST message contains the Uplink TxDirectCurrentList Information IE, the gNB-DU may take that into account when selecting L1 configuration.

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The UEAssistanceInformationEUTRA IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformationEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring LTE sidelink resources for the UE.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT MODIFICATION RESPONSE message, the result for all the requested or modified DRBs, SRBs, BH RLC Channels, Uu Relay RLC channels, PC5 Relay RLC channels, and SL DRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to be Setup List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed to be Modified List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to be Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of successfully modified SRBs with logical channel identities for primary path shall be included in the SRB Modified List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of BH RLC channels which are successfully established shall be included in the BH RLC Channel Setup List IE;

-	A list of BH RLC channels which failed to be established shall be included in the BH RLC Channel Failed to be Setup List IE;

-	A list of BH RLC channels which are successfully modified shall be included in the BH RLC Channel Modified List IE;

-	A list of BH RLC channels which failed to be modified shall be included in the BH RLC Channel Failed to be Modified List IE;

-	A list of Uu Relay RLC channels which are successfully established shall be included in the Uu RLC Channel Setup List IE;

-	A list of Uu Relay RLC channels which failed to be established shall be included in the Uu RLC Channel Failed to be Setup List IE;

-	A list of Uu Relay RLC channels which are successfully modified shall be included in the Uu RLC Channel Modified List IE;

-	A list of Uu Relay RLC channels which are failed to be modified shall be included in the Uu RLC Channel Failed to be Modified List IE;

-	A list of PC5 Relay RLC channels which are successfully established shall be included in the PC5 RLC Channel Setup List IE;

-	A list of PC5 Relay RLC channels which failed to be established shall be included in the PC5 RLC Channel Failed to be Setup List IE;

-	A list of PC5 Relay RLC channels which are successfully modified shall be included in the PC5 RLC Channel Modified List IE;

-	A list of PC5 Relay RLC channels which failed to be modified shall be included in the PC5 RLC Channel Failed to be Modified List IE;

-	A list of SL DRBs which are successfully established shall be included in the SL DRB Setup List IE;

-	A list of SL DRBs which failed to be established shall be included in the SL DRB Failed to be Setup List IE;

-	A list of SL DRBs which are successfully modified shall be included in the SL DRB Modified List IE;

-	A list of SL DRBs which failed to be modified shall be included in the SL DRB Failed to be Modified List IE.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Setup List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Modified List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30]. Otherwise, if the BAP Control PDU Channel IE is not present for any BH RLC channel, any available BH RLC channel can be used to transmit BAP Control PDUs as specified in TS 38.340 [30].

If the F1-C Transfer Path IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB or SL DRB or a BH RLC channel or a Uu Relay RLC channel or a PC5 Relay RLC channel, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the DU to CU RRC Information IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall perform RRC Reconfiguration as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP MODIFICATION REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be set up with an appropriate cause value for each SCell failed to setup.

If the C-RNTI IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT MODIFICATION RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

The UE Context Modify Procedure is not used to configure SRB0.

If in the UE CONTEXT MODIFICATION REQUEST, the Notification Control IE is included in the DRB to Be Setup List IE or the DRB to Be Modified List IE and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace the received UL PDU Session Aggregate Maximum Bit Rate and use it as specified in TS 23.501 [21].

If the gNB-DU UE Aggregate Maximum Bit Rate Uplink IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall:

-	replace the previously provided gNB-DU UE Aggregate Maximum Bit Rate Uplink with the new received gNB-DU UE Aggregate Maximum Bit Rate Uplink;

-	use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink for non-GBR Bearers for the concerned UE.

The gNB-DU UE Aggregate Maximum Bit Rate Uplink IE shall be sent in the UE CONTEXT MODIFICATION REQUEST if DRB to Be Setup List IE is included and the gNB-CU has not previously sent it. The gNB-DU shall store and use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink IE.

If the RLC Status IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall assume that RLC has been reestablished at the gNB-DU and may trigger PDCP data recovery.

If the GNB-DU Configuration Query IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall include the DU To CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Bearer Type Change IE is included in DRB to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall either reset the lower layers or generate a new LCID for the affected bearer as specified in TS 37.340 [7].

For NE-DC operation, if NeedforGap IE is included in the UE CONTEXT MODIFICATION REQUEST message,the gNB-DU shall generate measurement gap for the SeNB.

If the QoS Flow Mapping Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Lower Layer presence status change IE set to "suspend lower layers" is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-DU shall keep all lower layer configuration for UEs, and not transmit or receive data from UE.

If the Lower Layer presence status change IE set to "resume lower layers" is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall use the previously stored lower layer configuration for the UE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate a CellGroupConfig IE using full configuration and include it in the UE CONTEXT MODIFICATION RESPONSE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

For each QoS flow whose DRB has been successfully established or modified and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [21].

If the NR V2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its V2X services authorization information for the UE accordingly. If the NR V2X Services Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the LTE V2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its V2X services authorization information for the UE accordingly. If the LTE V2X Services Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE LTE Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE NR Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

If the PC5 Link Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE PC5 Link Aggregate Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services as defined in TS 23.287 [40].

If the TSC Traffic Characteristics IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-initiation", the gNB-DU shall consider that the request concerns a conditional handover or conditional PSCell addition or conditional PSCell change for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-replace", the gNB-DU shall replace the existing prepared conditional mobility identified by the gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-cancel", the gNB-DU shall consider that the gNB-CU is about to remove any reference to, and release any resources previously reserved for the candidate cells associated to the UE-associated signalling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE. If the Candidate Cells To Be Cancelled List IE is also included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider that only the resources reserved for the cells identified by the included NR CGIs are about to be released by the gNB-CU.

If the Transmission Stop Indicator IE is included within the DRB to Be Modified Item IE in the UE CONTEXT MODIFICATION REQUEST message and set to “true”, the gNB-DU shall, if supported, stop the data transmission for the DRB. It is up to gNB-DU implementation when to stop the UE scheduling for that DRB.

If the SCG Indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and it is set to “released”, the gNB-DU shall, if supported, deduce that an SCG is removed.

If the Estimated Arrival Probability IE is contained in the Conditional Intra-DU Mobility Information IE included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If the Location Measurement Information IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring measurement gaps for the UE.

If the F1-C Transfer Path NRDC IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account.

If for a given E-RAB for EN-DC operation the ENB DL Transport Layer Address IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If for a given Qos flow for NG-RAN operation the PDCP Terminating Node DL Transport Layer Address IE is included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the gNB-DU is an IAB-DU, and if the IAB Conditional RRC Message Delivery Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message together with the RRC-Container IE, and if its value is set to “true”, and if the RRC-Container IE is for a child IAB-MT of the gNB-DU, the gNB-DU shall, if supported, withhold the RRC message until one of the following conditions is met:

If the gNB-DU belongs to a migrating IAB-node, whose co-located IAB-MT has successfully performed the random-access procedure to the target parent node, and if the migrating IAB-node has one or more routing entries for the target path.

The gNB-DU receives a subsequent F1AP message including an RRC-Container IE for the same child node.

If the gNB-DU belongs to a descendant node of the migrating IAB-node, whose co-located IAB-MT has received an RRCReconfiguration message including the intra-donor migration configurations, e.g., new TNL address(es) and the new default UL  BAP routing ID.

If the gNB-DU belongs to a migrating IAB-node, whose co-located IAB-MT has successfully performed RLF recovery after handover failure, and if the migrating IAB-node has one or more routing entries for the target path.

If the MDT Polluted Measurement Indicator IE is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-DU shall take this information into account as specified in TS 38.401 [4].

If the SCG Activation Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may use it to configure SCG resources as specified in TS 37.340 [7] , and if supported, shall include the SCG Activation Status IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the CG-SDT Query Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message and set to ‘true’, the gNB-DU shall, if supported, provide the CG-SDT related resource configuration for the bearers indicated as SDT bearers in the SDT-MAC-PHY-CG-Config IE within the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU. If the SDT-MAC-PHY-CG-Config IE is also included in the UE CONTEXT MODIFICATION REQUEST message within the CU to DU RRC Information IE, the gNB-DU may provide the delta signalling version of the SDT-MAC-PHY-CG-Config IE within the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

If the 5G ProSe Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its 5G ProSe services authorization information for the UE accordingly. If the 5G ProSe Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the SDT Bearer Configuration Query Indication IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, provide the RLC bearer configuration in the SDT Bearer Configuration Info IE in the UE CONTEXT MODIFICATION RESPONSE message for each bearer indicated as SDT bearer.

If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided 5G ProSe UE PC5 Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

If the 5G ProSe PC5 Link Aggregate Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided 5G ProSe PC5 Link Aggregate Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services as defined in TS 23.304 [44].

If the Updated Remote UE Local ID IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, replace the previously provided Remote UE Local ID, if available in the UE context, with the received value.

If the Uu RLC Channel To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4].

If the Uu RLC Channel To Be Modified List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4].

If the Uu RLC Channel To Be Release List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the Uu Relay RLC channels in the list.

If the PC5 RLC Channel To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the PC5 Relay RLC channel configurations for a L2 U2N Remote UE or U2N Relay UE. If the F1AP-IDs are associated with a U2N Relay UE, the PC5 RLC Channel to be Setup Item IEs IE shall include the Remote UE Local ID and correspondingly, the PC5 RLC Channel Setup Item IEs IE and the PC5 RLC Channel Failed to be Setup Item IE in the UE CONTEXT MODIFICATION RESPONSE message shall include the Remote UE Local ID IE.

If the PC5 RLC Channel To Be Modified List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the PC5 Relay RLC channel configurations for a L2 U2N Remote UE or U2N Relay UE. If the F1AP-IDs are associated with a U2N Relay UE, the PC5 RLC Channel to be Modified Item IEs IE shall include the Remote UE Local ID IE and correspondingly, the PC5 RLC Channel Modified Item IEs IE and the PC5 RLC Channel Failed to be Modified Item IEs IE in the UE CONTEXT MODIFICATION RESPONSE message shall include the Remote UE Local ID IE.

If the PC5 RLC Channel To Be Release List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the PC5 Relay RLC channels in the list. If the F1AP-IDs are associated with a U2N Relay UE, the PC5 RLC Channel to be Released Item IEs IE shall include the Remote UE Local ID IE.

If the Path Switch Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it to configure the path switch from direct path to indirect path as specified in TS 38.401 [4].

If the MUSIM-GapConfig IE is contained in the CU to DU RRC Information IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, decide to use this IE for MUSIM gap configuration or select another one based on the received UEAssistanceInformation IE. If gNB-DU selects a different MUSIM gap configuration from received UEAssistanceInformation IE, then it shall include the selected MUSIM gap information to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

If MUSIM-GapConfig IE is not contained in the CU to DU RRC Information IE, then gNB-DU shall, if supported, send the selected MUSIM gap configuration based on the received UEAssistanceInformation IE, to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message. When MUSIM-GapConfig IE is received, the gNB-CU should use this value.

If the gNB-DU UE Slice Maximum Bit Rate List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported,

-	store and replace the previously provided gNB-DU UE Slice Maximum Bit Rate List, if any, with the new received gNB-DU UE Slice Maximum Bit Rate List;

-	use the received gNB-DU UE Slice Maximum Bit Rate List for the uplink traffic policing for each concerned slice as specified in TS 23.501 [21].

If the Multicast MBS Session Setup List IE or the Multicast MBS Session Remove List IE or both IEs are contained in the UE CONTEXT MODIFICATION REQUEST message the gNB-DU shall, if supported, store and use the information for configuring MBS Session Resources, if applicable.

If the UE Multicast MRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for configuring MBS Session Resources, if applicable. And if the MBS PTP Retransmission Tunnel Required IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Retransmission F1-U tunnel.

If the MBS PTP Forwarding Tunnel Required Information IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Forwarding F1-U tunnel.

If the Management Based MDT PLMN Modification List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, overwrite any previously stored Management Based MDT PLMN List information in the UE context and use the received information to determine subsequent selection of the UE for management based MDT defined in TS 32.422 [29].

If the InterFrequencyConfig-NoGap IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the ul-GapFR2-Config IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the Candidate L1/L2 Cell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider that these cells are candidate cells recommended for L1/L2 triggered mobility and will take them under consideration.

If the L1/L2 Cell List IE is included in the UE CONTEXT MODIFICATION RESPONSE message the gNB-CU will understand that the cells are configured for L1/L2 triggered mobility.

If the L1/L2 Cell Failed To Setup List IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the corresponding Cell(s) for L1/L2 triggered mobility failed to be set up with an appropriate cause value for each Cell failed to setup.

If the Candidate L1/L2 SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of candidate SCells recommended for L1/L2 triggered mobility and will take them under consideration.

If the L1/L2 SCell List IE is included in the UE CONTEXT MODIFICATION RESPONSE message the gNB-CU will understand that the SCells are configured for L1/L2 triggered mobility.

If the L1/L2 SCell Failed To Setup List IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) for L1/L2 triggered mobility failed to be set up with an appropriate cause value for each SCell failed to setup.

Editor’s note: The Candidate L1/L2 SCell To Be Setup List IE, the L1/L2 SCell List IE and the L1/L2 SCell Failed To Setup List IE are FFS

#### 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the UE context can be successfully performed, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. If the Conditional Intra-DU Mobility Information IE was included in the UE CONTEXT MODIFICATION REQUEST message and set to "CHO-initiation", the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT MODIFICATION REQUEST message, it shall reply with the UE CONTEXT MODIFICATION FAILURE message.

If the Conditional Intra-DU Mobility Information IE was included and set to "CHO-initiation" or "CHO-replace" but the SpCell ID IE was not included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

If the gNB-DU is not able to accept any of the cells in the Candidate L1/L2 Cell To Be Setup List IE, it shall reply with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

#### 8.3.4.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT MODIFICATION REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT MODIFICATION REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore those non-associated candidate cells.

In case of "CHO-replace" when the Target gNB-DU UE F1AP ID IE is included, if the candidate cell in the SpCell ID IE included in the UE CONTEXT MODIFICATION REQUEST message was not prepared using the same UE-associated signalling connection, the gNB-DU shall ignore this candidate cell.

<<<<<< NEXT CHANGE >>>>>>

### 8.3.5	UE Context Modification Required (gNB-DU initiated)

#### 8.3.5.1	General

The purpose of the UE Context Modification Required procedure is to modify the established UE Context, e.g., modifying and releasing radio bearer resources, or sidelink radio bearer resources or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change. The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: UE Context Modification Required procedure. Successful operation

The F1AP UE CONTEXT MODIFICATION REQUIRED message is initiated by the gNB-DU.

The gNB-CU reports the successful update of the UE context in the UE CONTEXT MODIFICATION CONFIRM message.

For a given bearer for which PDCP CA duplication was already configured, if two DL UP TNL Information IEs are included in UE CONTEXT MODIFICATION REQUIRED message for a DRB, the gNB-CU shall include two UL UP TNL Information IEs in UE CONTEXT MODIFICATION CONFIRM message. The gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2], and the first UP TNL Information IE is still for the primary path.

For a given bearer for which PDCP CA duplication was already configured, if one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT MODIFICATION REQUIRED message for a DRB, the gNB-CU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT MODIFICATION CONFIRM message. The gNB-CU and gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT MODIFICATION REQUIRED, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION CONFIRM message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], after completion of UE Context Modification Required procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION CONFIRM message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT MODIFICATION CONFIRM message shall be ignored.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION CONFIRM message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], after completion of UE Context Modification Required procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION CONFIRM message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

If the DU to CU RRC Information IE is included in the UE CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall perform RRC Reconfiguration as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

If the UE CONTEXT MODIFICATION CONFIRM message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the UE CONTEXT MODIFICATION REQUIRED message contains the RLC Status IE, the gNB-CU shall assume that RLC has been reestablished at the gNB-DU and may trigger PDCP data recovery.

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall consider that only the resources reserved for the candidate cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-CU UE F1AP ID IE are about to be released by the gNB-DU.

If the PC5 RLC Channel Required to be Modified List IE or the PC5 RLC Channel Required to be Released List IE is included in the UE CONTEXT MODIFICATION REQUIRED message and the F1AP-IDs is associated with a U2N Relay UE, the PC5 RLC Channel Required to be Modified List IE or the PC5 RLC Channel Required to be Released List shall include the Remote UE Local ID and correspondingly, the PC5 RLC Channel Modified Item IEs in the UE CONTEXT MODIFICATION CONFIRM message shall include the Remote UE Local ID IE.

If the UE Multicast MRB Required to Be Modified List IE is included in the UE CONTEXT MODIFICATION REQUIRED message containing for an MRB the MRB type reconfiguration IE set to "true" the gNB-CU shall take the MRB Reconfigured RLC mode IE into account to reconfigure the UE and to decide whether to request a PDCP status report as specified in TS 38.300 [6] and include the MBS PTP Retransmission Tunnel Required IE in the UE Multicast MRB Confirmed to Be Modified Item IEs IE.

If the Target L1/L2 Cell ID IE is included in the UE CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall consider that it indicates execution of L1/L2 triggered mobility to the target cell and shall include it as the Accepted L1/L2 Cell ID IE in the UE CONTEXT MODIFICATION CONFIRM message. (More details are FFS.)

Interaction with the Multicast Distribution Setup procedure:

If the UE CONTEXT MODIFICATION CONFIRM message contains for an MRB the MBS PTP Retransmission Tunnel Required IE in the UE Multicast MRB Confirmed to Be Modified Item IEs IE the gNB-DU shall, if supported, trigger the Multicast Distribution Setup procedure to setup requested F1-U resources, if applicable.

#### 8.3.5.2A	Unsuccessful Operation

Figure 8.3.5.2A-1: UE Context Modification Required procedure. Unsuccessful operation.

In case none of the requested modifications of the UE context can be successfully performed, the gNB-CU shall respond with the UE CONTEXT MODIFICATION REFUSE message with an appropriate cause value.

#### 8.3.5.3	Abnormal Conditions

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT MODIFICATION REQUIRED message were not prepared using the same UE-associated signaling connection, the gNB-CU shall ignore those non-associated candidate cells.

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SDT RLC Bearer Configuration | O |  | OCTET STRING | RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [8] | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID of the DL tunnel corresponding to such UL tunnel | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuraiton 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT RLC Bearer Configuration | O |  | OCTET STRING | RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [8] | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN ID 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| Trace Activation | O |  | 9.3.1.88 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS Information | M |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| Configured BAP Address | O |  | 9.3.1.111 | The BAP address configured for the corresponding child IAB-node. | YES | reject |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier |  |  | 9.3.1.121 |  | - |  |
| >>RLC mode | M |  | 9.3.1.27 |  | - |  |
| Conditional Inter-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | - | - |
| >Target gNB-DU UE F1AP ID | C-ifCHOmod |  | 9.3.1.5 | Allocated at the target gNB-DU | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.151 |  | YES | ignore |
| Serving NID | O |  | 9.3.1.155 |  | YES | reject |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| F1-C Transfer Path NRDC | O |  | 9.3.1.228 |  | YES | reject |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC,no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertaken or not. | YES | ignore |
| SCG Activation Request | O |  | 9.3.1.233 |  | YES | ignore |
| Old CG-SDT Session Info | O |  | CG-SDT Session Info 9.3.1.261 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.268 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | This IE applies only if the UE is authorized for 5G ProSe services, and only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| Uu RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | M |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 | This IE is not used in this version of the specification. |  |  |
| >>CHOICE PC5 RLC Channel QoS Information | M |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| Path Switch Configuration | O |  | 9.3.1.263 |  | YES | ignore |
| gNB-DU UE Slice Maximum Bit Rate List | O |  | 9.3.1.271 | The Slice Maximum Bit Rate List is the maximum aggregate UL bit rate per slice, to be enforced by the gNB-DU, if feasible. This IE is ignored if the DRB to Be Setup List IE is not present. | YES | ignore |
| Multicast MBS Session Setup List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has joined. | YES | reject |
| UE Multicast MRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |
| >>MBS PTP Forwarding Tunnel Required Information | O |  | MRB Progress Information 9.3.2.12 |  | - |  |
| >>Source MRB ID | O |  | 9.3.1.224 MRB ID | In case of inter-DU handover, indicates the MRB ID provided to the UE in the source cell. | - | ignore |
| Candidate L1/L2 Cell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Candidate L1/L2 Cell To Be Setup Item IEs |  | 1 .. <maxnoofL1/L2Cells> |  |  | EACH | ignore |
| >>Candidate L1/L2Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| Candidate L1/L2 SCell To Be Setup List (FFS) |  | 0..1 |  |  | YES | ignore |
| >Candidate L1/L2 SCell to Be Setup Item IEs |  | 1.. <maxnoofL1/L2SCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO (FFS) | O |  | INTEGER (1..64) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofL1/L2Cells | Maximum no. of Cells configured for L1/L2 triggered mobility allowed towards one UE, the maximum value is FFS. |
| maxnoofL1/L2SCells | Maximum no. of SCells configured for L1/L2 mobility triggered allowed towards one UE, the maximum value is FFS. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item Iist |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>> DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| SRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Failed to Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >DRB Failed to Setup Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channels which are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  |  | EACH | ignore |
| >SL DRB Failed To Setup Item IE |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell indicated in the UE CONTEXT SETUP REQUEST message. | YES | reject |
| SCG Activation Status | O |  | 9.3.1.234 |  | YES | ignore |
| Uu RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 | This IE is not used in this version of the specification. |  |  |
| PC5 RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 | This IE is not used in this version of the specification. | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| L1/L2 Cell List |  | 0..1 |  |  | YES | ignore |
| >L1/L2 Cell Item IEs |  | 1 .. <maxnoofL1/L2Cells> |  |  | EACH | ignore |
| >>L1/L2 Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| L1/L2 SCell Setup List |  | 0..1 |  |  | YES | ignore |
| >L1/L2 SCell Setup Item IEs (FFS) |  | 1.. <maxnoofL1/L2SCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO (FFS) | O |  | INTEGER (1..64) |  | YES | ignore |
| L1/L2 SCell Failed To Setup List (FFS) |  | 0..1 |  |  | YES | ignore |
| >L1/L2 SCell Failed to Setup Item |  | 1 .. <maxnoofL1/L2SCells> |  |  | EACH | ignore |
| >>L1/L2 SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| L1/L2 Cell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >L1/L2 Cell Failed to Setup Item |  | 1 .. <maxnoofL1/L2Cells> |  |  | EACH | ignore |
| >>L1/L2 Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofL1/L2Cells | Maximum no. of Cells configured for L1/L2 triggered mobility allowed towards one UE, the maximum value is FFS. |
| maxnoofL1/L2SCells | Maximum no. of SCells configured for L1/L2 triggered mobility allowed towards one UE, the maximum value is FFS. |

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT SRB. | YES | reject |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID of the DL tunnel corresponding to such UL tunnel | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT DRB. | YES | reject |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 |  | YES | ignore |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >>RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>Transmission Stop Indicator | O |  | 9.3.1.209 |  | YES | ignore |
| >>SDT Indicator Modify | O |  | ENUMTERATED (true, false, …) | Indicates SDT DRB or not. | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | Yes | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | M |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Modified List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | O |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels > |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Released Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| IAB Conditional RRC Message Delivery Indication | O |  | ENUMERATED (true, …) | Indicates whether the RRC message within should be withheld. This IE is only applicable if the UE is an IAB-MT, and the gNB-DU is an IAB-DU. | YES | reject |
| F1-C Transfer Path NRDC | O |  | 9.3.1.228 | This IE is only applicable if the UE is an IAB-MT. | YES | reject |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC,no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertaken or not. | YES | ignore |
| SCG Activation Request | O |  | 9.3.1.233 |  | YES | ignore |
| CG-SDT Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.268 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | This IE applies only if the UE is authorized for 5G ProSe services, and only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| Updated Remote UE Local ID | O |  | Remote UE Local ID 9.3.1.267 | This IE indicates the updated Remote UE Local ID for the U2N Remote UE associated with the F1AP-IDs | YES | ignore |
| Uu RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | M |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | O |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to Be Released Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC channel ID | M |  | 9.3.1.266 |  | - |  |
| PC5 RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>CHOICE PC5 RLC Channel QoS Information | M |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  |  |  |
| >>CHOICE PC5 RLC Channel QoS Information | O |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicate the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Released Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| Path Switch Configuration | O |  | 9.3.1.263 |  | YES | ignore |
| gNB-DU UE Slice Maximum Bit Rate List | O |  | 9.3.1.271 | The Slice Maximum Bit Rate List is the maximum aggregate UL bit rate per slice, to be enforced by the gNB-DU, if feasible. | YES | ignore |
| Multicast MBS Session Setup List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has joined. | YES | reject |
| Multicast MBS Session Remove List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has left. | YES | reject |
| UE Multicast MRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |
| >>MBS PTP Forwarding Tunnel Required Information | O |  | MRB Progress Information 9.3.2.12 |  | - |  |
| UE Multicast MRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| SL DRX Cycle List |  | 0..1 |  |  | YES | ignore |
| >SL DRX Cycle Item IEs |  | 1 .. <maxnoofSLdestinations > |  |  | EACH | ignore |
| >>RX UE ID | M |  | BIT STRING (SIZE(24)) | Indicates the destination L2 ID of RX UE associated to this UE. | - |  |
| >>CHOICE SL DRX Information | M |  |  |  | - |  |
| >>>SL DRX Cycle |  |  |  |  |  |  |
| >>>>SL DRX Cycle Length | M |  | ENUMERATED (ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...) | Indicates the desired SL DRX cycle for RX UE associated to this UE. | - |  |
| >>>No SL DRX |  |  |  |  | - |  |
| >>>>SL DRX configuration indicator | M |  | ENUMERATED(release,...) |  | - |  |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.3.1.274 |  | YES | ignore |
| SDT Bearer Configuration Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Candidate L1/L2 Cell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Candidate L1/L2 Cell To Be Setup Item IEs |  | 1 .. <maxnoofL1/L2Cells> |  |  | EACH | ignore |
| >>Candidate L1/L2 Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| Candidate L1/L2 SCell To Be Setup List (FFS) |  | 0..1 |  |  | YES | ignore |
| >Candidate L1/L2 SCell to Be Setup Item IEs |  | 1.. <maxnoofL1/L2SCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO (FFS) | O |  | INTEGER (1..64) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |
| maxnoofL1/L2Cells | Maximum no. of Cells configured for L1/L2 triggered mobility allowed towards one UE, the maximum value is FFS. |
| maxnoofL1/L2SCells | Maximum no. of SCells configured for L1/L2 triggered mobility allowed towards one UE, the maximum value is FFS. |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| DRB Modified List |  | 0..1 |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>RLC Status | O |  | 9.3.1.69 | Indicates the RLC has been re-established at the gNB-DU. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| SRB Failed to be Setup List |  | 0..1 |  | The List of SRBs which are failed to be established. | YES | ignore |
| >SRB Failed to be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Setup List |  | 0..1 |  | The List of DRBs which are failed to be setup. | YES | ignore |
| >DRB Failed to be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Modified List |  | 0..1 |  | The List of DRBs which are failed to be modified. | YES | ignore |
| >DRB Failed to be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (Not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Associated SCell List | O |  | 9.3.1.77 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| SRB Modified List |  | 0..1 |  |  | YES | ignore |
| >SRB Modified Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channels which are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| BH RLC Channel Modified List |  | 0..1 |  | The list of BH RLC channels which are successfully modified. | YES | ignore |
| >BH RLC Channel Modified Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Modified List |  | 0..1 |  | The list of BH RLC channels whose modification has failed. | YES | ignore |
| >BH RLC Channel Failed to be Modified Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Modified List |  | 0..1 |  | The List of SL DRBs which are successfully modified. | YES | ignore |
| >SL DRB Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  | The List of SL DRBs which are failed to be setup. | YES | ignore |
| >SL DRB Failed To Setup Item |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Failed To be Modified List |  | 0..1 |  | The List of SL DRBs which are failed to be modified. | YES | ignore |
| >SL DRB Failed To be Modified Item |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell indicated in the UE CONTEXT MODIFICATION REQUEST message. | YES | reject |
| SCG Activation Status | O |  | 9.3.1.234 |  | YES | ignore |
| Uu RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  |  |  |
| Uu RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Uu RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SDT Bearer Configuration Info | O |  | 9.3.1.277 |  | YES | ignore |
| L1/L2 Cell List |  | 0..1 |  |  | YES | ignore |
| >L1/L2 Cell Item IEs |  | 1 .. <maxnoofL1/L2Cells> |  |  | EACH | ignore |
| >>L1/L2 Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| L1/L2 SCell Setup List (FFS) |  | 0..1 |  |  | YES | ignore |
| >L1/L2 SCell Setup Item IEs |  | 1.. <maxnoofL1/L2SCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO (FFS) | O |  | INTEGER (1..64) |  | YES | ignore |
| L1/L2 SCell Failed To Setup List (FFS) |  | 0..1 |  |  | YES | ignore |
| >L1/L2 SCell Failed to Setup Item |  | 1 .. <maxnoofL1/L2SCells> |  |  | EACH | ignore |
| >>L1/L2 SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| L1/L2 Cell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >L1/L2 Cell Failed to Setup Item |  | 1 .. <maxnoofL1/L2Cells> |  |  | EACH | ignore |
| >>L1/L2 Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofL1/L2Cells | Maximum no. of Cells configured for L1/L2 triggered mobility allowed towards one UE, the maximum value is FFS. |
| maxnoofL1/L2SCells | Maximum no. of SCells configured for L1/L2 triggered mobility allowed towards one UE, the maximum value is FFS. |

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.2.10	UE CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-DU to request the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Required to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB Required to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DL UP TNL Information to be setup List |  | 0..1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>RLC Status | O |  | 9.3.1.69 | Indicates the RLC has been re-established at the gNB-DU. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| SRB Required to be Released List |  | 0..1 |  |  | YES | reject |
| >SRB Required to be Released List Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| DRB Required to be Released List |  | 0..1 |  |  | YES | reject |
| >DRB Required to be Released List Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| BH RLC Channel Required to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel Required to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| SL DRB Required to Be Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB Required to Be Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Required to be Released List |  | 0..1 |  |  | YES | reject |
| >SL DRB Required to be Release Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| Uu RLC Channel Required to be Modified List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel Required to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Required to be Released List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel Required to be Released Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| PC5 RLC Channel Required to be Modified List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel Required to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Required to be Released List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel Required to be Released Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| UE Multicast MRB Required to Be Modified List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Required to Be Modified Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MRB type reconfiguration | O |  | ENUMERATED (true, ...) |  | - |  |
| >>MRB Reconfigured RLC mode | C-ifMRBTypeReconf |  | MRB RLC Configuration 9.3.1.275 |  | - |  |
| UE Multicast MRB Required to Be Released List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Required to Be Released Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| Target L1/L2 Cell ID (FFS) | O |  | NR CGI 9.3.1.12 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 32. |

| Condition | Explanation |
| --- | --- |
| ifMRBTypeReconf | This IE shall be present if the MRB Type Reconfiguration IE is present. |

<<<<<< NEXT CHANGE >>>>>>

#### 9.2.2.11	UE CONTEXT MODIFICATION CONFIRM

This message is sent by the gNB-CU to inform the gNB-DU the successful modification.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range |  | IE type and reference |  | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  |  | YES | reject |
| gNB-CU UE F1AP ID | M |  |  | 9.3.1.4 |  |  | YES | reject |
| gNB-DU UE F1AP ID | M |  |  | 9.3.1.5 |  |  | YES | reject |
| Resource Coordination Transfer Container | O |  |  | OCTET STRING |  | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DRB Modified List |  | 0..1 |  |  |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  |  |  | EACH | ignore |
| >>DRB ID | M |  |  | 9.3.1.8 |  |  | - |  |
| >>UL UP TNL Information to be setup List |  | 1 |  |  |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  |  |  | - |  |
| >>>>UL UP TNL Information | M |  |  | UP Transport Layer Information 9.3.2.1 |  | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  |  | 9.3.1.114 |  |  | YES | ignore |
| >>>>DRB Mapping Info | O |  |  | Uu RLC Channel ID 9.3.1.266 |  | This IE is not used in this version of the specification. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  |  | UP Transport Layer Information 9.3.2.1 |  | gNB-DU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  |  | 9.3.1.114 |  |  | YES | ignore |
| RRC-Container | O |  |  | 9.3.1.6 |  | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Criticality Diagnostics | O |  |  | 9.3.1.3 |  |  | YES | ignore |
| Execute Duplication | O |  |  | ENUMERATED (true, ...) |  | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| Resource Coordination Transfer Information | O |  |  | 9.3.1.73 |  |  | YES | ignore |
| SL DRB Modified List |  |  | 0..1 |  |  |  | YES | reject |
| >SL DRB Modified Item IEs |  |  | 1 .. <maxnoofSLDRBs> |  |  |  | EACH | reject |
| >>SL DRB ID | M |  |  | 9.3.1.120 |  |  | - |  |
| Uu RLC Channel Modified List |  |  | 0..1 |  |  |  | YES | reject |
| >Uu RLC Channel Modified Item IEs |  |  | 1 .. <maxnoofUuRLCChannels> |  |  |  | - |  |
| >>Uu RLC Channel ID | M |  |  | 9.3.1.266 |  |  | - |  |
| PC5 RLC Channel Modified List |  |  | 0..1 |  |  |  | YES | reject |
| >PC5 RLC Channel Modified Item IEs |  |  | 1 .. <maxnoofPC5RLCChannels> |  |  |  |  | - |
| >>PC5 RLC Channel ID | M |  |  | 9.3.1.265 |  |  | - |  |
| >>Remote UE Local ID | O |  |  | 9.3.1.267 |  |  | - |  |
| UE Multicast MRB Confirmed to Be Modified List |  |  | 0..1 |  |  |  | YES | reject |
| >UE Multicast MRB Confirmed to Be Modified Item IEs |  |  | 1 .. <maxnoofMRBsforUE> |  |  |  | EACH | reject |
| >>MRB ID | M |  |  | 9.3.1.224 | MRB ID for the UE. |  | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  |  | 9.3.2.10 |  |  | - |  |
| Accepted L1/L2Cell ID (FFS) | O |  |  | NR CGI 9.3.1.12 |  |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |

<<<<<< END OF CHANGE >>>>>>