3GPP TSG-RAN WG3 Meeting #118	R3-226300

Toulouse, FR, 14-18 Nov, 2022

Title: 	(TPs for SON BLCRs for TS 38.473 and TS 38.423): RACH optimisation

Source: 	Huawei

Agenda item:	10.2.3

Document Type:	Other

- Introduction

In RAN3#117bis-e meeting, the following achieved agreement and identified scope are listed as below:

RACH partitioning:

RA report is enhanced to include feature combination related information. FFS whether it should be the feature/feature combination UE intends to initiate and/or feature/feature combination UE finally successfully initiates.

To be continued next meeting on the following information:

-Feature priorities configured by network, which is the determining factor for selection of RACH partition for feature combination;

-RA resources configured with feature indicator(s), which is the pool of RACH resources for RACH feature of feature combination;

-Configuration info per RA attempt or the time from RA attempt to reporting

-The actual information per RA attempt, such as SSB RSRP and MSG3 RSRP

RACH report retrieval:

- WA: RAN3 works on the network based solution for RACH report retrieval, i.e., gNB-DU indicates to gNB-CU about RACH occurrence

- WA: SN should indicate the potential availability of RA report to the MN, MN can fetch the RA report and transfer it to SN.

- Send LS to RAN2 with RAN3 assumption and ask RAN2 to provide feedback on UE based solution

SN RACH report in MR-DC

RAN3 has supported EN-DC, (NG)EN-DC, and NR-DC scenarios in Rel-17. No further work will be triggered in RAN3.

If RAN2 decides to support SN RA Report for EN-DC and (NG)EN-DC, UE should report the PScell identity outside the RACH report to help the network forward the report to the correct node.

Agree the Reply LS to RAN2 captures the following:

-     RAN3 has supported EN-DC, (NG)EN-DC, and NR-DC scenarios in Rel-17. No further work will be triggered by RAN3 in Rel-18

-     If RAN2 decides to support SN RA Report for EN-DC and (NG)EN-DC, UE should report the PScell identity outside the RACH report to help the network forward the report to the correct node

On the other hand, some progress is made in last RAN2#119bis-e meeting as below:

Agreements:

For RACH report about RACH partitioning information

1	Agree to add the following parameters into RACH report for RACH partitioning:

-	Feature or the combination of features that triggered the RACH

-	Used feature combination

In this paper, we discuss how to proceed on RACH optimization in the three aspects: RACH partitioning, RACH report retrieval, and SN RACH report in MR-DC.

- Discussion

  - RACH partitioning

In Rel-17, following features are discussing separated RACH resource with different objectives:

- SDT: to request large resource in Msg3/MsgA PUSCH

- Coverage enhancement: to request Msg3 PUSCH repetition

- RAN slicing: to guarantee the RACH resource to sensitive slice or slice group

- RedCap: to indicate RedCap UE in Msg1/MsgA

Given that these features can be operated in combinational manner, the common RA operation for feature combination is also supported by introducing a feature called RACH partition. RACH partitioning specifies a framework for RACH resource configuration per feature or feature combination to enable early identification of a particular feature or feature combination on network side. Specifically, random access resources are configured with one or multiple feature indications at the network side, which indicates a feature or feature combination applicable to the random access resources. Besides, RACH partitioning also specifies the selection of the set of RACH resources based on feature prioritization assigned in featurePriorities at the UE side when a feature or feature combination maps to more than one RACH resources

However, a mismatch might occur between the feature/feature combination applicable to the selected RACH resource and the feature/feature combination the UE intended to initiate via RA procedure.

For instance, the available sets of Random Access resources broadcasted in SIB1 are applicable to feature or feature combination which includes RedCap+SDT+ Slice group1, RedCap, RedCap+CovEnh, SDT+CovEnh, and Slice group1+SDT. In addition, the feature priorities broadcasted in SIB1 are RedCap>Slice group1> SDT>CovEnh.

When feature combination RedCap+SDT+CovEnh is initiated by the UE , i.e., triggering the RACH procedure, the UE selects the RA resource among the available sets of RA resource based on feature prioritization since there is no available RA resource indicated for feature combination RedCap+SDT+CovEnh.

The selection process in the UE is illustrated in Figure.1.

- The UE firstly selects the RA resources which apply to RedCap with highest priority, then SDT with second highest priority and finally CovEnh.

- As a result, UE identifies the RA resource indicated for RedCap+SDT+Slice group1 and finally RA procedure for RedCap+SDT.

- In this example, the intended feature/feature combination is RedCap+SDT+CovEnh and the feature/feature combination of RedCap+SDT+Slice group1 is the selected RACH resource by the UE.

- Then the intersection of those two feature combinations is the feature/feature combination applicable to this RA procedure, which is a subset of the intended feature/feature combination.

The mismatch occurs mainly due to improper RA resource segmentation for feature/feature combination and inappropriate feature priorities, which needs adjustment in RACH optimization.

Proposal 1: Send a LS to RAN2 to report the feature priorities in RACH report.

Figure 1: Selection of RA resources based on feature prioritization

RAN2 has agreed “used feature” which originates in [1]. The meaning of  “used feature” is not very clear, which needs further clarification.

For example, for the intended feature combination 1, 2, and 3, the UE based on the feature priorities selects the RACH resource for feature combination 2,3 and 4 (selected feature(s)) and finally use the selected RACH resource to initiate the RA procedure for feature 2 and 3.

In our understanding, we prefer that the “used feature” is to indicate the selected feature(s), i.e., the feature applicable to the used/selected RACH resource.

In our understanding, the motivation is to allow network to assign RACH resources to RACH feature(s) in the most efficient way, the network needs to know the gap between the preferred feature(s) and the feature(s) which RACH resource is configure with. Besides, the network can defer the feature/feature combination which uses the RA procedure by intersecting the intended ones and the selected ones.

Proposal 2: To send a LS to RAN2 for clarification on the meaning of “used feature” whether it refers to the selected feature(s) (feature combination 2,3 and 4 in above example) or the used feature(s) by the UE (feature combination 2 and 3) .

The UE performs RA procedure via selected RACH resource. The RA report shall log the actual used RACH resource configuration, attempted beam indexes, and the measured RSRP during the RA procedure for RACH feature/feature combination with the existing IE in RA report.

We would like to clarify that the configuration info is recorded in granularity of RA procedure and only the attempt info is recorded in granularity of RA attempt.

Proposal 3:  The RA report shall log the used RACH resource configuration, attempted beam indexes, and the measured RSRP for RACH feature/feature combination with the same granularity in existing RA report.

  - RACH report retrieval

When the UE generates RACH reports, the network needs to be aware that these RACH reports are available so that network can decide whether to fetch these RACH reports from UE. Fetching RACH reports is also referred to as RACH reports retrieval. For network-based solution, a trigger from a gNB-DU to a gNB-CU is introduced when a RACH event is observed by gNB-DU. This was discussed in Rel-17 in RAN3, but was not agreed. Since there is no conclusion for this issue in RAN2, RAN3 should reconsider the network-based solution

For support of network-based solution, when it comes to SN RA report retrieval, a new XNA/X2 signalling in MR-DC case, since the SN needs to notify MN of the availability of RACH report, so that MN knows when to fetch SN RACH report from UE.

Since no progress has been made in RAN2, we continue to ask RAN2 to consider the option1 and discuss option 2 in RAN3.

Proposal 4:	To consider network-based solution for RACH report retrieval.

- Proposal 5:	To proceed the TPs for TS 38.473 and TS 38.423 BL CR in section 6.

4. Conclusion

In this contribution, we propose:

Proposal 1: Send a LS to RAN2 to report the feature priorities in RACH report.

Proposal 2: To send a LS to RAN2 for clarification on the meaning of “used feature” whether it refers to the selected feature(s) (feature combination 2,3 and 4 in above example) or the used feature(s) by the UE (feature combination 2 and 3) .

Proposal 3:  The RA report shall log the used RACH resource configuration, attempted beam indexes, and the measured RSRP for RACH feature/feature combination with the same granularity in existing RA report.

Proposal 4:	To consider network-based solution for RACH report retrieval.

- Proposal 5:	To proceed the TPs for TS 38.473 and TS 38.423 BL CR in section 6.

5. Reference

- R2-2209567	Discussion on RACH report enhancement for RACH partitioning	vivo	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

4 Annex – draft LS

3GPP TSG-RAN WG3 Meeting #118	TDoc <TDoc#>

Toulouse, FR, 14-18 Nov, 2022

Title:	[Draft] LS on RACH enhancement regarding RACH partitioning

Response to:

Release:	Rel-18

Work Item:	NR_ENDC_SON_MDT_enh2-Core

Source:	Huawei [will be RAN3]

To:	RAN2

Cc:

Contact person:	Hongzhuo Zhang

Zhanghongzhuo(at)Huawei(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 discussed the RACH partitioning optimization and agreed to record the following information in RA report.

- the feature priorities.

- the used RACH resource configuration, attempted beam indexes, and the measured RSRP for RACH feature/feature combination with the same granularity in existing RA report.

Furthermore, RAN3 would like to ask RAN2 to clarify the meaning of “used feature” whether it refers to the selected feature(s) or the actual used feature(s) by the UE during RA procedure.

2	Actions

To RAN2 group

ACTION:

RAN3 respectfully asks RAN2 to take the above into account and provide feedbacks.

3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#119		2023-02-27 - 2023-03-03		Athens, GR

RAN3#119-bis-e		2023-04-17 - 2023-04-26		Electronic

RAN3#120		2023-05-22 - 2023-05-26		Korea, KR

5. Annex A-Text Proposals for SON BL CR for TS 38.473

Start of changes

8	F1AP procedures

8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| GNB-DU RESOURCE COORDINATION | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| RACH Report Available Indication | RACH REPORT AVAILABLE INDICATION |
| RACH Report Indication | RACH REPORT INDICATION |

Start of the next change

8.3.X		RACH Report Available Indication

8.3.X.1		General

This procedure is initiated by the gNB-DU to indicate the RACH Report Available to gNB-CU.

The procedure uses UE-associated signalling.

8.3.X.2		Successful Operation

Figure 8.3.X.2-1: RACH REPORT AVAILABLE INDICATION procedure.

The gNB-DU initiates the procedure by sending the RACH REPORT AVAILABLE INDICATION message to the gNB-CU.

8.3.X.3	Abnormal Conditions

Not applicable.

Start of the next change

8.3.Y		RACH Report Indication

8.3.Y.1		General

This procedure is initiated by the gNB-CU to send the UE RACH report to gNB-DU.

The procedure uses UE-associated signalling.

8.3.Y.2		Successful Operation

Figure 8.3.Y.2-1: RACH REPORT INDICATION procedure.

The gNB-CU initiates the procedure by sending the RACH REPORT INDICATION message to the gNB-DU.

8.3.Y.3	Abnormal Conditions

Not applicable.

Start of the next change

9.2.2.X		RACH Report Available Indication

This message is sent by the gNB-DU to notify the RACH occurrence to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |

Start of the next change

9.2.2.Y		RACH Report Indication

This message is sent by the gNB-CU to provide information about the UE RACH report to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| UE RACH Report | M |  | FFS (pending to RNA2 discussion) |  | YES | ignore |

End of changes

6. Annex B-Text Proposals for SON BL CR for TS 38.423

8.4.12	Access And Mobility Indication

8.4.12.1	General

The purpose of the Access and Mobility Indication procedure is to transfer Access and Mobility related information between NG-RAN nodes.

8.4.12.2	Successful Operation

Figure 8.2.12.2-1: Access And Mobility Indication, successful operation

NG-RAN node1 initiates the procedure by sending the ACCESS AND MOBILITY INDICATION message sent to NG-RAN node2.

If the Successful HO Report IE is included in the ACCESS AND MOBILITY INDICATION message, NG-RAN node2 may use it to optimize handover configurations.

If the RACH Report Available Indication IE is included in the ACCESS AND MOBILITY INDICATION message, NG-RAN node2 may consider to request RACH report from UE.

8.4.12.3	Abnormal Conditions

Not applicable.

9.1.3.25	ACCESS AND MOBILITY INDICATION

This message is sent by NG-RAN node1 to transfer access and mobility related information to NG-RAN node2.

Direction: NG-RAN node 1  NG-RAN node 2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | YES | ignore |
| >>UE Assistant Identifier | O |  | NG-RAN node UE XnAP ID 9.2.3.16 |  | YES | ignore |
| Successful HO Report List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report List Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | – |  |
| >>Successful HO Report Container | O |  | OCTET STRING | SuccessHO-Report-r17 IE as defined in subclause 6.2.2 in TS 38.331 [10]. | – |  |
| RACH Report Available Indication | O |  |  | RACH Report Available Indication-r18 IE as defined in subclause 9.2.2.X in TS 38.473 | – | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |