3GPP TSG-RAN WG3 Meeting #118	R3-226321

Toulouse, FR, 14-18 Nov, 2022

Title: 	(TPs to TS 38.300, TS 38.413 BL CRs) MBS reception in RAN sharing scenario

Source: 	Huawei, CBN

Agenda item:	15.2

Document Type:	other

# 1. Introduction

In RAN3#117bis-e meeting, there were agreements about MBS reception in RAN sharing scenarios:

RAN3 believes that Solution(s) which assume MOCN RAN nodes can identify the same MBS service based on the information provided by 5GC should be supported. The following principles should be considered when discussing solutions on which information should be provided from 5GC: Principle1: The solution provided by RAN3 for RAN sharing should not have impact on Rel-17 UE and Rel-17 gNB. Principle3: The identity providing a reference to the same MBS service should not depend on the momentarily participating operators considering of the possibility for sharing operators leaving or entering the common ongoing session from time to time, that’s to say the solution should be robust to cover the cases that the shared PLMNs start and stop the MBS session at the same time and start and stop the MBS session at the different time. Principle4: It could not be assumed that MB-SMF/AF/MBSF is aware which NG-RAN node or which cell within a NG-RAN node is shared since currently NG-RAN node only inform AMF of the supported PLMN and no coordination with MB-SMF/AF/MBSF. RAN3 think that a solution based on information received from 5GC is desired. Solutions 2,7,24 and 29 can work, while solutions 2, 7 with majority support in RAN3. Solution 24 brings configuration efforts which may have flexibility and scalability issue in case MBS services are dynamically added or removed.

In this contribution, we will further analysis on the support of MBS reception in RAN sharing scenarios focusing on the work on the broadcast service.

# 2. Discussion

Broadcast session management in RAN sharing scenario

- MBS session ID

In RAN3 #117bis meeting, there was an agreement that “RAN3 believes that Solution(s) which assume MOCN RAN nodes can identify the same MBS service based on the information provided by 5GC should be supported.” SA2 has proposed various solutions as follows, and we will analyse corresponding impacts of different solutions on NG interface and Uu interface.

Solution #24: This solution relies on RAN configuration does not need any new parameter as proposed in existing solutions but the service-id part of the TMGI of the RAN sharing partners that corresponds to the same content is configured in RAN.

For solution #24, RAN node is configured with specific service-ids or service-id ranges corresponding to the same content for each of their RAN sharing partners. Although this solution can work, RAN3 has agreed that “Solutions 2,7,24 and 29 can work, while solutions 2, 7 with majority support in RAN3.” and “Solution 24 brings configuration efforts which may have flexibility and scalability issue in case MBS services are dynamically added or removed.” Thus, we propose to confirm solution #24 shall not be supported due to the limited flexibility and scalability with much efforts.

Proposal 1: Solution #24 shall not be supported due to the limited flexibility and scalability with much efforts.

Solution #29: The present solution aims to use a single TMGI to minimize the required updates and to avoid that multiple TMGIs for the same multicast data are broadcasted over the radio to save bandwidth.

In the last meeting, for the case that “FFS whether it would be regarded as an abnormal case for a Rel-17 gNB when receiving a foreign TMGI”, there is no explicit requirement in the existing specification. We assume that it is up to the NG-RAN node implementation that whether providing services for the foreign TMGI or not. Thus, it is possible for some Rel-17 gNBs cannot provide the broadcast service for the foreign TMGI. Based on this, for solution #29, it is unclear whether the MOCN TMGI is workable for all of Rel-17 UEs and gNBs, since the gNB may not allocated the session resource for the TMGI with inconsistent PLMN ID, and there even will be problem for the service continuity for the UE considering the non-homogeneous support of MOCN. Therefore, it is preferred to not use the foreign TMGI to avoid potential compatibility issue.

Proposal 2: Solution #29 shall not be supported due to the potential compatibility issue for Rel-17 gNBs and UEs.

Solution #2: In general the proposal is based on the additional identifier (i.e. identifier of the broadcast MBS service) provided by AF during MBS session Create procedure. The identifier of the broadcast MBS service is non-PLMN specific, which would be included and the same when AF sends requests for establishing the broadcast MBS session for the same broadcast MBS service to different PLMNs. The Identifier is in the form of IP addresses, which is the target IP address and optionally includes the source IP address.

Solution #7: This solution utilizes the associated session identifier (e.g. SSM used by AF) to be the identifier to associate broadcast MBS sessions from different CNs which transmitting the same content. NG-RAN establishes the user planes for the first broadcast MBS session it receives. The NG-RAN delivers the packets received from the established user plane over the air. For the other broadcast MBS sessions which are associated with the broadcast MBS session, the NG-RAN creates the broadcast MBS session contexts, advertises the TMGIs, but does not establish the user planes. Another option is to let AF select one allocated TMGI as the associated session identifier when creating MBS sessions towards all CNs. In this case, the NG-RAN includes only this selected TMGI in radio interface which points to the configuration, regardless of whether the NG-RAN is dedicated or MOCN shared. AF also includes only this selected TMGI in the service announcement, so that UE use it to receive the broadcast MBS session data.

Based on the agreement in last RAN3 meeting, solution #2 and #7 have majority supports, therefore we prefer to continue the discussion and focus on these two solutions.

For solution #7, it can be observed that there are two options: 1) the first option: SSM as the associated identifier; 2) the second option: use a selected TMGI as the associated identifier. Based on this, the first option of solution #7 is similar with solution #2 using different TMGIs with same IP address provided by 5GC, which has no impact on Rel-17 gNBs and UEs. The only difference between solution#2 and solution#7 option 1 is, the additional identifier used in solution#2 is the target IP address and optionally the source IP address, and solution #7 uses the identifier e.g. SSM, which consists the target IP address and the source IP address as defined in TS 23.247.

From the aspect of Uu interface, solution #2 uses different TMGIs of each partner in sharing NG-RAN node, which has no impact on Rel-17 UEs. However, for the second option of solution #7 (use a selected TMGI as the associated identifier), the NG-RAN only uses the selected TMGI on air interface, which has the same problem with solution #29 on using foreign TMGI.

Proposal 3: The second option of solution #7 (the selected TMGI as associated identifier) has the same problem of solution #29.

From our view, CN shall ensure the same target IP address for the same MBS service even if the service may be provided by different AFs which have different source IP address. Therefore, the additional identifier consisting of the target IP address and optionally the source IP address should be supported, while the details is pending to SA2. From RAN3 point of view, the NGAP specification can introduce the following MBS RAN Sharing Identifier IE into the MBS Session ID IE:

9.3.2.X	MBS RAN Sharing Identifier [FFS]

This IE denotes the MBS RAN Sharing Identifier of the MBS service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target IP Address | M |  | Transport Layer Address 9.3.2.4 |  |
| Source IP Address | O |  | Transport Layer Address 9.3.2.4 |  |

Proposal 4: NG-RAN should identify the same MBS service by using different TMGIs with MBS RAN Sharing Identifier, i.e. target IP address and source IP address.

- S-NSSAI and MBS QoS Flows

For the different sessions transferring the same content for different PLMNs, the S-NSSAIs provided by different PLMNs will be probably inconsistent. In such case, the NG-RAN node can transmit the data of one selected PLMN to UEs of different PLMNS using the S-NSSAI which indicated in the signalling of the selected PLMN. In the last meeting, S-NSSAIs issue have been initially discussed and most companies has agreed that it is up to the NG-RAN node implementation on how to use them to transmit data. Although few companies consider the problem of the coordination of S-NSSAIs among different PLMNs, which will be considered by SA2, anyway RAN3 needs no efforts to do.

In the last meeting, RAN3 has agreed that “The identity providing a reference to the same MBS service should not depend on the momentarily participating operators considering of the possibility for sharing operators leaving or entering the common ongoing session from time to time, that’s to say the solution should be robust to cover the cases that the shared PLMNs start and stop the MBS session at the same time and start and stop the MBS session at the different time”. Based on the discussion in last meeting, for the case that the shared PLMNs start and stop the MBS session at the same time, homogeneous handling among NG-RAN nodes is beneficial by selecting the highest demanding QoS flow parameters. For the case that the selected QoS parameters depends on the order of them which received by the NG-RAN node from different PLMNs, it may be more feasible. Anyway, the different ways to solve this issue are both the implementation for NG-RAN node, with major supported in last meeting.

Proposal 5: the NG-RAN node may receive inconsistent S-NSSAIs or QoS parameters for the same service from different PLMNs, it is up to the NG-RAN node implementation on how to use them.

- MBS Service Area

In the last meeting, RAN3 also has FFS that “Any network sharing mechanism defined in Rel-18, it is only for the overlapped MBS service area.” So we can easily observe that for area limited local broadcast sessions,  the Rel-18 optimized mechanism for RAN sharing scenario can be performed by the NG-RAN node in the overlapped area.

Proposal 6: For area limited local broadcast sessions, the Rel-18 optimized mechanism for RAN sharing scenario can be performed by the NG-RAN node in the overlapped area.

In the last meeting, RAN3 also has FFS that “for location dependent broadcast service, NG-RAN node need to recognize the same area with different MBS area session ID e.g. based on the corresponding cell list/TA list.” We assume that it is up to the NG-RAN node implementation on how to transmit optimized data in the overlapped area regardless of same or different area session IDs.

Proposal 7: For location dependent broadcast sessions, NG-RAN node needs to recognize the same area with same or different MBS area session ID, and the Rel-18 optimized mechanism for RAN sharing scenario can also be performed by the NG-RAN node in the overlapped area(s).

- Shared NG-U tunnel managment

In the NGAP procedure, each NGAP BROADCAST SESSION SETUP REQUEST message may carry TNL information, so the shared NG-U tunnel management needs to be discussed. In the previous meetings, several options were discussed as follows:

- Option 1-1: establish the NG-U tunnels for each session for different PLMNs

- Option 1-2: establish only one NG-U tunnel for multiple session from different PLMNs

- Option 1-3: establish one primary NG-U tunnel and one backup NG-U tunnel for multiple session from different PLMNs

For option 1-1, the existing tunnel management procedure is reused completely, and need to introduce additional tunnel management procedure. As the tunnel is established for each PLMN, the data from different operators are available at the NG-RAN node, it is easier for the NG-RAN node to continue providing the service to the UEs of other PLMNs via the shared NG-U tunnels if the session is to be released for one PLMN, which can improve MBS service continuity and reliability. In addition, as there will be limited number of shared operators, the cost on the multiple shared NG-U tunnel transmission is negligible.

For option 1-2, there will be only one shared NG-U tunnel established, which is beneficial form NG-U resources view point, but is more complex for NG-U tunnel management. For example, it will lead to extra interruption when the PLMN of the only established shared NG-U tunnel triggers the release of the Broadcast session, the NG-RAN node may need to triggered to establish another NG-U tunnel towards other PLMN by using new procedure, during this period, the packet transmission gap may happen which is not good from service continuity point of view and more standard efforts are foreseen.

For option 1-3, if the number of operators is 2, option1-3 is same with option1. In addition, when it is up to the NG-RAN node implementation on how to define the primary and backup tunnel, which seems that there is no difference compared with option 1-1. Otherwise, more RAN3 work needed for who and how to decide the primary and backup tunnel. And if the number is more than 2, option 1-3 cannot still solve the drawbacks of option 1-2, i.e. the extra interaction and interruption when the PLMN of the only established shared NG-U tunnel triggers the release of the Broadcast session.

Proposal 8: The NG-RAN node establish the NG-U tunnels for each broadcast session for different PLMNs in RAN sharing scenario.

F1 impact

In the last meeting, the F1AP impact for this topic has been discussed, we will further analyses on this aspect.

In the existing F1AP specification, the gNB-CU shall provide the mtch-neighbourCell IE and MRB PDCP Config Broadcast IE for each TMGI to the gNB-DU, which is used for gNB-DU to generate MCCH message. Within the MRB PDCP Config Broadcast IE, the gNB-CU indicates the PDCP SN size, header compression parameter and t-Reordering. For shared CU scenario, the shared CU shall use the same set of these parameters for different TMGIs of the same MBS service. And also, the gNB-CU indicate whether the session is provided in the neighbour cells via mtch-neighbourCell IE which refer to the MBS Broadcast Neighbour Cell List IE. For shared CU scenario, the shared CU shall set the bitmap for the different TMGIs individually considering the RAN sharing situation for each PLMN can be different.

On the details for the F1AP design, in the last meeting, most of companies support the gNB-CU transfers the information provided by 5GC which is used to identify the MBS sessions aimed at the same MBS service to gNB-DU. For this solution, the gNB-CU reuse the existing procedure to establish different MBS session contexts by using their individual TMGI with an additional identifier (e.g. the target IP address), which has no impact on Rel-17 gNBs and Rel-17 UEs.On the other hand, it may also be feasible for using the existing procedure once to include a TMGI list. However, this way is more complex than using will multiple procedures, e.g. the neighbour cell list shall be different for the multiple TMGIs of different PLMN.

Therefore, we prefer that the gNB-CU reuse the existing procedure multiple times to establish different MBS session contexts by using their individual TMGI with an additional identifier (e.g. the target IP address).

Proposal 9: For the shared CU scenario, the gNB-CU reuses the existing procedure multiple times to establish different MBS session contexts for different TMGIs of the same MBS service.

Proposal 10: For the shared CU scenario, the gNB-CU provides the same set of parameters including PDCP SN size, header compression parameter and t-Reordering for different TMGIs of the same MBS service.

Proposal 11: the gNB-CU forwards the information provided by 5GC which is used to identify the MBS sessions aimed at the same MBS service to the gNB-DU via existing Broadcast Context Management procedure.

# 3. Proposals

In this contribution, we analysis on the support of MBS reception in RAN sharing scenarios, and get the following proposals:

Down selection of solutions:

Proposal 1: Solution #24 shall not be supported due to the limited flexibility and scalability with much efforts.

Proposal 2: Solution #29 shall not be supported due to the potential compatibility issue for Rel-17 gNBs and UEs.

Proposal 3: The second option of solution #7 (the selected TMGI as associated identifier) has the same problem of solution #29.

Proposal 4: NG-RAN should identify the same MBS service by using different TMGIs with MBS RAN Sharing Identifier, i.e. target IP address and source IP address.

Inconsistent information handling

Proposal 5: the NG-RAN node may receive inconsistent S-NSSAIs or QoS parameters for the same service from different PLMNs, it is up to the NG-RAN node implementation on how to use them.

Proposal 6: For area limited local broadcast sessions, the Rel-18 optimized mechanism for RAN sharing scenario can be performed by the NG-RAN node in the overlapped area.

Proposal 7: For location dependent broadcast sessions, NG-RAN node needs to recognize the same area with same or different MBS area session ID, and the Rel-18 optimized mechanism for RAN sharing scenario can also be performed by the NG-RAN node in the overlapped area(s).

Shared NG-U tunnel management

Proposal 8: The NG-RAN node establish the NG-U tunnels for each broadcast session for different PLMNs in RAN sharing scenario.

F1AP impact

Proposal 9: For the shared CU scenario, the gNB-CU reuses the existing procedure multiple times to establish different MBS session contexts for different TMGIs of the same MBS service.

Proposal 10: For the shared CU scenario, the gNB-CU provides the same set of parameters including PDCP SN size, header compression parameter and t-Reordering for different TMGIs of the same MBS service.

Proposal 11: the gNB-CU forwards the information provided by 5GC which is used to identify the MBS sessions aimed at the same MBS service to the gNB-DU via existing Broadcast Context Management procedure.

Based on the proposals above, the TPs to TS 38.300 and TS 38.413 are provided in Annex A and B in this paper.

# 4. Reference

- RP-222678 Reply LS on the scope of resource efficiency for MBS reception in RAN sharing scenario, contact company: CATT

- S2-2207470 LS on FS_5MBS_Ph2 progress, contact company: Huawei

# Annex A. TP to TS 38.300

-----------------Start of the Changes-------------------

### 16.10.6	Broadcast Handling

// skip unchanged part

#### 16.10.6.X	Support of RAN sharing scenario

RAN sharing is supported for MBS broadcast session. In particular, NG-RAN shall be able to identify the MBS session provided from different operators’ 5GCs aim at the same MBS service based on the information provided by 5GC, and allocate the same PTM radio resource in a shared cell for transmission of the same MBS service provided by different operators.

-----------------End of the Changes-------------------

# Annex B. TP to TS 38.413

-----------------Start of the Changes-------------------

#### 9.3.1.206	MBS Session ID

This IE uniquely identifies the MBS Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TMGI | M |  | OCTET STRING (SIZE(6)) | Encoded as defined in TS 23.003 [23]. |
| NID | O |  | 9.3.3.42 |  |
| MBS RAN Sharing Identifier [FFS] | O |  | 9.3.2.X | This IE is used for RAN nodes to identify the same MBS service. |

-----------------Next of the Change-------------------

#### 9.3.2.X	MBS RAN Sharing Identifier [FFS]

This IE denotes the MBS RAN Sharing Identifier of the MBS service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target IP Address | M |  | Transport Layer Address 9.3.2.4 |  |
| Source IP Address | O |  | Transport Layer Address 9.3.2.4 |  |

-----------------Next of the Change-------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AlternativeQoSParaSetList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-ConfiguredTACIndication,

id-CurrentQoSParaSetIndex,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

id-DataForwardingNotPossible,

id-DataForwardingResponseERABList,

id-DirectForwardingPathAvailability,

id-DL-NGU-UP-TNLInformation,

id-EndpointIPAddressAndPort,

id-EnergySavingIndication,

id-ExtendedPacketDelayBudget,

id-ExtendedRATRestrictionInformation,

id-ExtendedReportIntervalMDT,

id-ExtendedSliceSupportList,

id-ExtendedTAISliceSupportList,

id-ExtendedUEIdentityIndexValue,

id-EUTRA-PagingeDRXInformation,

id-GlobalCable-ID,

id-GlobalRANNodeID,

id-GlobalTNGF-ID,

id-GlobalTWIF-ID,

id-GlobalW-AGF-ID,

id-GUAMIType,

id-IncludeBeamMeasurementsIndication,

id-IntersystemSONInformationRequest,

id-IntersystemSONInformationReply,

id-IntersystemResourceStatusUpdate,

id-LastEUTRAN-PLMNIdentity,

id-LastVisitedPSCellList,

id-LocationReportingAdditionalInfo,

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-M6DelayThreshold,

id-M7ReportAmount,

id-MaximumIntegrityProtectedDataRate-DL,

id-MBS-AreaSessionID,

id-MBS-QoSFlowsToBeSetupList,

id-MBS-QoSFlowsToBeSetupModList,

id-MBS-QoSFlowToReleaseList,

id-MBS-ServiceArea,

id-MBS-SessionFSAIDList,

id-MBS-SessionID,

id-MBS-ActiveSessionInformation-SourcetoTargetList,

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-RANsharingIdentifier,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

id-MDTConfiguration,

id-MicoAllPLMN,

id-NetworkInstance,

id-NGAPIESupportInformationRequestList,

id-NGAPIESupportInformationResponseList,

## Skip some omitted part

MBS-SessionID-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-RANsharingIdentifier	CRITICALITY ignore	EXTENSION MBS-RANsharingIdentifier		PRESENCE optional		},

...

}

MBS-RANsharingIdentifier ::= SEQUENCE {

target-IP-Address				TransportLayerAddress,

source-IP-Address				TransportLayerAddress,																	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MBS-RANsharingIdentifier-ExtIEs} } 		OPTIONAL,

...

}

MBS-RANsharingIdentifier-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionFailedtoSetupList ::= SEQUENCE (SIZE(1.. maxnoofMBSSessions)) OF MBSSessionFailedtoSetupItem

MBSSessionFailedtoSetupItem ::= SEQUENCE {

mBS-SessionID			MBS-SessionID,

mBS-AreaSessionID		MBS-AreaSessionID															OPTIONAL,

cause					Cause,

iE-Extensions			ProtocolExtensionContainer { { MBSSessionFailedtoSetupItem-ExtIEs} }		OPTIONAL,

...

}

## Skip some omitted part

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AllowedNSSAI											ProtocolIE-ID ::= 0

id-AMFName												ProtocolIE-ID ::= 1

id-AMFOverloadResponse									ProtocolIE-ID ::= 2

id-AMFSetID												ProtocolIE-ID ::= 3

id-AMF-TNLAssociationFailedToSetupList					ProtocolIE-ID ::= 4

id-AMF-TNLAssociationSetupList							ProtocolIE-ID ::= 5

id-AMF-TNLAssociationToAddList							ProtocolIE-ID ::= 6

id-AMF-TNLAssociationToRemoveList						ProtocolIE-ID ::= 7

id-AMF-TNLAssociationToUpdateList						ProtocolIE-ID ::= 8

id-AMFTrafficLoadReductionIndication					ProtocolIE-ID ::= 9

id-AMF-UE-NGAP-ID										ProtocolIE-ID ::= 10

id-AssistanceDataForPaging								ProtocolIE-ID ::= 11

id-BroadcastCancelledAreaList							ProtocolIE-ID ::= 12

id-BroadcastCompletedAreaList							ProtocolIE-ID ::= 13

id-CancelAllWarningMessages								ProtocolIE-ID ::= 14

id-Cause												ProtocolIE-ID ::= 15

id-CellIDListForRestart									ProtocolIE-ID ::= 16

id-ConcurrentWarningMessageInd							ProtocolIE-ID ::= 17

id-CoreNetworkAssistanceInformationForInactive			ProtocolIE-ID ::= 18

id-CriticalityDiagnostics								ProtocolIE-ID ::= 19

id-DataCodingScheme										ProtocolIE-ID ::= 20

id-DefaultPagingDRX										ProtocolIE-ID ::= 21

id-DirectForwardingPathAvailability						ProtocolIE-ID ::= 22

id-EmergencyAreaIDListForRestart						ProtocolIE-ID ::= 23

id-EmergencyFallbackIndicator							ProtocolIE-ID ::= 24

id-EUTRA-CGI											ProtocolIE-ID ::= 25

id-FiveG-S-TMSI											ProtocolIE-ID ::= 26

id-GlobalRANNodeID										ProtocolIE-ID ::= 27

id-GUAMI												ProtocolIE-ID ::= 28

id-HandoverType											ProtocolIE-ID ::= 29

id-IMSVoiceSupportIndicator								ProtocolIE-ID ::= 30

id-IndexToRFSP											ProtocolIE-ID ::= 31

id-InfoOnRecommendedCellsAndRANNodesForPaging			ProtocolIE-ID ::= 32

id-LocationReportingRequestType							ProtocolIE-ID ::= 33

id-MaskedIMEISV											ProtocolIE-ID ::= 34

id-MessageIdentifier									ProtocolIE-ID ::= 35

id-MobilityRestrictionList								ProtocolIE-ID ::= 36

id-NASC													ProtocolIE-ID ::= 37

id-NAS-PDU												ProtocolIE-ID ::= 38

id-NASSecurityParametersFromNGRAN						ProtocolIE-ID ::= 39

id-NewAMF-UE-NGAP-ID									ProtocolIE-ID ::= 40

id-NewSecurityContextInd								ProtocolIE-ID ::= 41

id-NGAP-Message											ProtocolIE-ID ::= 42

id-NGRAN-CGI											ProtocolIE-ID ::= 43

id-NGRANTraceID											ProtocolIE-ID ::= 44

id-NR-CGI												ProtocolIE-ID ::= 45

id-NRPPa-PDU											ProtocolIE-ID ::= 46

id-NumberOfBroadcastsRequested							ProtocolIE-ID ::= 47

id-OldAMF												ProtocolIE-ID ::= 48

id-OverloadStartNSSAIList								ProtocolIE-ID ::= 49

id-PagingDRX											ProtocolIE-ID ::= 50

id-PagingOrigin											ProtocolIE-ID ::= 51

id-PagingPriority										ProtocolIE-ID ::= 52

id-PDUSessionResourceAdmittedList						ProtocolIE-ID ::= 53

id-PDUSessionResourceFailedToModifyListModRes			ProtocolIE-ID ::= 54

id-PDUSessionResourceFailedToSetupListCxtRes			ProtocolIE-ID ::= 55

id-PDUSessionResourceFailedToSetupListHOAck				ProtocolIE-ID ::= 56

id-PDUSessionResourceFailedToSetupListPSReq				ProtocolIE-ID ::= 57

id-PDUSessionResourceFailedToSetupListSURes				ProtocolIE-ID ::= 58

id-PDUSessionResourceHandoverList						ProtocolIE-ID ::= 59

id-PDUSessionResourceListCxtRelCpl						ProtocolIE-ID ::= 60

id-PDUSessionResourceListHORqd							ProtocolIE-ID ::= 61

id-PDUSessionResourceModifyListModCfm					ProtocolIE-ID ::= 62

id-PDUSessionResourceModifyListModInd					ProtocolIE-ID ::= 63

id-PDUSessionResourceModifyListModReq					ProtocolIE-ID ::= 64

id-PDUSessionResourceModifyListModRes					ProtocolIE-ID ::= 65

id-PDUSessionResourceNotifyList							ProtocolIE-ID ::= 66

id-PDUSessionResourceReleasedListNot					ProtocolIE-ID ::= 67

id-PDUSessionResourceReleasedListPSAck					ProtocolIE-ID ::= 68

id-PDUSessionResourceReleasedListPSFail					ProtocolIE-ID ::= 69

id-PDUSessionResourceReleasedListRelRes					ProtocolIE-ID ::= 70

id-PDUSessionResourceSetupListCxtReq					ProtocolIE-ID ::= 71

id-PDUSessionResourceSetupListCxtRes					ProtocolIE-ID ::= 72

id-PDUSessionResourceSetupListHOReq						ProtocolIE-ID ::= 73

id-PDUSessionResourceSetupListSUReq						ProtocolIE-ID ::= 74

id-PDUSessionResourceSetupListSURes						ProtocolIE-ID ::= 75

id-PDUSessionResourceToBeSwitchedDLList					ProtocolIE-ID ::= 76

id-PDUSessionResourceSwitchedList						ProtocolIE-ID ::= 77

id-PDUSessionResourceToReleaseListHOCmd					ProtocolIE-ID ::= 78

id-PDUSessionResourceToReleaseListRelCmd				ProtocolIE-ID ::= 79

id-PLMNSupportList										ProtocolIE-ID ::= 80

id-PWSFailedCellIDList									ProtocolIE-ID ::= 81

id-RANNodeName											ProtocolIE-ID ::= 82

id-RANPagingPriority									ProtocolIE-ID ::= 83

id-RANStatusTransfer-TransparentContainer				ProtocolIE-ID ::= 84

id-RAN-UE-NGAP-ID										ProtocolIE-ID ::= 85

id-RelativeAMFCapacity									ProtocolIE-ID ::= 86

id-RepetitionPeriod										ProtocolIE-ID ::= 87

id-ResetType											ProtocolIE-ID ::= 88

id-RoutingID											ProtocolIE-ID ::= 89

id-RRCEstablishmentCause								ProtocolIE-ID ::= 90

id-RRCInactiveTransitionReportRequest					ProtocolIE-ID ::= 91

id-RRCState												ProtocolIE-ID ::= 92

id-SecurityContext										ProtocolIE-ID ::= 93

id-SecurityKey											ProtocolIE-ID ::= 94

id-SerialNumber											ProtocolIE-ID ::= 95

id-ServedGUAMIList										ProtocolIE-ID ::= 96

id-SliceSupportList										ProtocolIE-ID ::= 97

id-SONConfigurationTransferDL							ProtocolIE-ID ::= 98

id-SONConfigurationTransferUL							ProtocolIE-ID ::= 99

id-SourceAMF-UE-NGAP-ID									ProtocolIE-ID ::= 100

id-SourceToTarget-TransparentContainer					ProtocolIE-ID ::= 101

id-SupportedTAList										ProtocolIE-ID ::= 102

id-TAIListForPaging										ProtocolIE-ID ::= 103

id-TAIListForRestart									ProtocolIE-ID ::= 104

id-TargetID												ProtocolIE-ID ::= 105

id-TargetToSource-TransparentContainer					ProtocolIE-ID ::= 106

id-TimeToWait											ProtocolIE-ID ::= 107

id-TraceActivation										ProtocolIE-ID ::= 108

id-TraceCollectionEntityIPAddress						ProtocolIE-ID ::= 109

id-UEAggregateMaximumBitRate							ProtocolIE-ID ::= 110

id-UE-associatedLogicalNG-connectionList				ProtocolIE-ID ::= 111

id-UEContextRequest										ProtocolIE-ID ::= 112

id-UE-NGAP-IDs											ProtocolIE-ID ::= 114

id-UEPagingIdentity										ProtocolIE-ID ::= 115

id-UEPresenceInAreaOfInterestList						ProtocolIE-ID ::= 116

id-UERadioCapability									ProtocolIE-ID ::= 117

id-UERadioCapabilityForPaging							ProtocolIE-ID ::= 118

id-UESecurityCapabilities								ProtocolIE-ID ::= 119

id-UnavailableGUAMIList									ProtocolIE-ID ::= 120

id-UserLocationInformation								ProtocolIE-ID ::= 121

id-WarningAreaList										ProtocolIE-ID ::= 122

id-WarningMessageContents								ProtocolIE-ID ::= 123

id-WarningSecurityInfo									ProtocolIE-ID ::= 124

id-WarningType											ProtocolIE-ID ::= 125

id-AdditionalUL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 126

id-DataForwardingNotPossible							ProtocolIE-ID ::= 127

id-DL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 128

id-NetworkInstance										ProtocolIE-ID ::= 129

id-PDUSessionAggregateMaximumBitRate					ProtocolIE-ID ::= 130

id-PDUSessionResourceFailedToModifyListModCfm			ProtocolIE-ID ::= 131

id-PDUSessionResourceFailedToSetupListCxtFail			ProtocolIE-ID ::= 132

id-PDUSessionResourceListCxtRelReq						ProtocolIE-ID ::= 133

id-PDUSessionType										ProtocolIE-ID ::= 134

id-QosFlowAddOrModifyRequestList						ProtocolIE-ID ::= 135

id-QosFlowSetupRequestList								ProtocolIE-ID ::= 136

id-QosFlowToReleaseList									ProtocolIE-ID ::= 137

id-SecurityIndication									ProtocolIE-ID ::= 138

id-UL-NGU-UP-TNLInformation								ProtocolIE-ID ::= 139

id-UL-NGU-UP-TNLModifyList								ProtocolIE-ID ::= 140

id-WarningAreaCoordinates								ProtocolIE-ID ::= 141

id-PDUSessionResourceSecondaryRATUsageList				ProtocolIE-ID ::= 142

id-HandoverFlag											ProtocolIE-ID ::= 143

id-SecondaryRATUsageInformation							ProtocolIE-ID ::= 144

id-PDUSessionResourceReleaseResponseTransfer			ProtocolIE-ID ::= 145

id-RedirectionVoiceFallback								ProtocolIE-ID ::= 146

id-UERetentionInformation								ProtocolIE-ID ::= 147

id-S-NSSAI												ProtocolIE-ID ::= 148

id-PSCellInformation									ProtocolIE-ID ::= 149

id-LastEUTRAN-PLMNIdentity								ProtocolIE-ID ::= 150

id-MaximumIntegrityProtectedDataRate-DL					ProtocolIE-ID ::= 151

id-AdditionalDLForwardingUPTNLInformation				ProtocolIE-ID ::= 152

id-AdditionalDLUPTNLInformationForHOList				ProtocolIE-ID ::= 153

id-AdditionalNGU-UP-TNLInformation						ProtocolIE-ID ::= 154

id-AdditionalDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 155

id-SecurityResult										ProtocolIE-ID ::= 156

id-ENDC-SONConfigurationTransferDL						ProtocolIE-ID ::= 157

id-ENDC-SONConfigurationTransferUL						ProtocolIE-ID ::= 158

id-OldAssociatedQosFlowList-ULendmarkerexpected			ProtocolIE-ID ::= 159

id-CNTypeRestrictionsForEquivalent						ProtocolIE-ID ::= 160

id-CNTypeRestrictionsForServing							ProtocolIE-ID ::= 161

id-NewGUAMI												ProtocolIE-ID ::= 162

id-ULForwarding											ProtocolIE-ID ::= 163

id-ULForwardingUP-TNLInformation						ProtocolIE-ID ::= 164

id-CNAssistedRANTuning									ProtocolIE-ID ::= 165

id-CommonNetworkInstance								ProtocolIE-ID ::= 166

id-NGRAN-TNLAssociationToRemoveList						ProtocolIE-ID ::= 167

id-TNLAssociationTransportLayerAddressNGRAN				ProtocolIE-ID ::= 168

id-EndpointIPAddressAndPort								ProtocolIE-ID ::= 169

id-LocationReportingAdditionalInfo						ProtocolIE-ID ::= 170

id-SourceToTarget-AMFInformationReroute					ProtocolIE-ID ::= 171

id-AdditionalULForwardingUPTNLInformation				ProtocolIE-ID ::= 172

id-SCTP-TLAs											ProtocolIE-ID ::= 173

id-SelectedPLMNIdentity									ProtocolIE-ID ::= 174

id-RIMInformationTransfer								ProtocolIE-ID ::= 175

id-GUAMIType											ProtocolIE-ID ::= 176

id-SRVCCOperationPossible								ProtocolIE-ID ::= 177

id-TargetRNC-ID											ProtocolIE-ID ::= 178

id-RAT-Information										ProtocolIE-ID ::= 179

id-ExtendedRATRestrictionInformation					ProtocolIE-ID ::= 180

id-QosMonitoringRequest									ProtocolIE-ID ::= 181

id-SgNB-UE-X2AP-ID										ProtocolIE-ID ::= 182

id-AdditionalRedundantDL-NGU-UP-TNLInformation			ProtocolIE-ID ::= 183

id-AdditionalRedundantDLQosFlowPerTNLInformation		ProtocolIE-ID ::= 184

id-AdditionalRedundantNGU-UP-TNLInformation				ProtocolIE-ID ::= 185

id-AdditionalRedundantUL-NGU-UP-TNLInformation			ProtocolIE-ID ::= 186

id-CNPacketDelayBudgetDL								ProtocolIE-ID ::= 187

id-CNPacketDelayBudgetUL								ProtocolIE-ID ::= 188

id-ExtendedPacketDelayBudget							ProtocolIE-ID ::= 189

id-RedundantCommonNetworkInstance						ProtocolIE-ID ::= 190

id-RedundantDL-NGU-TNLInformationReused					ProtocolIE-ID ::= 191

id-RedundantDL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 192

id-RedundantDLQosFlowPerTNLInformation					ProtocolIE-ID ::= 193

id-RedundantQosFlowIndicator							ProtocolIE-ID ::= 194

id-RedundantUL-NGU-UP-TNLInformation					ProtocolIE-ID ::= 195

id-TSCTrafficCharacteristics							ProtocolIE-ID ::= 196

id-RedundantPDUSessionInformation 						ProtocolIE-ID ::= 197

id-UsedRSNInformation									ProtocolIE-ID ::= 198

id-IAB-Authorized										ProtocolIE-ID ::= 199

id-IAB-Supported										ProtocolIE-ID ::= 200

id-IABNodeIndication									ProtocolIE-ID ::= 201

id-NB-IoT-PagingDRX										ProtocolIE-ID ::= 202

id-NB-IoT-Paging-eDRXInfo								ProtocolIE-ID ::= 203

id-NB-IoT-DefaultPagingDRX								ProtocolIE-ID ::= 204

id-Enhanced-CoverageRestriction							ProtocolIE-ID ::= 205

id-Extended-ConnectedTime								ProtocolIE-ID ::= 206

id-PagingAssisDataforCEcapabUE							ProtocolIE-ID ::= 207

id-WUS-Assistance-Information							ProtocolIE-ID ::= 208

id-UE-DifferentiationInfo								ProtocolIE-ID ::= 209

id-NB-IoT-UEPriority									ProtocolIE-ID ::= 210

id-UL-CP-SecurityInformation							ProtocolIE-ID ::= 211

id-DL-CP-SecurityInformation							ProtocolIE-ID ::= 212

id-TAI													ProtocolIE-ID ::= 213

id-UERadioCapabilityForPagingOfNB-IoT					ProtocolIE-ID ::= 214

id-LTEV2XServicesAuthorized								ProtocolIE-ID ::= 215

id-NRV2XServicesAuthorized								ProtocolIE-ID ::= 216

id-LTEUESidelinkAggregateMaximumBitrate					ProtocolIE-ID ::= 217

id-NRUESidelinkAggregateMaximumBitrate					ProtocolIE-ID ::= 218

id-PC5QoSParameters										ProtocolIE-ID ::= 219

id-AlternativeQoSParaSetList							ProtocolIE-ID ::= 220

id-CurrentQoSParaSetIndex								ProtocolIE-ID ::= 221

id-CEmodeBrestricted									ProtocolIE-ID ::= 222

id-EUTRA-PagingeDRXInformation							ProtocolIE-ID ::= 223

id-CEmodeBSupport-Indicator								ProtocolIE-ID ::= 224

id-LTEM-Indication										ProtocolIE-ID ::= 225

id-EndIndication										ProtocolIE-ID ::= 226

id-EDT-Session											ProtocolIE-ID ::= 227

id-UECapabilityInfoRequest								ProtocolIE-ID ::= 228

id-PDUSessionResourceFailedToResumeListRESReq			ProtocolIE-ID ::= 229

id-PDUSessionResourceFailedToResumeListRESRes			ProtocolIE-ID ::= 230

id-PDUSessionResourceSuspendListSUSReq					ProtocolIE-ID ::= 231

id-PDUSessionResourceResumeListRESReq					ProtocolIE-ID ::= 232

id-PDUSessionResourceResumeListRESRes					ProtocolIE-ID ::= 233

id-UE-UP-CIoT-Support									ProtocolIE-ID ::= 234

id-Suspend-Request-Indication							ProtocolIE-ID ::= 235

id-Suspend-Response-Indication							ProtocolIE-ID ::= 236

id-RRC-Resume-Cause										ProtocolIE-ID ::= 237

id-RGLevelWirelineAccessCharacteristics					ProtocolIE-ID ::= 238

id-W-AGFIdentityInformation								ProtocolIE-ID ::= 239

id-GlobalTNGF-ID										ProtocolIE-ID ::= 240

id-GlobalTWIF-ID										ProtocolIE-ID ::= 241

id-GlobalW-AGF-ID										ProtocolIE-ID ::= 242

id-UserLocationInformationW-AGF							ProtocolIE-ID ::= 243

id-UserLocationInformationTNGF							ProtocolIE-ID ::= 244

id-AuthenticatedIndication								ProtocolIE-ID ::= 245

id-TNGFIdentityInformation								ProtocolIE-ID ::= 246

id-TWIFIdentityInformation								ProtocolIE-ID ::= 247

id-UserLocationInformationTWIF							ProtocolIE-ID ::= 248

id-DataForwardingResponseERABList						ProtocolIE-ID ::= 249

id-IntersystemSONConfigurationTransferDL				ProtocolIE-ID ::= 250

id-IntersystemSONConfigurationTransferUL				ProtocolIE-ID ::= 251

id-SONInformationReport									ProtocolIE-ID ::= 252

id-UEHistoryInformationFromTheUE						ProtocolIE-ID ::= 253

id-ManagementBasedMDTPLMNList							ProtocolIE-ID ::= 254

id-MDTConfiguration										ProtocolIE-ID ::= 255

id-PrivacyIndicator										ProtocolIE-ID ::= 256

id-TraceCollectionEntityURI								ProtocolIE-ID ::= 257

id-NPN-Support											ProtocolIE-ID ::= 258

id-NPN-AccessInformation								ProtocolIE-ID ::= 259

id-NPN-PagingAssistanceInformation						ProtocolIE-ID ::= 260

id-NPN-MobilityInformation								ProtocolIE-ID ::= 261

id-TargettoSource-Failure-TransparentContainer			ProtocolIE-ID ::= 262

id-NID													ProtocolIE-ID ::= 263

id-UERadioCapabilityID									ProtocolIE-ID ::= 264

id-UERadioCapability-EUTRA-Format						ProtocolIE-ID ::= 265

id-DAPSRequestInfo									ProtocolIE-ID ::= 266

id-DAPSResponseInfoList								ProtocolIE-ID ::= 267

id-EarlyStatusTransfer-TransparentContainer 			ProtocolIE-ID ::= 268

id-NotifySourceNGRANNode								ProtocolIE-ID ::= 269

id-ExtendedSliceSupportList								ProtocolIE-ID ::= 270

id-ExtendedTAISliceSupportList							ProtocolIE-ID ::= 271

id-ConfiguredTACIndication								ProtocolIE-ID ::= 272

id-Extended-RANNodeName									ProtocolIE-ID ::= 273

id-Extended-AMFName										ProtocolIE-ID ::= 274

id-GlobalCable-ID										ProtocolIE-ID ::= 275

id-QosMonitoringReportingFrequency						ProtocolIE-ID ::= 276

id-QosFlowParametersList								ProtocolIE-ID ::= 277

id-QosFlowFeedbackList									ProtocolIE-ID ::= 278

id-BurstArrivalTimeDownlink								ProtocolIE-ID ::= 279

id-ExtendedUEIdentityIndexValue							ProtocolIE-ID ::= 280

id-PduSessionExpectedUEActivityBehaviour				ProtocolIE-ID ::= 281

id-MicoAllPLMN											ProtocolIE-ID ::= 282

id-QosFlowFailedToSetupList								ProtocolIE-ID ::= 283

id-SourceTNLAddrInfo									ProtocolIE-ID ::= 284

id-ExtendedReportIntervalMDT							ProtocolIE-ID ::= 285

id-SourceNodeID											ProtocolIE-ID ::= 286

id-NRNTNTAIInformation									ProtocolIE-ID ::= 287

id-UEContextReferenceAtSource							ProtocolIE-ID ::= 288

id-LastVisitedPSCellList								ProtocolIE-ID ::= 289

id-IntersystemSONInformationRequest						ProtocolIE-ID ::= 290

id-IntersystemSONInformationReply						ProtocolIE-ID ::= 291

id-EnergySavingIndication								ProtocolIE-ID ::= 292

id-IntersystemResourceStatusUpdate						ProtocolIE-ID ::= 293

id-SuccessfulHandoverReportList							ProtocolIE-ID ::= 294

id-MBS-AreaSessionID 									ProtocolIE-ID ::= 295

id-MBS-QoSFlowsToBeSetupList							ProtocolIE-ID ::= 296

id-MBS-QoSFlowsToBeSetupModList							ProtocolIE-ID ::= 297

id-MBS-ServiceArea 										ProtocolIE-ID ::= 298

id-MBS-SessionID 										ProtocolIE-ID ::= 299

id-MBS-DistributionReleaseRequestTransfer				ProtocolIE-ID ::= 300

id-MBS-DistributionSetupRequestTransfer 				ProtocolIE-ID ::= 301

id-MBS-DistributionSetupResponseTransfer				ProtocolIE-ID ::= 302

id-MBS-DistributionSetupUnsuccessfulTransfer			ProtocolIE-ID ::= 303

id-MulticastSessionActivationRequestTransfer			ProtocolIE-ID ::= 304

id-MulticastSessionDeactivationRequestTransfer			ProtocolIE-ID ::= 305

id-MulticastSessionUpdateRequestTransfer				ProtocolIE-ID ::= 306

id-MulticastGroupPagingAreaList							ProtocolIE-ID ::= 307

id-MBS-SupportIndicator									ProtocolIE-ID ::= 309

id-MBSSessionFailedtoSetupList							ProtocolIE-ID ::= 310

id-MBSSessionFailedtoSetuporModifyList					ProtocolIE-ID ::= 311

id-MBSSessionSetupResponseList							ProtocolIE-ID ::= 312

id-MBSSessionSetuporModifyResponseList					ProtocolIE-ID ::= 313

id-MBSSessionSetupFailureTransfer						ProtocolIE-ID ::= 314

id-MBSSessionSetupRequestTransfer						ProtocolIE-ID ::= 315

id-MBSSessionSetupResponseTransfer						ProtocolIE-ID ::= 316

id-MBSSessionToReleaseList								ProtocolIE-ID ::= 317

id-MBSSessionSetupRequestList 							ProtocolIE-ID ::= 318

id-MBSSessionSetuporModifyRequestList 					ProtocolIE-ID ::= 319

id-MBS-ActiveSessionInformation-SourcetoTargetList		ProtocolIE-ID ::= 323

id-MBS-ActiveSessionInformation-TargettoSourceList		ProtocolIE-ID ::= 324

id-OnboardingSupport									ProtocolIE-ID ::= 325

id-TimeSyncAssistanceInfo								ProtocolIE-ID ::= 326

id-SurvivalTime											ProtocolIE-ID ::= 327

id-QMCConfigInfo										ProtocolIE-ID ::= 328

id-QMCDeactivation										ProtocolIE-ID ::= 329

id-PDUSessionPairID										ProtocolIE-ID ::= 331

id-NR-PagingeDRXInformation								ProtocolIE-ID ::= 332

id-RedCapIndication										ProtocolIE-ID ::= 333

id-TargetNSSAIInformation								ProtocolIE-ID ::= 334

id-UESliceMaximumBitRateList							ProtocolIE-ID ::= 335

id-M4ReportAmount										ProtocolIE-ID ::= 336

id-M5ReportAmount										ProtocolIE-ID ::= 337

id-M6ReportAmount										ProtocolIE-ID ::= 338

id-M7ReportAmount										ProtocolIE-ID ::= 339

id-IncludeBeamMeasurementsIndication					ProtocolIE-ID ::= 340

id-M6DelayThreshold   									ProtocolIE-ID ::= 341

id-PagingCause											ProtocolIE-ID ::= 342

id-PagingCauseIndicationForVoiceService					ProtocolIE-ID ::= 343

id-PEIPSassistanceInformation							ProtocolIE-ID ::= 344

id-FiveG-ProSeAuthorized								ProtocolIE-ID ::= 345

id-FiveG-ProSeUEPC5AggregateMaximumBitRate				ProtocolIE-ID ::= 346

id-FiveG-ProSePC5QoSParameters							ProtocolIE-ID ::= 347

id-MBSSessionModificationFailureTransfer				ProtocolIE-ID ::= 348

id-MBSSessionModificationRequestTransfer				ProtocolIE-ID ::= 349

id-MBSSessionModificationResponseTransfer				ProtocolIE-ID ::= 350

id-MBS-QoSFlowToReleaseList								ProtocolIE-ID ::= 351

id-MBS-SessionTNLInfo5GC								ProtocolIE-ID ::= 352

id-TAINSAGSupportList									ProtocolIE-ID ::= 353

id-SourceNodeTNLAddrInfo								ProtocolIE-ID ::= 354

id-NGAPIESupportInformationRequestList					ProtocolIE-ID ::= 355

id-NGAPIESupportInformationResponseList					ProtocolIE-ID ::= 356

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-MBS-RANsharingIdentifier								ProtocolIE-ID ::= FFS

END

-- ASN1STOP

-----------------End of the Changes-------------------