3GPP TSG-RAN WG3 Meeting #118	R3-226339

Toulouse, FR, 14-18 Nov, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0907 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the delay issue on initialization of initialRX-DELIV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For multicast session, in some cases, the MRBs cannot be configured as expected during multicast session establishment procedure, and configuring MRB after receiving multicast data will lead to extra delay for data reception for UE. Thus, it is necessary to introduce initial MBS QFI SN in NGAP: DISTRIBUTION SETUP RESPONSE message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce initial MBS QFI SN in NGAP: DISTRIBUTION SETUP RESPONSE message Impact Analysis: This CR has an isolated impact towards the previous version of the specification, and will not cause any non-backward compatible issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The issue that the delay to configure MRB still remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.236， 9.4.5， 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of the first change--------------------

#### 9.3.1.236	MBS QoS Flows To Be Setup List

This IE contains a list of QoS flows and their associated QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS QoS Flows Setup Request Item |  | 1..<maxnoofMBSQoSFlows> |  |  | - |  |
| >MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.12 |  | - |  |
| >Initial MBS QFI SN | O |  | INTEGER (0..232-1) | This IE indicates the Initial MBS QFI SN of the multicast MBS QoS Flows, which is used to determine the PDCP configuration. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is 64. |

--------------Start of the Next change--------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AlternativeQoSParaSetList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-ConfiguredTACIndication,

id-CurrentQoSParaSetIndex,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

id-DataForwardingNotPossible,

id-DataForwardingResponseERABList,

id-DirectForwardingPathAvailability,

id-DL-NGU-UP-TNLInformation,

id-EndpointIPAddressAndPort,

id-EnergySavingIndication,

id-ExtendedPacketDelayBudget,

id-ExtendedRATRestrictionInformation,

id-ExtendedReportIntervalMDT,

id-ExtendedSliceSupportList,

id-ExtendedTAISliceSupportList,

id-ExtendedUEIdentityIndexValue,

id-EUTRA-PagingeDRXInformation,

id-GlobalCable-ID,

id-GlobalRANNodeID,

id-GlobalTNGF-ID,

id-GlobalTWIF-ID,

id-GlobalW-AGF-ID,

id-GUAMIType,

id-IncludeBeamMeasurementsIndication,

id-InitialMBS-QFI-SN,

id-IntersystemSONInformationRequest,

id-IntersystemSONInformationReply,

id-IntersystemResourceStatusUpdate,

id-LastEUTRAN-PLMNIdentity,

id-LastVisitedPSCellList,

id-LocationReportingAdditionalInfo,

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-M6DelayThreshold,

id-M7ReportAmount,

id-MaximumIntegrityProtectedDataRate-DL,

id-MBS-AreaSessionID,

id-MBS-QoSFlowsToBeSetupList,

id-MBS-QoSFlowsToBeSetupModList,

id-MBS-QoSFlowToReleaseList,

id-MBS-ServiceArea,

id-MBS-SessionFSAIDList,

id-MBS-SessionID,

id-MBS-ActiveSessionInformation-SourcetoTargetList,

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

//skip unchanged part

InfoOnRecommendedCellsAndRANNodesForPaging ::= SEQUENCE {

recommendedCellsForPaging		RecommendedCellsForPaging,

recommendRANNodesForPaging		RecommendedRANNodesForPaging,

iE-Extensions		ProtocolExtensionContainer { {InfoOnRecommendedCellsAndRANNodesForPaging-ExtIEs} }	OPTIONAL,

...

}

InfoOnRecommendedCellsAndRANNodesForPaging-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

InitialMBS-QFI-SN ::= INTEGER (0..4294967295)

IntegrityProtectionIndication ::= ENUMERATED {

required,

preferred,

not-needed,

...

}

IntegrityProtectionResult ::= ENUMERATED {

performed,

not-performed,

...

}

//skip unchanged part

MBS-QoSFlowsToBeSetupList ::= SEQUENCE (SIZE(1.. maxnoofMBSQoSFlows)) OF MBS-QoSFlowsToBeSetupItem

MBS-QoSFlowsToBeSetupItem ::= SEQUENCE {

mBSqosFlowIdentifier				QosFlowIdentifier,

mBSqosFlowLevelQosParameters		QosFlowLevelQosParameters,

iE-Extensions						ProtocolExtensionContainer { {MBS-QoSFlowsToBeSetupItem-ExtIEs} }	OPTIONAL,

...

}

MBS-QoSFlowsToBeSetupItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-InitialMBS-QFI-SN			CRITICALITY ignore	EXTENSION InitialMBS-QFI-SN				PRESENCE optional },

...

}

--------------Start of the Next change--------------------

### 9.4.7	Constant Definitions

//skip unchanged part

id-PagingCause											ProtocolIE-ID ::= 342

id-PagingCauseIndicationForVoiceService					ProtocolIE-ID ::= 343

id-PEIPSassistanceInformation							ProtocolIE-ID ::= 344

id-FiveG-ProSeAuthorized								ProtocolIE-ID ::= 345

id-FiveG-ProSeUEPC5AggregateMaximumBitRate				ProtocolIE-ID ::= 346

id-FiveG-ProSePC5QoSParameters							ProtocolIE-ID ::= 347

id-MBSSessionModificationFailureTransfer				ProtocolIE-ID ::= 348

id-MBSSessionModificationRequestTransfer				ProtocolIE-ID ::= 349

id-MBSSessionModificationResponseTransfer				ProtocolIE-ID ::= 350

id-MBS-QoSFlowToReleaseList								ProtocolIE-ID ::= 351

id-MBS-SessionTNLInfo5GC								ProtocolIE-ID ::= 352

id-TAINSAGSupportList									ProtocolIE-ID ::= 353

id-SourceNodeTNLAddrInfo								ProtocolIE-ID ::= 354

id-NGAPIESupportInformationRequestList					ProtocolIE-ID ::= 355

id-NGAPIESupportInformationResponseList					ProtocolIE-ID ::= 356

id-MBS-SessionFSAIDList									ProtocolIE-ID ::= 357

id-MBSSessionReleaseResponseTransfer					ProtocolIE-ID ::= 358

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-InitialMBS-QFI-SN									ProtocolIE-ID ::= FFS

END

-- ASN1STOP

--------------End of the changes--------------------