3GPP TSG-RAN WG3 Meeting #118	R3-226344

Toulouse, FR, 14-18 Nov, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.473 | CR | 0017 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Missing Criticality Diagnostics over W1AP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, BT, Deutsche Telekom, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_arch_evo-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Criticality Diagnostics is used to inform the peer node that parts of a received message have not been comprehended or were missing, or if the message contained logical errors and indicate which IEs were not comprehended or were missing. Therefore for Class1 procedure, in RAN3 interfaces, we include the Criticality Diagnostics in the response messages and the failure messages. But in W1AP, the Criticality Diagnostics IE is missing in some response messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Criticality Diagnostics IE in the following W1AP messages: - W1 SETUP RESPONSE - NG-ENB-DU RESOURCE COORDINATION RESPONSE Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it only adds the missing Criticality Diagnostics as optional IE with criticality ignore in some response messages.The impact can be considered isolated because the change only adding the Criticality Diagnostics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Criticality Diagnostics IE is missing in many response messages, the receiving node is not able to inform the related errors to the sendng node. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.5, 9.2.1.14, 9.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------Start of the First Change-----------

#### 9.2.1.5	W1 SETUP RESPONSE

This message is sent by the ng-eNB-CU to transfer information associated to a W1-C interface instance.

Direction: ng-eNB-CU  ng-eNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0.. 1 |  |  | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellinng-eNBDU> |  | List of cells to be activated | EACH | reject |
| >> E-UTRAN CGI | M |  | 9.3.1.12 |  | - |  |
| >> E-UTRAN PCI | O |  | INTEGER (0..503) | Physical Cell ID | - |  |
| >>ng-eNB-CU System Information | O |  | 9.3.1.33 | RRC container with system information owned by ng-eNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.47 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.55 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.61 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellinng-eNBDU | Maximum no. cells that can be served by an ng-eNB-DU. Value is 512. |

-----------Start of the Next Change-----------

#### 9.2.1.14	NG-ENB-DU RESOURCE COORDINATION RESPONSE

This message is sent by an ng-eNB-DU to an ng-eNB-CU, to express the desired resource allocation for data traffic, as a response to the NG-ENB-DU RESOURCE COORDINATION REQUEST.

Direction: ng-eNB-DU  ng-eNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| E-UTRA – NR Cell Resource Coordination Response Container | M |  | OCTET STRING | Includes the XNAP E-UTRA – NR CELL RESOURCE COORDINATION RESPONSE message as defined in clause 9.1.2.24 in TS 38.423 [7]. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.61 |  | YES | ignore |

-----------Start of the Next Change-----------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for W1AP.

--

-- **************************************************************

W1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) w1ap (3) version1 (1) w1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

//skip the unchanged part

-- **************************************************************

--

-- W1 Setup Response

--

-- **************************************************************

W1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {W1SetupResponseIEs} },

...

}

W1SetupResponseIEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID		CRITICALITY reject	TYPE TransactionID		PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List	CRITICALITY reject	TYPE Cells-to-be-Activated-List		PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

//skip the unchanged part

-- **************************************************************

--

-- NGENBDU RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

NGENBDUResourceCoordinationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{NGENBDUResourceCoordinationResponse-IEs}},

...

}

NGENBDUResourceCoordinationResponse-IEs W1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-EUTRA-NR-CellResourceCoordinationReqAck-Container	CRITICALITY reject	TYPE EUTRA-NR-CellResourceCoordinationReqAck-Container		PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-----------End of the Changes-----------